3GPP TSG-RAN WG2 Meeting #123	R2-2308306

Toulouse, France, August 21-25, 2023

Agenda Item:	7.11.3

Source:	CMCC

Title:	Discussion on shared processing

Document for:	Discussion

Introduction

At the RAN2#122 meeting[1], the following agreements were agreed regarding the shared processing：

The granularity for capability of receiving MBS broadcast from a non-serving cell is at FeatureSetDownlinkPerCC level. This capability does not imply simultaneous reception on multiple CCs. No additional signalling is introduced to control information to be reported by the UE (on top of what we have already agreed). When sending MII, UE reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell. FFS whether there are cases where this information is not available at the UE and what happens then. FFS if any special handling is needed when the non-serving cell updates the configuration (which is relevant for MII) No additional information is added to MII on top of what has been already agreed.

In this contribution, we provide our view on the related open issues.

Discussion

In last meeting, it was agreed that “When sending MII, UE reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell.”  This agreement aims to avoid a two-step reporting. But it cannot avoid the situation that though the network allows the MII reporting, but UE does not have the full information.

One way to solve this problem is to send an indication to the serving cell, and the serving cell may acquire the missing information from the non-serving cell. However, this approach is only suitable to the case that serving cell and non-serving cell are intra-gNB or there’s Xn interface between them. Besides, the current agreement is to avoid the two-step mechanism, therefore, this approach is not acceptable.

In our view, the current MII contents agreed are all necessity for the UE to receive the MBS service from the non-serving cell, and all these information will be provided by the non-serving cell and/or USD. Once a UE decides to send MII (with the serving cell’s permission indication in the system information), it means the UE has the requirements for the shared processing, therefore, the UE should acquire the MII content which is not available at it from the non-serving cell, for example, acquiring SIB21 and MCCH of the non-serving cell.

Proposal 1: In case that the information of MII is not available at the UE, it’s UE implementation, for example, acquiring SIB 21 and the MCCH of the non-serving cell.

If UE cannot acquire the necessary information, UE will suspend the MII transmission.

As to the MII content, it was agreed to report frequency, bandwidth, SCS. It should be noticed, there’s some NR characters are introduced for NR MBS, such as CFR, which is used to indicated the bandwidth used for broadcast. We think it should be considered to be reported in MII, too.

CFR for broadcast is indicated in SIB 20 CFR-ConfigMCCH-MTCH, locationAndBandwidthBroadcast is actually used to indicate the frequency domain resource used by the broadcast service [2].

CFR-ConfigMCCH-MTCH information element

-- ASN1START

-- TAG-CFR-CONFIGMCCH-MTCH-START

CFR-ConfigMCCH-MTCH-r17 ::= SEQUENCE {

locationAndBandwidthBroadcast-r17          LocationAndBandwidthBroadcast-r17  OPTIONAL,  -- Need S

pdsch-ConfigMCCH-r17                       PDSCH-ConfigBroadcast-r17          OPTIONAL,  -- Need S

commonControlResourceSetExt-r17            ControlResourceSet                 OPTIONAL   -- Cond NotSIB1CommonControlResource

}

LocationAndBandwidthBroadcast-r17 ::= CHOICE {

sameAsSib1ConfiguredLocationAndBW          NULL,

locationAndBandwidth                       INTEGER (0..37949)

}

-- TAG-CFR-CONFIGMCCH-MTCH-STOP

-- ASN1STOP

CFR-ConfigMCCH-MTCH field descriptions

commonControlResourceSetExt An additional common control resource set which may be configured and used for searchSpaceMCCH/searchSpaceMTCH or UE-specific search space in the BWP where searchSpaceMCCH is configured. It is contained in the bandwidth of locationAndBandwidthBroadcast.

locationAndBandwidthBroadcast Indicates starting PRB and the number of PRBs of CFR used for MCCH and MTCH reception. Value sameAsSib1ConfiguredLocationAndBW means the CFR for broadcast has the same location and size as the locationAndBandwidth for initial BWP configured in SIB1. Value locationAndBandwidth is used to configure CFR with bandwidth that is larger than and fully contains the bandwidth for the initial DL BWP and CORESET#0 configured in SIB1. If the field is absent, the CFR for broadcast has the same location and size as CORESET0.

pdsch-ConfigMCCH Indicates PDSCH parameters used for MCCH transmission. If the field is absent, PDSCH paramers used for MCCH are the same as those of PDSCH configuration provided in initialDownlinkBWP in SIB1.

Proposal 2: the bandwidth reporting in MII can be or include CFR related information, for example locationAndBandwidthBroadcast.

Conclusions

In this contribution, we analyse the open issues of shared processing, following are our proposals:

Proposals:

Proposal 1: In case that the information of MII is not available at the UE, it’s UE implementation, for example, acquiring SIB 21 and the MCCH of the non-serving cell.

Proposal 2: the bandwidth reporting in MII can be or include CFR related information, for example locationAndBandwidthBroadcast.

References

- R2_122_meeting_report;

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification;