3GPP TSG-RAN WG2 Meeting #123	R2-2308356

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.14.3

Source:	Huawei, HiSilicon

Title:	Discussion on Rel-17 left-over issues

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss two issues which are seen as left-overs from Rel-17:

- Threshold based RAN visible QoE reporting

- RAN overload handling enhancements

# Threshold based RAN visible QoE reporting

RAN3 made the following agreements related to threshold based RVQoE topic which have relevance for RAN2:

In RAN3#119bis: RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options: a. Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports) b. Option 2: Periodically based on a gNB configured reporting periodicity c. Option 3: A certain number of times based on gNB configured report amount In RAN3#120: The network can configure UE with two types of events for threshold based RVQoE reporting, the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold. Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed.

RAN2 on the other hand made the following agreements during RAN2#122 meeting:

- RAN2 confirms that buffer level threshold-based triggering of RVQoE reporting is triggered by application layer. - RAN2 waits for RAN3’s conclusion on the buffer level threshold-based RVQoE reporting.

The first RAN2 agreement just confirmed a previous assumption which was made by RAN2 based on SA4 reply LS in [1]. However, it should be noted that SA4 had further discussions on buffer level based QoE triggering and has reached a conclusion that such mechanism is not feasible, as can be seen in the latest SA4 LS in [2]:

However, in SA4#124 meeting, SA4 has further discussed this problem and came to an understanding that buffer level on its own is not an appropriate information for RAN to be able to assist the application due to a variety of reasons, e.g. - A higher buffer level may help reduce the probability of a playback stall, but may also increase the playback latency, so it’s a trade-off that needs to be balanced by applications. Different proprietary application mechanisms attempting to strike this balance may result in different application behavior for same buffer level or same content. - Also, different applications (e.g. low-latency live vs. on-demand video applications) have very different behaviour of maintaining the buffer level.

SA4 further informs that based on the above, SA4 “does not confirm RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer” and that “SA4 is considering appropriate alternatives for this purpose. SA4 requests RAN2 and RAN3 to take this information in account, and that SA4 will keep RAN2 and RAN3 updated on progress in this direction.” [2].

Observation 1: SA4 does not believe application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18.

However, it should be noted that the essence of SA4 updated reply is that buffer level metric is not an appropriate one for triggering RVQoE regardless of whether triggering is made at application layer or in AS layer. SA4 intends to work on some alternatives for this issue.

Observation 2: According to SA4 feedback, buffer level is not an appropriate metric for triggering RVQoE reporting regardless of whether triggering is made in AS layer or in application layer.

Observation 3: SA4 is working on alternative solutions in this area and will keep RAN2 and RAN3 updated once they make a progress.

We believe that considering this feedback, RAN WGs should cease the work on this topic until further information from SA4 is received. Otherwise, RAN WGs may unnecessarily spend time on mechanisms which prove to be infeasible or not useful.

- RAN2 should suspend all the work on threshold based RVQoE until further progress is made by SA4.

# QoE reporting enhancement for overload scenario

RAN2 tentatively discussed QoE reporting enhancement for overload scenario during RAN2#119bis meeting and it was decided to postpone the discussion and wait for further RAN3 progress.

7: RAN2 to postpone the discussion of the QoE reporting enhancement for overload scenario to the next meeting (based on the progress of RAN3).

RAN2 also made the following agreement in the scope of QoE for MBS broadcast discussion:

- 4: As a default behavior, when the UE’s buffer for storing QoE reports is full and a new report arrives, the UE should discard older report(s) to make room for the new one. - 5: FFS whether it is possible to provide information (e.g. priority, service type, etc.) to UE about buffering for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full.

In general, companies agreed that selection policies or priorities for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full would be useful. However, it was unclear how such priorities or policies would be defined and it was eventually decided to postpone the decision. It should be noted that a related topic was discussed by RAN3 and it was decided to request some clarifications on this aspect from SA5, see the LS in [3]. RAN3’s main intention is to support this mechanism to deal with RAN overload scenario, e.g. for the gNB to decide which QoE configurations to pause. From RAN2 point of view, if such information is forwarded to the UE, the UE may use it to decide which reports to keep either while reporting is paused or when buffering MBS QoE reports while in RRC INACTIVE/IDLE state. We then propose that RAN2 agrees about the usefulness of this mechanism, but only discusses the details once the reply from SA5 is received and RAN3 clarifies the contents of the intended assistance information provided from OAM to RAN.

- RAN2 agrees that assistance information for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full is useful, e.g. for storing QoE reports during pause or in RRC IDLE/INACTIVE state. RAN2 should wait for RAN3 conclusion on the contents of assistance information provided from OAM to RAN before working on the details.

# Conclusion

Based on the discussion in the paper, we observe and propose the following:

Threshold based RAN visible QoE reporting

Observation 1: SA4 does not believe application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18.

Observation 2: According to SA4 feedback, buffer level is not an appropriate metric for triggering RVQoE reporting regardless of whether triggering is made in AS layer or in application layer.

Observation 3: SA4 is working on alternative solutions in this area and will keep RAN2 and RAN3 updated once they make a progress.

- RAN2 should suspend all the work on threshold based RVQoE until further progress is made by SA4.

QoE reporting enhancement for overload scenario

- RAN2 agrees that assistance information for the UE to decide which reports to discard in case the UE’s QoE buffer becomes full is useful, e.g. for storing QoE reports during pause or in RRC IDLE/INACTIVE state. RAN2 should wait for RAN3 conclusion on the contents of assistance information provided from OAM to RAN before working on the details.

# References

[1] S4-230684, Reply LS on buffer level threshold-based RVQoE reporting, SA4

[2] S4aI230140, Reply LS on buffer level threshold-based RVQoE reporting, SA4

[3] R3-232047, LS on the feasibility of introducing assistance information for handling of QoE reporting during RAN overload, RAN3