3GPP TSG-RAN WG2 Meeting #123	R2-2308398

Toulouse, France, August 21 – 25, 2023

Agenda item:	7.2.4

Source: 	Qualcomm Incorporated

Title: 	Enhancements for LPHAP

Document for:		Discussion

# 1.	Introduction

The objective for "Low-Power, High-Accuracy Positioning (LPHAP)" includes essentially four different topics for RAN2 [1]:

- Specify enhancements for enabling LPHAP use-case 6 as defined in TS 22.104 including: - Extending eDRX cycle beyond 10.24s in RRC_INACTIVE state towards meeting the battery life requirement for LPHAP [RAN2, RAN3, RAN4] - Positioning-specific enhancement for eDRX cycle beyond 10.24s to be defined as part of Rel-18 WI on expanded and improved NR positioning. - NOTE: Work on this objective should be coordinated with that in Rel-18 WI on eRedCap. Towards this, the feature of extending eDRX cycle beyond 10.24s should be defined as part of Rel-18 WI on eRedCap. - NOTE: Inputs from RAN1 as necessary may be facilitated via LSs - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1]. - Specify solutions for DL PRS measurements for a UE in RRC_IDLE state and reporting of the measurements in RRC_CONNECTED state [RAN2]. - Specify solutions for alignment between eDRX and PRS configurations [RAN2]. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

During the Study Item Phase it was agreed at RAN2#119e [2]:

"RAN2 to consider at least the 'Low Power Periodic and Triggered 5GC-MT-LR Procedures' in TS 23.273.  Other procedures are not excluded from discussion."

A simplified/RAN-centric procedure of the 'Low Power Periodic and Triggered 5GC-MT-LR Procedures' in RRC_INACTIVE state as defined in section 6.7 of TS 23.273 [3] is shown in Annex A of this contribution (according to [4]) and is used as baseline for the discussion.

# 2.	SRS for positioning configuration enhancements

The 'SRS for positioning configuration enhancements' item includes the following main objectives (see also section 1):

- SRS for positioning configurations in multiple cells.

- Pre-configuration of one or multiple SRS for positioning configurations.

- SRS for positioning activation/request procedure(s).

RAN2 made some progress on introducing a "validity area" for the SRS for positioning configuration in RRC_INACTIVE state [5],[6] as follows:

Agreements:

When configured with SRS configuration along with SRS validity area, if the UE reselects to another cell within the SRS validity area during SRS transmission, the UE continues the SRS transmission, subject to validation for SRS transmission.

Wait for RAN1 progress for the validation of SRS transmission with issues such as interference, timing advance and spatial relation information, etc.

Agreements:

RAN2 assume when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

LS to RAN3 to confirm this.

Agreement:

The SRS validity area configuration contains a list of cells in which it is valid.  FFS validity timer or if we would depend only on explicit release by the network.

Agreements:

RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE.

Agreement:

SRS configuration request can be indicated via Msg3/MsgA transmission.  FFS if the request is in the RRC message or an accompanying MAC CE.

Agreements:

RAN2 consider that the LMF should determine the area-specific SRS configuration.  Details are up to RAN3.

LS to RAN3 to indicate this conclusion, including RAN1 to prompt them for parameters.  To be included in the LS from [AT122][415].

and on SRS pre-configuration [6]:

Agreement:

RAN2 will introduce an activation indication and/or request for preconfigured SRS using at least Msg3/MsgA; FFS if Msg1 would be supported also.  FFS RRC signalling or MAC CE for the Msg3/MsgA case, as for the configuration request.  This agreement does not imply that the UE will be allowed to transmit autonomously.

For the SRS for positioning configuration in multiple cells, RAN1 made the following agreements [5], [6]:

Agreement For SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support the following: - An SRS positioning validity area consists of cells configured in the same band and the same carrier, and the following parameters with respect to BWP information of SRS for positioning configuration are commonly applied across cells within the validity area: - BWP parameters - locationAndBandwidth - subcarrierSpacing - cyclicPrefix

Agreement - For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, at least the following parameters in SRS for positioning configuration are commonly configured across cells within the validity area: - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping-r16 - resourceType - FFS: whether sequenceId is configured commonly across cells or per cell

Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, sequenceID in SRS for positioning configuration is commonly configured across cells within the validity area.

Working assumption For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, support the following options: - Option 1: Pathloss RS is absent in the configuration. - Option 2: Pathloss RS is provided in the configuration.

Working assumption For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support that spatial relation information can be absent or present in the configuration.

The above agreements can be summarized as follows:

Observation 1:	The UE, configured with SRS with validity area, can continue SRS transmission during cell reselection if the UE reselects to a cell within the validity area.

Observation 2:	When the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

Observation 3:	An SRS for positioning configuration valid in multiple cells has the following common parameter, valid across all cells in the validity area:
-	same BWP (locationAndBandwidth, subcarrierSpacing, cyclicPrefix);
-	same SRS-PosResourceSet parameter (srs-PosResourceSetId, srs-PosResourceIdList,
		resourceType);
-	same SRS-PosResource parameter (srs-PosResourceId, transmissionComb, resourceMapping,
		freqDomainShift, freqHopping, groupOrSequenceHopping, resourceType, sequenceID).

Observation 4:	Pathloss parameter/pathloss reference for SRS may or may not be present in the SRS configuration.

Observation 5:	Spatial relation parameter for SRS may or may not be present in the SRS configuration.

Observation 6:	The UE may send an activation indication and/or request for preconfigured SRS.

The RRC Release is used in Rel-17 to provide the SRS configuration for RRC inactive state, as illustrated in Annex A (Step 10 in Figure A.1). This is because of the SDT Procedure #1 in Figure A.1. When an Event is detected, the UE instigates a SDT procedure for SRB2 and sends an Event Report with a request for SRS configuration to an LMF. The LMF would then configure SRS in the target device like in RRC connected state. The gNB would then provide the SRS configuration for RRC inactive in the RRC Release to complete the SDT procedure. However, to avoid frequent SRS (re)configurations (i.e., every time an Event is detected), the positioning SRS could be pre-configured in the target device (similar to pre-configured DL-PRS assistance data). That is, the target device has the necessary SRS configuration for RRC inactive already configured before an Event is detected. This SRS (pre-)configuration could happen when the deferred MT-LR is configured in the target device as illustrated in Figure 2-1 below. At this stage, the UE would be still in RRC connected state. With pre-configured SRS, the UE would then only need to send an SRS configuration activation request message to the serving gNB to ask for permission to transmit the pre-configured SRS (independent on whether the SRS configuration is associated with an area validity or not). In addition, the SRS pre-configuration could be included in a posSIB, which could also be requested on-demand in any RRC state and provided in RRC Reconfiguration when in connected state.

Figure 2-1: Deferred MT-LR with positioning SRS pre-configuration.

1a.	During the deferred MT-LR configuration phase (e.g., during step 15 of Figure 6.3.1-1 in TS 23.273 [3]), the LMF sends a NRPPa Positioning Information Request message to the serving gNB including a request for preconfigured positioning SRS. The request may include one or more "Requested SRS Transmission Characteristics" IEs [7], each defining a positioning SRS configuration (e.g., Number Of Periodic Transmissions, Resource Type, Bandwidth, Resource Set List, SRS Frequency, etc.).

1b.	The serving gNB determines one or more positioning SRS configurations and provides these configurations to the target device for transmission of positioning SRS by the target device at a later time (i.e., the target device does not transmit any positioning SRS when pre-configured). 
Each positioning SRS configuration has an associated identifier.
Each positioning SRS may have a validity time and/or a validity area.

1c.	The serving gNB provides the set of preconfigured positioning SRS to the LMF (e.g., one or more "SRS Configuration" IEs [7] where each positioning SRS has an associated ID, and probably an validity area indication).

NOTE 1:	During the deferred MT-LR initiation phase, an LMF may also provide DL-PRS assistance data to the target device for UL+DL positioning. The pre-configured DL-PRS assistance data may consist of multiple instances, where each instance may be applicable to a different area within the network.

NOTE 2:	A target device may obtain the DL-PRS assistance data and/or UL-SRS (pre-)configuration also via posSI.

NOTE 3:	The SRS for positioning validity area and the validity area for the pre-configured DL-PRS assistance data are independent of each other.

3.	After (or slightly before) an event is detected, the UE sends an RRC Resume Request to the receiving gNB  along with an "SRS Activation Request" MAC-CE. 
The "SRS Activation Request" may include the IDs of a desired preconfigured SRS configuration (e.g., in priority order) to be activated.

4.	The receiving gNB identifies the last serving gNB using the I-RNTI and retrieves the UE context (incl. the preconfigured positioning SRS information) by means of Xn-AP Retrieve UE Context procedure.

The receiving gNB determines the positioning SRS configuration based on the pre-configuration during step 1. The receiving gNB may determine the Pathloss Reference Information (e.g., alpha, p0, pathlossReferenceRS-Pos [8]) or Spatial Relation Information (e.g., SRS-SpatialRelationInfoPos [8]) for the positioning SRS valid for the receiving gNB. The receiving gNB may also determine the TA timer and RSRP-change-threshold. The receiving gNB uses the assistance information received from the LMF at Step 1a to determine the above SRS parameter.

NOTE 4:	For the spatial relation/pathloss reference information of an SRS for positioning configuration in multiple cells, RAN1 is considering two options (Observations 4/5):
Option 1: Spatial relation/pathloss reference information is absent in the pre-configuration.
Option 2: Spatial relation/pathloss reference information is provided in the pre-configuration.

5.	The receiving gNB may then send an "SRS Activation" message to the UE which includes the ID of the preconfigured SRS to be activated, and possibly the pathloss reference, spatial relation, the TA timer and RSRP-change-threshold.

6.	The receiving gNB sends a NRPPa Positioning Information Update message to the LMF including the ID of the activated positioning SRS and the cell ID of the current UE serving cell.

7.	The LMF sends a NRPPa Measurement Request message to a group of gNBs/TRPs incl. the positioning SRS measurement configuration (i.e., based on the ID received at Step 6). The group of gNBs/TRPs for the UL measurements may be selected based on the current UE serving cell ID as received at Step 6.

8.	The receiving gNB sends an RRC Release message to the UE to release the UE to RRC_INACTIVE state. If step 5 did not occur, the RRC Release message includes the SRS Activation message.

Observation 7:	The pre-configuration of positioning SRS can occur during the location preparation phase of the 'Low Power Periodic and Triggered 5GC-MT-LR Procedures' (e.g., together with the pre-configuration of DL-PRS assistance data, if applicable). One or more positioning SRS configurations can be provided to the gNB/UE during the location preparation phase using modified NRPPa Positioning Information Exchange procedures.

Proposal 1:	RRC Reconfiguration can be used to provide SRS (pre-)configuration with or without validity area for use in RRC_INACTIVE.

Proposal 2:	posSI can be used to provide SRS (pre-)configuration with or without validity area for use in RRC_INACTIVE.

Proposal 3:	Define an "SRS for positioning Activation Request/Indication" UL MAC-CE for pre-configured SRS which can include an identifier of the SRS for positioning configuration requested to be activated (possibly a list in preferred order).

Proposal 4:	Define an "SRS for positioning Activation" DL MAC-CE for pre-configured SRS which can include an identifier of the allowed/activated SRS for positioning configuration together with the SRS for positioning configuration parameter which were not pre-configured (e.g., pathloss reference, spatial relation), or which overwrite pre-configured SRS for positioning parameter.

Proposal 5:	The target device starts transmitting the (pre-configured) SRS for positioning once the "SRS for positioning Activation" DL-MAC CE has been received.

Proposal 6:	The serving gNB of the target device sends a NRPPa message to the LMF once the SRS for positioning has been activated in the target device. This NRPPa message carries information on the activated SRS for positioning and the current serving cell ID to enable the LMF to request UL measurements from TRPs.

The 'SRS deactivation request' could be UE triggered (e.g., included at Step 12 in Figure A.1 when measurements are completed) or LMF triggered (e.g., after Step 14 in Figure A.1 when TRP measurements have been received). For LMF triggered deactivation, the NRPPa Positioning Deactivation [7] procedures could be reused as shown in Figure 2-2 below.

For UE triggered deactivation request, the UE could send an UL-MAC-CE deactivation request along with the RRC Resume Request at Step 12 in Figure 2-3. The receiving gNB could then inform the LMF using the NRPPa Positioning Information Update message as shown in Figure 2-3 below.

Figure 2-2: Deferred MT-LR with LMF-triggered positioning SRS deactivation request.

Figure 2-3: Deferred MT-LR with UE-triggered positioning SRS deactivation request.

Proposal 7:	Define a NRPPa message which allows an LMF to request SRS for positioning deactivation from the target device's serving gNB.

Proposal 8:	Define a UL MAC-CE which allows the UE to request SRS for positioning deactivation from the serving gNB.

Proposal 9:	Define a DL MAC-CE for 'SRS for positioning deactivation'. The target device stops transmitting the indicated SRS for positioning when the message has been received.

# 3.	Alignment between eDRX and PRS configurations

## 3.1	eDRX alignment with configured DL-PRS

The AMF and/or NG-RAN could take the configured PRS information (e.g., PRS periodicity) and configured UE reporting activity (e.g., periodic, area, or motion event) and positioning method (e.g., UL-only, DL-only, UL+DL) into account when configuring eDRX in the target device.

For RRC_INACTIVE state, the AMF already provides assistance information for RRC state transition and (e)DRX information to the NG-RAN in the IE 'Core Network Assistance Information for RRC INACTIVE' [9]. An LMF could provide the necessary assistance to an AMF as illustrated in Figure 3.1-1 (Option 1, Step 18). The LCS Activity information could include the information of the configured (deferred) location request (e.g., PeriodicEventInfo, AreaEventInfo, MotionEventInfo, configured positioning method, etc.) together with PRS information (e.g., periodicity). The AMF could then include the LCS specific information received from an LMF in the 'Core Network Assistance Information for RRC INACTIVE' in Step 19a, Option 1 in Figure 3.1-1. The AMF/NG-RAN Node may then use this information to decide on a suitable RRC state and (e)DRX cycle, taking the expected UE reporting interval/time and PRS periodicity into account.

Figure 3.1-1 also illustrates the option where the LMF provides the assistance information directly to the NG-RAN using NRPPa (Option 2, Step 19a). However, this has the disadvantage that CN paging (e)DRX for RRC_IDLE can not be considered for the alignment with PRS (since the NRPPa message is transparent to an AMF and the (e)DRX configuration for RRC_INACTIVE and RRC_IDLE are independent). Also, all the information needed from the CN for RRC state transition would be in a single IE/procedure if the 'Core Network Assistance Information' is also used for LCS/positioning.

Proposal 10:	For the (e)DRX alignment with configured DL-PRS, support adding the UE configured LCS reporting activity and configured DL-PRS information (e.g., DL-PRS periodicity) to the IE 'Core Network Assistance Information for RRC INACTIVE' (TS 38.413). Inform RAN3 and SA2 on RAN2's decision.

Figure 3.1-1: Initiation of Deferred MT-LR with LMF assistance to the AMF (Option 1) or NG-RAN (Option 2).

## 3.2	DL-PRS alignment with configured eDRX

In section 3.1 above, the LMF provides assistance information to the AMF/NG-RAN to assist the NG-RAN with the state transition to RRC_INACTIVE state and to assist the AMF/NG-RAN with possible alignment of eDRX with already configured DL-PRS.

The opposite scenario, where the UE/NG-RAN/AMF assists the LMF to configure DL-PRS aligned with already configured eDRX is also possible but seems generally an anti-causal problem.  The DL-PRS Assistance Data are typically configured while the UE is in RRC_CONNECTED state (e.g., during Step 1 in Figure A.1 (see Annex A), or step 15 in Figure 3.1-1 above), or may receive the assistance data via broadcast (posSI). At this point of the procedure, an LMF does not know whether the NG-RAN will move the UE to RRC_INACTIVE or RRC_IDLE and if so, may not know any eDRX configuration information which can be used for possible alignment with DL-PRS.

In addition, the "radio capabilities" (e.g., eDRX in this case) are usually selected based on "service requirements" (e.g., LCS activity) and not the other way around (i.e., adapt the service request (e.g., LCS reporting activity or DL-PRS) to the eDRX).

For e.g., CN buffering of MT data when the UE is in RRC_INACTIVE state with long eDRX [10], RAN3 made already an agreement to use a Class 1 procedure to report to the AMF when the UE enters RRC_INACTIVE state with eDRX > 10.24s [11]. Although, RAN3 clarified that this new procedure is for eDRX cycle lengths larger than 10.24s [12], this new procedure could be reused for providing eDRX information from the AMF to the LMF, as illustrated in Figure 3.2-1, Option 1.  Note, a new NG-RAN triggered NRPPa procedure may also possible (Option 2 in Figure 3.2-1), but similar to the discussion in section 3.1 above may not provide any CN DRX information for idle mode to the LMF. However, as indicated in [13], it is up to NG-RAN implementation whether and when to send this N2 message with eDRX indication to the AMF.

Figure 3.2-1: Providing eDRX information to the LMF.

Therefore, a more reliable solution would be that a UE informs the LMF about configured (e)DRX (either directly or indirectly), e.g., at Step 3 or 12 in Figure A.1 using LPP. A UE would always know the periodic or triggered location request configuration (i.e., when a report would be needed) and all configured (e)DRX information independent on RRC/CM state. The LMF may then take the (e)DRX information into account when requesting DL-PRS from the TRPs using the on-demand DL-PRS procedures [7] as illustrated in Figure 3.2-2.

The LPP Request Assistance Data for on-demand DL-PRS can already include a desired DL-PRS periodicity [14]. A UE may determine the desired DL-PRS periodicity and set the dl-prs-ResourceSetPeriodicityReq in IE NR-On-Demand-DL-PRS-Information accrding to its (e)DRX cycle and configured LCS activity.  Any additional parameter (e.g., slot offset) if needed could be easily added.

Proposal 11:	For the DL-PRS alignment with configured (e)DRX, the LPP Request Assistance Data for on-demand DL-PRS is used. The UE may set the dl-prs-ResourceSetPeriodicityReq according to the configured LCS reporting activity and (e)DRX configuration. FFS whether additional parameter are needed (e.g., DL-PRS slot offset).

Figure 3.2-2: On-demand DL-PRS procedure for aligning DL-PRS with UE DRX Active Time.

# 4.	Summary

In this contribution, we discussed the remaining issues for LPHAP. The following observation and proposals were made.

SRS for positioning configuration enhancements

Observation 1:	The UE, configured with SRS with validity area, can continue SRS transmission during cell reselection if the UE reselects to a cell within the validity area.

Observation 2:	When the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

Observation 3:	An SRS for positioning configuration valid in multiple cells has the following common parameter, valid across all cells in the validity area:
-	same BWP (locationAndBandwidth, subcarrierSpacing, cyclicPrefix);
-	same SRS-PosResourceSet parameter (srs-PosResourceSetId, srs-PosResourceIdList,
		resourceType);
-	same SRS-PosResource parameter (srs-PosResourceId, transmissionComb, resourceMapping,
		freqDomainShift, freqHopping, groupOrSequenceHopping, resourceType, sequenceID).

Observation 4:	Pathloss parameter/pathloss reference for SRS may or may not be present in the SRS configuration.

Observation 5:	Spatial relation parameter for SRS may or may not be present in the SRS configuration.

Observation 6:	The UE may send an activation indication and/or request for preconfigured SRS.

Observation 7:	The pre-configuration of positioning SRS can occur during the location preparation phase of the 'Low Power Periodic and Triggered 5GC-MT-LR Procedures' (e.g., together with the pre-configuration of DL-PRS assistance data, if applicable). One or more positioning SRS configurations can be provided to the gNB/UE during the location preparation phase using modified NRPPa Positioning Information Exchange procedures.

Proposal 1:	RRC Reconfiguration can be used to provide SRS (pre-)configuration with or without validity area for use in RRC_INACTIVE.

Proposal 2:	posSI can be used to provide SRS (pre-)configuration with or without validity area for use in RRC_INACTIVE.

Proposal 3:	Define an "SRS for positioning Activation Request/Indication" UL MAC-CE for pre-configured SRS which can include an identifier of the SRS for positioning configuration requested to be activated (possibly a list in preferred order).

Proposal 4:	Define an "SRS for positioning Activation" DL MAC-CE for pre-configured SRS which can include an identifier of the allowed/activated SRS for positioning configuration together with the SRS for positioning configuration parameter which were not pre-configured (e.g., pathloss reference, spatial relation), or which overwrite pre-configured SRS for positioning parameter.

Proposal 5:	The target device starts transmitting the (pre-configured) SRS for positioning once the "SRS for positioning Activation" DL-MAC CE has been received.

Proposal 6:	The serving gNB of the target device sends a NRPPa message to the LMF once the SRS for positioning has been activated in the target device. This NRPPa message carries information on the activated SRS for positioning and the current serving cell ID to enable the LMF to request UL measurements from TRPs.

Proposal 7:	Define a NRPPa message which allows an LMF to request SRS for positioning deactivation from the target device's serving gNB.

Proposal 8:	Define a UL MAC-CE which allows the UE to request SRS for positioning deactivation from the serving gNB.

Proposal 9:	Define a DL MAC-CE for 'SRS for positioning deactivation'. The target device stops transmitting the indicated SRS for positioning when the message has been received.

Alignment between eDRX and PRS configurations

Proposal 10:	For the (e)DRX alignment with configured DL-PRS, support adding the UE configured LCS reporting activity and configured DL-PRS information (e.g., DL-PRS periodicity) to the IE 'Core Network Assistance Information for RRC INACTIVE' (TS 38.413). Inform RAN3 and SA2 on RAN2's decision.

Proposal 11:	For the DL-PRS alignment with configured (e)DRX, the LPP Request Assistance Data for on-demand DL-PRS is used. The UE may set the dl-prs-ResourceSetPeriodicityReq according to the configured LCS reporting activity and (e)DRX configuration. FFS whether additional parameter are needed (e.g., DL-PRS slot offset).

# References

[1]		RP-231460, "Revised WID on Expanded and Improved NR Positioning".

[2]		RP-222982, "Status Report to TSG".

[3]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[4]	R2-2203949, "LS to SA2 on Positioning in RRC_INACTIVE State", RAN2.

[5]		RP-230327, "Status Report to TSG", RAN#99.

[6]		RP-231092, "Status Report to TSG", RAN#100.

[7]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[8]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[9]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[10]	R2-2211136, "LS On long eDRX support for RRC_INACTIVE", SA2.

[11]	R3-226776, "Reply LS on long eDRX support for RRC_INACTIVE", RAN3.

[12]	R2-2302059, "Reply LS on long eDRX support for RRC_INACTIVE", RAN3.

[13]	S2-2301858, "Reply LS on long eDRX support for RRC_INACTIVE", SA2.

[14]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

# Annex A:	Low Power Deferred MT-LR Procedure with Event Reporting in RRC_INACTIVE state for UL+DL Positioning

Figure A.1: Low Power Deferred MT-LR Procedure with Event Reporting in RRC_INACTIVE state for UL+DL Positioning.

1.	Steps 1-21 for the deferred 5GC-MT-LR procedure for periodic or triggered location events specified in TS 23.273, clause 6.3.1 are performed.

The LMF may perform one or more positioning procedures at Step 15 of TS 23.273, clause 6.3.1 to request and obtain the UE positioning capabilities or provide any necessary assistance data to the target device.

The LCS Periodic-Triggered Location Invoke at Step 16 of TS 23.273, clause 6.3.1 includes an embedded LPP Request Location Information message which indicates the allowed or required Multi-RTT location measurements for each location event reported.

The UE is released by the last serving gNB from RRC_CONECTED to RRC_INACTIVE by RRCRelease with SuspendConfig.

2.	The UE monitors for occurrence of the trigger or periodic event requested during step 1.

3.	When an event is detected (or slightly before) the UE sends an RRC UL Information Transfer message containing an UL NAS Transport message along with the RRC Resume Request with SDT. The UE includes an LCS Event Report in the payload container of the UL NAS Transport message, and the Deferred Routing Identifier received during Step 1 in the Additional Information of the UL NAS Transport message as defined in TS 24.501.

The LCS Event Report includes an embedded LPP Request Assistance Data message with IE NR-Multi-RTT-RequestAssistanceData and nr-AdType set to 'ul-srs' to request an UL-SRS for Multi-RTT positioning as specified in TS 37.355.

NOTE:	The receiving gNB of the UE when UE performs step 3 might be the same or different from the last serving gNB where the UE is released to the RRC_INACTIVE state.

4.	The receiving gNB sends the LCS Event Report with the LPP Request Assistance Data message in an NGAP Uplink NAS Transport message to the serving AMF. The AMF determines the LMF from the Deferred Routing Identifier received in the Additional Information IE of the UL NAS TRANSPORT message and forwards the LCS Event Report with embedded LPP message via triggering Namf_Communication_N1MessageNotify service operation towards the LMF. The AMF also includes the Payload Container Type and the Correlation Identifier set to the Deferred Routing Identifier.

NOTE:	If the anchor gNB is not changed from the last serving gNB to the receiving gNB, the LCS event report is forwarded form the receiving gNB to the last serving gNB via XnAP message RRC TRANSFER as in TS 38.423. Subsequent downlink/uplink messages are also forwarded between last serving gNB to the receiving gNB via XnAP message RRC TRANSFER.

5.	The LMF sends a NRPPa Positioning Information Request message to the receiving gNB to request UL-SRS for the target device.

6.	The receiving gNB determines the resources available for UL-SRS.

7.	The receiving gNB provides the UL-SRS configuration information to the LMF in a NRPPa Positioning Information Response message.

8.	The LMF sends a NRPPa Measurement Request to a group of gNBs incl. the UL-SRS measurement configuration.

9.	The LMF sends a SS LCS Event Report Acknowledgement to the receiving gNB. The receiving gNB then provides the SS Event Report Acknowledgement to the UE at Step 9b via Subsequent DL SDT.

10.	The receiving gNB sends a RRCRelease message with suspendConfig to keep the UE in RRC_INACTIVE state. The RRCRelease message includes the UL-SRS Configuration.

11.	The UE performs DL-PRS measurements and each configured TRP performs UL-SRS measurements.

12.	The UE sends an RRC UL Information Transfer message containing an UL NAS Transport message along with the RRC Resume Request with SDT. The UE includes the LCS Event Report and LPP Provide Location Information message in the payload container of the UL NAS Transport message, and the Deferred Routing Identifier received during Step 1 in the Additional Information of the UL NAS Transport message as defined in TS 24.501.

13.	The receiving gNB sends the LCS Event Report with the LPP Provide Location Information message in an NGAP Uplink NAS Transport message to the serving AMF. The AMF determines the LMF from the Deferred Routing Identifier received in the Additional Information IE of the UL NAS TRANSPORT message and forwards the LCS Event Report with embedded LPP message via triggering Namf_Communication_N1MessageNotify service operation towards the LMF. The AMF also includes the Payload Container Type and the Correlation Identifier set to the Deferred Routing Identifier.

14.	After performing the UL-SRS measurements, the gNBs provide the UL measurements to the LMF in a NRPPa Measurement Response message.

15. When all LPP Provide Location Information messages have been received, the LMF sends a SS LCS Event Report Acknowledgement to the receiving gNB. The receiving gNB then provides the SS Event Report Acknowledgement to the UE at Step 15b via Subsequent DL SDT.

16.	The receiving gNB sends a RRCRelease message with suspendConfig to keep the UE in RRC_INACTIVE state.

17.	Steps 28-31 for the deferred 5GC-MT-LR procedure for periodic or triggered location events specified in TS 23.273, clause 6.3.1 are performed.