3GPP RAN WG2 Meeting #123	R2-2308524

Toulouse, France August 21st – 25th, 2023

Agenda Item:	7.7.4.1.1

Source:	InterDigital

Title:	NTN-TN mobility and service continuity

Document for:	Discussion, Decision

# Introduction

Adaptations to NR mobility must address the following issues in NTN: 1) increased signalling latency due to large propagation delay; 2) frequent cell change due to satellite movement; and 3) lower robustness of measurement-based mobility due to reduced signal strength variation between cell center and cell edge and large regions of cell overlap. Rel-17 NR NTN partially addressed these issues via enhancements to conditional handover, measurement reporting, and measurement rules for cell (re)selection.

An objective for Rel-18 NR NTN is to continue enhancements for both NTN-NTN and NTN-TN mobility and service continuity. Specifically, cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized).

The following agreements have been made in recent meetings regarding mobility enhancements for TN-NTN mobility and service continuity in Rel-18 NTN. FFS topics which are still open are highlighted in yellow below:

In RAN2#121bis-e [1], the following agreements relevant to NTN-TN mobility were made:

- For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one). FFS on the SIB. FFS on whether additional information in dedicated signalling is needed/useful

- Area center location and its radius for TN coverage information is signalled using Ellipsoid-Point and radius separately. FFS if Rel-17 referenceLocation and distanceThresh are directly reused

- Decision on the size of TN coverage area list is postponed until more is known on the format of this information and how is it sent.

- The discussion on how to indicate the frequency information for each TN coverage area should be combined with the discussion on which SIB will be used to indicate the TN coverage area, possibly based on evaluation of the signalling overhead

- The acquired TN area coverage information remains valid until the next system information update of the SIB including TN coverage info

- On a frequency band number shared by TN and NTN (e.g., n1), if NTN-specific assistance information is NOT provided for a neighbour cell configured in SIB3/SIB4, UE assumes this is a TN neighbour cell. This understanding is also applicable for Rel-17 and it does not need any spec update

And in RAN2#122 [2], the following agreements relevant to NTN-TN mobility were made:

- An RRC_IDLE/RRC_INACTIVE UE is not required to perform neighbour cell measurements for cell reselection for a TN frequency in the area, if configured, where there is no coverage of that frequency, regardless of the frequency priority

- Reuse the same format of Rel-17 referenceLocation and distanceThresh for signaling the TN coverage area centre and radius

- TN coverage info is NOT included in SIB19. FFS if we use an existing SIB or a new one

- We don’t introduce RRC dedicated signalling to provide more accurate TN coverage information

- We no longer consider option 3 alone for signaling the frequency information for TN coverage area (in case option 3 should be combined with option 1). Come back in the next meeting to decide between option 2 (plus possible fixes if needed) and option 1+3.

# NTN-TN mobility: Remaining open issues

## Which SIB is used to signal TN coverage information

Assistance information including a geographic description of TN coverage and associated frequencies is broadcast to support TN measurement relaxation. It is currently FFS whether this information is carried in a new SIB or an existing one.

A key issue with reusing an existing SIB is the size of the TN coverage information, which could be significant. Taking the RAN2#121 meeting location as an example, a 1000km diameter cell covers the entirety of Greece (see figure 1 in R2-2301365), which contains 66 cities with populations greater than 15,000 people. Describing a TN coverage area requires 8 bytes (distanceThresh (2 bytes) + referenceLocation (6 bytes)). Using 8 bytes per city would still require 528 bytes and would exclude any TN coverage within rural areas or towns with less than 15,000 people. This also neglects any signalling needed to provide the list of associated TN frequencies.

Observation 1:	The size of TN coverage information for NTN cells with even modest diameter can be hundreds of bytes, which may not be suitable to add to existing SIBs.

Considering the possible size of TN coverage information, the cleanest solution would be to introduce a new SIB to broadcast the TN coverage information.

Proposal 1:	Introduce a new SIB to broadcast TN coverage information.

## Size of the TN coverage area list

Another postponed issue is the number of TN coverage areas which can be broadcast. A main factor to consider is the large difference between supported cell sizes and required accuracy. For example, a LEO with 100km diameter may entirely cover a single TN area, whereas GEO with 3500km diameter can cover multiple countries.

Observation 2:	NTN cell sizes range from 100 km to 3500 km. The diversity of deployment scenarios will require a significantly different number of TN coverage areas to be broadcast.

When evaluating the maximum number of supported TN coverage areas, RAN2 should use the worst case scenario (i.e., GEO with 3500km diameter) to determine the maximum list size.

Proposal 2:	RAN2 considers the largest supported NTN cell (i.e., GEO with 3500km diameter) when determing the maximum number of TN coverage areas.

## How to indicate frequency information for TN coverage area

If an IDLE/INACTIVE UE is outside of a TN coverage area, the UE is not required to perform neighbour cell measurements for cell reselection on a TN frequency in the area. It is currently FFS how this frequency information is indicated. This topic was most recently discussed in RAN2#122 offline discussion [103], which evaluated the following options:

- Option 1: Frequency information (i.e. a list of TN frequencies) for each TN coverage area is indicated directly under each TN coverage area configuration.

- Option 2: A TN coverage area configuration is associated with a TN coverage Area ID. The frequency information for TN coverage area is indicated by adding TN coverage area IDs in SIB4 and SIB5.

- Option 3: Frequency index bitmap is indicated under each TN coverage area, where the frequency index refers to the frequency’s position in the frequency list of the current SIB4.

Discussion conclusion resulted in downscoping potential options to Option 2 or a combination of Option 1+3.

Combining Options 1 and 3 creates a flexible and signalling efficient solution which uses information already known to the UE (via SIB4), as well as enabling direct indication of frequencies not present in SIB4. Considering the number of frequencies in the InterFreqCarrierFreqList is 8 (given by maxFreq), an additional frequency index bitmap indicated under each TN coverage area would only require an additional byte per area, which is an acceptable amount of additional overhead.

Observation 3:	Providing frequencies per TN coverage area via Option 1+3 is signalling efficient (by using information already provided in SIB4) as well as flexible (by allowing additional frequencies to be directly indicated).

Observation 4:	A frequency index bitmap referencing the current SIB4 frequency list would only require an additional byte per TN coverage area

Agreeing to a new TN coverage information SIB (i.e., Proposal 1) would further support this solution since it enables TN coverage information and associated frequency information to be self contained within the new SIB, avoiding impact to legacy system information and simplifying the overall procedure.

Proposal 3:	A frequency index bitmap is indicated under each TN coverage area, where the frequency index refers to the frequency’s position in the frequency list of the current SIB4. If frequency information for a TN coverage area is not present in SIB4, frequency information is indicated directly in the new TN coverage SIB (i.e., Option 1 + 3).

SIB4 contains information relevant for inter-frequency cell re-selection (i.e. information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection). Similarly, SIB5 contains information relevant for inter-RAT cell re-selection (i.e. information about E-UTRA frequencies and E-UTRAs neighbouring cells relevant for cell re-selection).

Like SIB4, the number of SIB5 frequencies in CarrierFreqListEUTRA is 8 (given by maxEUTRA-Carrier), so an additional frequency index bitmap indicated under each TN coverage area would require an additional byte to support indication of E-UTRA carrier frequencies (in addition to NR frequencies)

Observation 5:	An additional frequency index bitmap referencing the current SIB5 frequency list would require an additional byte per TN coverage area to add information about E-UTRA carrier frequencies.

RAN2 should discuss whether to also include an additional frequency index bitmap which refers to SIB5.

Proposal 4:	RAN2 to discuss whether to support an additional frequency index bitmap under each TN coverage area where the index refers to the SIB5 frequency list.

# Conclusion

In this contribution the following observations and proposals are made concerning NTN-TN mobility and service continuity:

Observation 1:	The size of TN coverage information for NTN cells with even modest diameter can be hundreds of bytes, which may not be suitable to add to existing SIBs.

Observation 2:	NTN cell sizes range from 100 km to 3500 km. The diversity of deployment scenarios will require a significantly different number of TN coverage areas to be broadcast.

Observation 3:	Providing frequencies per TN coverage area via Option 1+3 is signalling efficient (by using information already provided in SIB4) as well as flexible (by allowing additional frequencies to be directly indicated).

Observation 4:	A frequency index bitmap referencing the current SIB4 frequency list would only require an additional byte per TN coverage area

Observation 5:	An additional frequency index bitmap referencing the current SIB5 frequency list would require an additional byte per TN coverage area to add information about E-UTRA carrier frequencies.

Proposal 1:	Introduce a new SIB to broadcast TN coverage information.

Proposal 2:	RAN2 considers the largest supported NTN cell (i.e., GEO with 3500km diameter) when determing the maximum number of TN coverage areas.

Proposal 3:	A frequency index bitmap is indicated under each TN coverage area, where the frequency index refers to the frequency’s position in the frequency list of the current SIB4. If frequency information for a TN coverage area is not present in SIB4, frequency information is indicated directly in the new TN coverage SIB (i.e., Option 1 + 3).

Proposal 4:	RAN2 to discuss whether to support an additional frequency index bitmap under each TN coverage area where the index refers to the SIB5 frequency list.

# References

R2-2306553 - RAN2#121bis-e Meeting Report

Draft_R2_122_meeting_report_v1