3GPP TSG-RAN WG2 Meeting #123											R2-2308561

Toulouse, France, 21– 25 August, 2023

Agenda item:       5.2

Source: 	Huawei, HiSilicon

Title: 	Summary on NR V2X RRC corrections

Document for:	Discussion and Decision

# Introduction

This document summarizes the contributions submitted to AI 5.2 on RRC corrections. Rapporteur’s recommendations on how to treat them are provided.

# Correction CRs to be discussed together

The following correction CRs could be discussed together. The agreed changes for RRC spec could be merged into a Rapporteur’s miscellaneous Rel-16 correction CR and Rel-17 mirror CR.

Table 1: Correction CRs

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2307096 | Correction on PUCCH resource field description and SSB transmission initiation | OPPO | 1. In 6.3.5, Currently the field description for sl-N1PUCCH-AN and sl-N1PUCCH-AN-Type2 describes the IE as “HARQ resource for PUCCH”, which is misleading since the two IEs indicates PUCCH resource carrying HARQ A/N feedback. 2. In 5.8.5.2, for out of coverage case, different from in-coverage case, the clause of 5.8.5.3 is not mentioned, yet clause 5.8.5.3 also includes the case for out-of-coverage, so it is not clear why the misalignement between the two cases is needed. 1. In 6.3.5, correct the field description from “HARQ resource for PUCCH” to “PUCCH resource for HARQ feedback” 2. In 5.8.5.3, for out of coverage case, add the reference to “5.8.5.3” to align with in-coverage case as well. | Rapp thinks the two changes bring more clarity to the existing texts. |
| R2-2307753 | Correction on NR Sidelink RRC | Philips International B.V. | 1. In clause 6.3.5 Sidelink information elements, the sl-RNTI field in the IE SL-ScheduledConfig has an error in the field description which refers to C-RNTI. Instead, it shall refer to the SL-RNTI. | Rapp understands the motivation for this change, however the actual change can be discussed e.g. to not lose the implication of "dynamic grant" by "C-RNTI". |

Proposal 1: Discuss the CRs listed in Table 1 offline, the agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR.

# Correction CRs to be discussed separately

The following correction CRs (and discussion papers) can be treated online first.

Table 2: Correction CRs

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2307562 | Potential issue caused by using destination index | Huawei, HiSilicon, vivo, Ericsson | Proposal 1: RAN2 to agree the companion CRs to solve the issue for dedicated SL configuration using destination index. | The NOTE based solution was not fully discussed during last meeting. |
| R2-2307563 | Correction on destination index for SL measurement configuration | Huawei, HiSilicon, vivo, Ericsson |  |  |
| R2-2307565 | Correction on destination index for SL DRX configuration | Huawei, HiSilicon, vivo, Ericsson |  |  |
| R2-2308933 | Correction on carrier frequency for SL-RSRP measurement | Huawei, HiSilicon |  |  |
| R2-2308107 | Discussion on LS from RAN5 | ZTE Corporation, Sanechips | - Send the reply LS to RAN5 to clarify that interpretation-1 is correct: This field is not the indication of the central frequency of measurement resources since NR SL-RSRP measurement is based on PSSCH-DMRS. this interpretation and current specification does not require TX UE to keep sending PSSCH-DMRS. - RAN2 is suggested to agree the CR(R2-2308105 for R16 and R2-2308106 for R17). |  |
| R2-2308105 | Correction on sidelink measurement | ZTE Corporation, Sanechips |  | The changes on the procedure texts can be agreeable as they may bring more clarity. On the FD of frequencyinfoSL, according to the contact of said RAN5 LS, more clarification is still needed on the actual field value. |

Proposal 2: Discuss the CRs/discussion papers listed in Table 2 online first.

# Conclusions

The contributions submitted to AI 5.2 are summarized, Rapporteur’s recommendations are as follows.

Proposal 1: Discuss the CRs listed in Table 1 offline, the agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR.

Proposal 2: Discuss the CRs/discussion papers listed in Table 2 online first.