3GPP TSG RAN WG2 #123	                                R2-2308578

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	7.6.3.1

Source:	InterDigital, Inc.

Title:	Open issues on mobility enhancements (not covered by [Post122][112]).

Document for:	Discussion

# Introduction

The WI on IoT NTN enhancements [1] includes the following objectives for mobility enhancement:

The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4].

In RAN2#122 the following agreements were made [2]:

Agreements: - For earth-fixed cells, introduce t-ServiceStart for neighbor cells. If UE is aware of the t-ServiceStart of the neighbour cell then may be used (up to UE implementation) to determine when to start measurements of that neighbor cell - If the serving cell t-service expires, stop T310 (if running) and start T311 (i.e. perform cell search and re-establishment without attempting to recover on the current cell for the duration of T310). FFS on discontinuous coverage - The distance between the UE and a second reference location (e.g. within a neighbour cell) is not taken into account. - R18 location and time based trigger for measurements (for connected mode and for idle) apply to both NB-IoT and eMTC.

In addition the following proposal was discussed but postponed [3]

Proposal 1: RAN2 to discuss whether to allow NTN CHO events to trigger independently. - Postponed to the next meeting

In this contribution we elaborate on the remaining issues for NB-IoT and eMTC NTN mobility enhancement in Rel-18 which were not treated as part of [Post122][112] [4].

# Neighbour cell measurements before RLF in NB-IoT

As per the agreements in the previous meeting, the IoT-NTN UE in R18 may use t-ServiceStart for neigbour cells to determine when to start measurement for that neighbour cell. In addition, if the serving cell t-service expires then UE stops T310 (if running) and starts T311 (i.e. to perform the re-establishment), for example as shown in figure 1.

Figure 1: RLF enhancement for IoT-NTN in Rel-18 overlapping coverage

For the discontinuous coverage case it is FFS what behaviour to apply. We think similar behaviour can apply for discontinuous coverage. By taking into account the serving cell t-service as well as the neighbour cell t-service start in the same way as agreed for overlapping coverage case, the coverage gap can be accounted for naturally, for example as shown in figure 2.

Figure 2: RLF enhancement for IoT-NTN in Rel-18 discontinuous coverage

In the discontinuous coverage case, similar to the overlapping coverage case, there is no need to use T310 in order to attempt recovery on cell 1, because cell 1 is known to have stopped. The only real difference between the cases is the start time of the new cell (cell 2). Since this occurs after a coverage gap, and not before t-Service of the serving cell, it makes no sense to trigger measurements before t-Service, and it makes no sense to start T311 immediately upon t-Service. Rather, T311 should start and measurements should be triggered after t-ServiceStart of the new cell.

One of the agreements from the previous meeting allows the UE to trigger measurements only after the coverage gaps: If UE is aware of the t-ServiceStart of the neighbour cell then may be used (up to UE implementation) to determine when to start measurements of that neighbor cell

In other words, if the UE knows the t-ServiceStart is after a coverage gap, then UE may avoid triggering measurements before RLF and trigger only after the coverage gap.

Regarding RLF and re-establishment handling, the agreement from the previous meeting is as follows:

- If the serving cell t-service expires, stop T310 (if running) and start T311 (i.e. perform cell search and re-establishment without attempting to recover on the current cell for the duration of T310). FFS on discontinuous coverage

The first part should also apply to discontinuous coverage, i.e. : If the serving cell t-service expires, stop T310 (if running)

The second part should be modified to support discontinuous coverage: and start T311 (i.e. perform cell search and re-establishment without attempting to recover on the current cell for the duration of T310) upon t-ServiceStart of the upcoming cell.

Proposal 1: For discontinuous coverage: If the serving cell t-service expires, stop T310 (if running), and start T311 (i.e. perform cell search and re-establishment without attempting to recover on the current cell for the duration of T310) upon t-ServiceStart of the upcoming cell.

# CHO Enhancements

## 3.1	Background

In RAN2#120 some agreements were made regarding conditional reconfiguration, as follows:

- CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.: UE is allowed to perform HO only during T1 to T2. - For eMTC NTN, introduce a location-based conditional reconfiguration trigger based on condEventD1 in NR, where the event will be satisfied if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold. (similar to condEventD1 in NR) - For eMTC NTN, introduce event A4 based conditional trigger (similar to condEventA4 in NR). - FFS whether time and location-based trigger conditions may be configured independently (i.e., without a jointly configured event A4 measurement condition) for eMTC NTN.

In addition the following proposal was discussed but postponed [3]

Proposal 1: RAN2 to discuss whether to allow NTN CHO events to trigger independently. - Postponed to the next meeting

## 3.2	 Discussion

In our opinion, the first agreement from RAN2#120 on time based trigger should be clarified such that the stage 3 behaviour is more clear and NR behaviour can be used as a baseline.

Proposal 2: For eMTC NTN, clarify that the agreed time-based conditional reconfiguration trigger is based on condEventT1 in NR, where the event will be satisfied if conditional handover execution occurs between T1 and T2, where T2 = T1 + a duration (similar to condEventT1 in NR)

There also remains an FFS whether time and location-based trigger conditions may be configured independently (i.e., without a jointly configured event A4 measurement condition) for eMTC NTN, which was also postponed from RAN2#122.

For NR, both time and location-based trigger conditions must be simultaneously configured with a measurement condition (e.g. A4) in Rel-17 NTN. For eMTC, since power saving is one of the main considerations, we may need to allow only time based triggers to be enabled by the network for some devices or scenarios such that the UE does not necessarily need to fulfil e.g. RSRP threshold if configured with location and/or time based triggers only.

Proposal 3: Time and location-based trigger conditions may be configured independently (i.e., without a jointly configured measurement condition) for eMTC NTN.

# Conclusion

In this contribution we discuss the potential measurements before RLF for R18 IoT-NTN and have the following proposals:

For RLF enhancement:

Proposal 1: For discontinuous coverage: If the serving cell t-service expires, stop T310 (if running), and start T311 (i.e. perform cell search and re-establishment without attempting to recover on the current cell for the duration of T310) upon t-ServiceStart of the upcoming cell.

For conditional reconfiguration:

Proposal 2: For eMTC NTN, clarify that the agreed time-based conditional reconfiguration trigger is based on condEventT1 in NR, where the event will be satisfied if conditional handover execution occurs between T1 and T2, where T2 = T1 + a duration (similar to condEventT1 in NR)

Proposal 3: Time and location-based trigger conditions may be configured independently (i.e., without a jointly configured measurement condition) for eMTC NTN.

# References

- RP-223519,  “Revised WID on IoT NTN enhancements”

- R2-2306665	[offline-113] Measurements before RLF	Interdigital

- R2-2306486	On IoT NTN CHO and other mobility enhancements	Samsung

- Report of [Post122][112][IoT NTN Enh] Mobility enhancements, Mediatek