3GPP TSG-RAN WG2 #123	R2-2308586

Toulouse, France, 2023-08-21 - 2023-08-25

Agenda Item:	7.5.2

Source:	Ericsson

Title:	Discussion on XR awareness

Document for:	Discussion, Decision

# Introduction

In [1], a new Work Item for XR aims at specifying the following enhancements related to XR awareness and having an impact on RAN2:

-	Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2);

-	Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2);

-	Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3)

In RAN2#122 Incheon the following highlighted proposals were agreed to be focused on with respect to UL XR awareness information:

UL jitter information

Proposal 1. UE reports to RAN the range of jitter in its UL traffic, defined in the same way as the one for N6 jitter.

Proposal 2. Range of UL jitter is reported per logical channel. Network can configure for which logical channel UE should report jitter information.

Proposal 3. Network can configure whether UE reports UL jitter information periodically or only when the range exceeds a threshold.

Other UL traffic information

Proposal 6. UE can report UL traffic periodicity of a logical channel to RAN. This information is a complement, not a replacement, to the traffic periodicity provided by CN to RAN.

Proposal 7. UE can report its preferred start offset for a CG to RAN.

Proposal 8. UL traffic information can be signaled via the RRC message UE Assistance Information.

Focus on P1-3, P6-8

Proposal 1: the UL Jitter information can be defined associated with the Periodicity in uplink, which can be defined as BAT offset, i.e., variation of burst arrival time for UL traffic resulting from UL jitter which value can be positive or negative.

Proposal 2: the range of UL Jitter information is better to be set as [-4, +4].

Focus on P1-2

Proposal 1 RRC UAI framework is updated for Rel-18 to support signalling UL jitter and associated periodicity information.

Proposal 2 UE report statistical UL jitter range X and associated periodicity Y on a per QoS flow

Proposal 3 RAN2 discuss the value range of UL jitter X and associated periodicity Y

Proposal 4 If RRC UAI is enhanced with UL jitter and associated periodicity information, RAN2 should discuss the

configuration of report triggers, e.g. prohibition timer or change threshold that regulate when UE is allowed to send UL jitter and periodicity information.

Focus on P1-4

Subsequently the following agreements was made:

1. UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter.

2: Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side.

2. UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information.

3 RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

In this contribution we discuss the recent agreements with respect to RAN and UE impacts.

# 2	Discussion

## 2.1 	Discussion on content for UAI for XR

RAN2 has agreed that UE can report jitter information associated with UL XR traffic. With such information available at gNB, the gNB may select a more suitable configuration of features e.g. CG and pre-scheduling. RAN2 also agreed that jitter be defined the similar way as the one for N6 jitter. SA2 has defined N6 jitter in TS 23.501 v18.2.2 with the following definition:

“The DL periodicity associated N6 jitter indicates the positive or negative deviation of the arrival time of first packet of a Data Burst compared to the ideal Data Burst start time which is be determined based on the DL periodicity.”

As of now N6 jitter signalled over the NGAP awaits RAN3 assessment, however RAN2 may still discuss the definition of jitter and provide feedback to RAN3. The RAN1 simulation assumptions in TR 38.838 for jitter in DL assumed a jitter range of [-4,4] ms. We believe this can serve as starting point for a solution of jitter info in the UL. In line with the SA2 definition: “N6 jitter indicates the positive or negative deviation of the arrival time…” one signalling option illustrated in Figure 1 could be to signal the max deviation as an absolute value. For example if the total jitter range is [-X,X] then it is enough to signal X ms.

Figure 1 Illustration of representing jitter information

As for the reported values of UL jitter, RAN2 can discuss the reporting format. We propose that jitter is reported in step of 0.5 ms starting from 0.5 ms eg. (0.5, 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 6, 7, 8, 9, 10) ms

Jitter is represented as a range [-X, X] ms

Jitter range X is reported in RRC UAI with steps of 0.5ms starting from 0.5ms

In addition to jitter information, RAN2 has agreed to also signal a “Reference time” similar to Burst Arrival Time (BAT).  The BAT is currently carried in TSCAI and used in the context of TSN. The BAT is defined as follows in TS 32.501:

“The latest possible time when the first packet of the data burst arrives at either ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow directions)”.

Moreover, TS 38.413 section 9.3.1.133 outlines the signalling of the BAT as the ReferenceTime IE as defined in TS 38.331 (shown below) where the value is truncated to 1us granularity.

Figure 2 ReferenceTime IE as it is defined in TS 38.331

Since the referenceTime IE is in fact a single timestamp and does not contain information of the interval between bursts, i.e. Y in Figure 1, we think that complementary periodicity information is also needed. We note that Periodicity information has been agreed to be sent in TSCAI from the CN. However, considering that UE periodicity may change rapidly due to e.g. network conditions or user setting, the CN TSCAI signalling may be too slow to react. Thus, it would be preferable to get this information faster, directly from the UE through RRC UAI signalling.

Signalling periodicity information with RRC is faster than CN signalling.

Considering that XR traffic flow periodicity may take on multiple different values, the relevance of jitter information may depend on the period. For example, a 4 ms jitter range (opt1) may be considered large or small depending on the overall traffic periodicity. If the period is e.g. 16.667 ms with 4 ms jitter, UE may get treatment A in RAN, but if period is instead 4 ms with 4 ms jitter, RAN may find treatment B more suitable.

The jitter range can be large or small depending on the overall traffic periodicity.

Traffic periodicity information is useful to assess the relevance of the jitter information.

In addition to jitter range and reference time, the periodicity information is also necessary to be signalled by the UE in the UAI.

The reference time signalled in UAI will thus be interpreted as the start time offset of the traffic period. With jitter, periodicity and reference time information, gNB will have knowledge of when traffic is expected to arrive in the UE and may optimizes scheduling resources for UL.

With jitter, periodicity and reference time information gNB will have knowledge of when traffic is expected to arrive in the UE and may optimizes scheduling resources for UL.

In addition to jitter, periodicity and reference time information, it was also agreed in RAN#122 that the information is sent per QFI. Thus, the associated QFI should also be included in the RRC UAI such that awareness information may be reported for each QFI.

The associated QFI is also signalled in the RRC UAI.

In terms of RRC format for signalling we suggest periodicity is reported similar as defined in TS 38.413 (NGAP) shown below.

#### 9.3.1.131	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.132 |  | - |  |
| Burst Arrival Time | O |  | 9.3.1.133 |  | - |  |
| Survival Time | O |  | 9.3.1.221 |  | YES | ignore |

#### 9.3.1.132	Periodicity

This IE indicates the Periodicity of the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | INTEGER (0..640000, …) | Periodicity expressed in units of 1 us. |  |  |

Periodicity information is reported in the UAI with similar format as reported in TSCAI.

In summary the RRC UAI may take the following form in TS 31.331

UEAssistanceInformation-v1700-IEs ::= SEQUENCE {…

…

nonCriticalExtension                  UEAssistanceInformation-v1800-IEs OPTIONAL

}

UEAssistanceInformation-v1800-IEs ::=     SEQUENCE {

ulTrafficInformation                  UlTrafficInformation-r18 OPTIONAL,

nonCriticalExtension                  SEQUENCE {}              OPTIONAL

}

UlTrafficInformation-r18 ::= SEQUENCE (SIZE(1..maxNrofUlTrafficPattern-r18)) OF UlTrafficPatternInfo-r18

UlTrafficPatternInfo-r18 ::=      SEQUENCE {

trafficPattern-QoS-identity-r18  INTEGER{0..FFS}

jitterRange-r18			         ENUMERATED{0.5ms,1ms,1.5ms,2ms,2.5ms,3ms,3.5ms,4ms,4.5ms,5ms, 6ms,7ms,8ms,9ms,10ms}										OPTIONAL,

periodicity-r18			         INTEGER{0..640000}

referenceTime-18 		         ReferenceTime-r18;

} OPTIONAL,

## 2.2	Discussion on network configuration of XR awareness

In the existing RRC specification the OtherConfig IE is used to configure the triggering conditions for the UAI information, e.g. prohibit timers. It would thus be suitable that the OtherConfig is updated to include triggering conditions also for signalling UL jitter, periodicity and reference time information. For example, to avoid excessive signalling in case the periodicity changes often due to rate adaptation, a prohibit timer can be included or a change threshold can be configured indicating how much the UL jitter or periodicity must have changed from the previously reported value before reporting a new value. If the deltaX (change in jitter) or deltaY (change in periodicity) exceeds the change threshold or the prohibit timer has expired, the UE may send an updated UAI report.

RAN2 should discuss the configuration of report triggers, e.g. prohibit timer or change threshold, that regulate when UE is allowed to send UL jitter, periodicity and reference time information.

The triggering conditions for UE sending UAI with XR awareness information are carried in the OtherConfig IE.

In addition to triggering conditions for RRC UAI, flags indicating for which QFI the traffic information is associated with should also be in the otherConfig IE.  Thus, gNB may configure multiple triggering conditions for different QFIs and get traffic information associated to each active QoS flow.

The QFI is also signalled in the OtherConfig IE.

## 2.3	Discussion on TSCAI for XR and RAN2 impacts

In [2] SA2 provides the following LS to RAN2/RAN3

1. Overall Description:

SA2 thanks RAN2 for their LS in (S2-2306323/R2-2304569) on TSCAI for XR and would like to provide the following feedback:

Question: RAN2 has discussed the provision of assistance information from CN to RAN for XR services and is wondering whether TSCAI can be provided for both GBR and non-GBR QoS flows in case of XR.

SA2 Answer: for either GBR or non-GBR QoS flow, TSCAI can be provided to the RAN.

2. Actions:

To RAN2 and RAN3 group:

ACTION: 	SA2 respectfully asks RAN2 and RAN3 to take the above information into consideration.

In NGAP, the TSCAI is provided as part of the TSC Traffic Characteristics IE (TS 38.413 section 9.3.1.130), which is included in the PDU Session Resource Setup Request Transfer (TS 38.413 section 9.3.4.1) and PDU Session Resource Modify Request Transfer (section 9.3.4.3) IEs. There, we can see that the IE is bound only to GBR QoS flows and is ignored otherwise by the NG-RAN.

| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

NG-RAN node according to the current RAN3 NGAP specification ignores the TSCAI IE when the QoS flow is non-GBR. The TSCAI related function will not be performed but the QoS flow may be setup successfully.

In Release 17, the TSCAI in NGAP is valid only for GBR QoS flow and is ignored otherwise.

However, we note since this is clearly a RAN3 topic, we propose to leave the discussion for RAN3.

The (non-)GBR TSCAI discussion is left for RAN3 to discuss and act on.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Signalling periodicity information with RRC is faster than CN signalling.

Observation 2	The jitter range can be large or small depending on the overall traffic periodicity.

Observation 3	Traffic periodicity information is useful to assess the relevance of the jitter information.

Observation 4	With jitter, periodicity and reference time information gNB will have knowledge of when traffic is expected to arrive in the UE and may optimizes scheduling resources for UL.

Observation 5	In Release 17, the TSCAI in NGAP is valid only for GBR QoS flow and is ignored otherwise.

Based on the discussion in the previous sections we propose the following:

Jitter is represented as a range [-X, X] ms

Proposal 1

Proposal 2	Jitter range X is reported in RRC UAI with steps of 0.5ms starting from 0.5ms

Proposal 3	In addition to jitter range and reference time, the periodicity information is also necessary to be signalled by the UE in the UAI.

Proposal 4	The associated QFI is also signalled in the RRC UAI.

Proposal 5	Periodicity information is reported in the UAI with similar format as reported in TSCAI.

Proposal 6	RAN2 should discuss the configuration of report triggers, e.g. prohibit timer or change threshold, that regulate when UE is allowed to send UL jitter, periodicity and reference time information.

Proposal 7	The triggering conditions for UE sending UAI with XR awareness information are carried in the OtherConfig IE.

Proposal 8	The associated QFI is also signalled in the OtherConfig IE.

Proposal 9	The use of implicit EoDB by existing Padding BSR is left to gNB implementation

Proposal 10	The (non-)GBR TSCAI discussion is left for RAN3 to discuss and act on.

# References

RP-223502, “New WID on XR Enhancements for NR”, 3GPP TSG RAN Meeting #98-e, 12-16 Dec. 2022.

S2-2306323/R2-2304569 LS reply on TSCAI for XR, SA WG2 Meeting #157, Rel-18, May, 2023