3GPP TSG-RAN WG2 Meeting #123                                                             R2-2308615

Toulouse, France, August 21 – 25, 2023

Agenda item: 		7.4.2.2

Source: 		Qualcomm Incorporated

Title: 	RRC aspects of LTM

Document for:	Discussion

# 1	Introduction

In RAN2#122, the following agreements have been achieved [1]:

For L1 measurements for LTM: - The RS configuration is provided to the UE per LTM candidate cell. - RAN2 assumes that Each candidate DU needs to know the RS configuration of each candidate DUs in order to provide the LTM candidate configuration. - RAN2 assumes that The CU transmits to each C-DU the RS configuration of S-DU (if this is an LTM candidate cell) and/or other C-DUs, to generate the corresponding L1 configuration for LTM. - RAN2 assumes C-DU generates the RS configuration and send to the CU. The CU transmits to the Source DU the RS configuration per LTM candidate cell and the associated LTM candidate (when the CU receives LTM candidate configuration(s) from the C-DU). It is up to RAN3 whether the RS configuration is sent before (or at the same time of) the C-DU creates the LTM candidate configuration (and whether is semi-statis or UE associated). - The RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration. - CSI reports for LTM candidates (neighbour cell reports for the purpose of LTM cell switch) are configured by the serving cell in an IE that is like CSI-ReportConfig for LTM within the ServingCellConfig since this is the cell in which the report is to be transmitted. - RAN2 assumes the following about CSI measurement reporting for LTM (final decision up to RAN1): - UE reports all measured LTM candidate cells in a single report; or - UE reports one or a subset of measured LTM candidate cell(s) in a report. For non-TA parts, we do MAC reset, which overrides earlier agreements on partial MAC reset. As earlier agreed RLC-AM can continue at LTM cell switch (intended for intra-DU).

In RAN2#121bis-e, the following agreements have been achieved [2]:

- The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed. - The location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. The RS configuration, per RAN1 agreement, can include PCI or logical ID, SMTC location, frequency location, and SCS. - RAN2 assumes that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells (same location as RS configuration). - RAN2 assumes that For L1 measurements of LTM candidate cells, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s). - FFS if the LTM specific L1 measurements of an LTM candidate SCell is independent of its activation status.

In RAN2#121, the following agreement has been achieved [3]:

To determine if to reset L2 or not is based on RRC configuration (e.g. set of cells. FFS if separate for RLC, MAC, PDCP).

This contribution discusses various RRC configuration aspects of LTM including configuring a serving cell as candidate for subsequent LTM, L2 reset and L1 measurements/reporting. This contribution also discusses early decoding and delta configuration.

# 2	Configuring the current serving cell group as an LTM candidate cell group for subsequent LTM

In subsequent LTM, the current serving cell group becomes an LTM candidate cell group after the first LTM execution. Any LTM candidate cell group requires LTM-specific configuration, e.g., an LTM candidate ID. This implies LTM-specifc configuration is also needed for the UE’s current serving cell group. Two options are possible:

- Option 1: During LTM preparation, the UE receives an RRCReconfiguration message containing an LTM config where:

  - The LTM config comprises a list of LTM candidate configs (containing RRCReconfiguration) with LTM candidate IDs, where one member of this list is that of the UE’s current serving cell group.

  - The UE immediately applies the RRCReconfiguration of the LTM member candidate corresponding to the UE’s current serving cell group, e.g., which provides LTM-related reporting configuration, LTM-related TCI state configuration, etc.

- Option 2: During LTM preparation, the UE receives an RRCReconfiguration message containing an update of the UE’s current serving cell group + an LTM config, where:

  - The LTM config comprises a list of LTM candidate configs (containing RRCReconfiguration) with LTM candidate IDs, none of which corresponds to the UE’s current serving cell group.

  - The UE immediately applies the update of the UE’s current serving cell group, which assigns an LTM candidate ID to the UE’s current serving cell group, and, e.g., provides LTM-related reporting configuration, LTM-related TCI state configuration, etc.

Proposal 1: RAN2 to discuss the following options to configure the UE’s current serving cell group as an LTM candidate for subsequent LTM:

-	Option 1: During LTM preparation, the UE receives a list of LTM candidate configs + IDs, one of which corresponds to the UE’s current serving cell group, where the UE immediately applies the RRCReconfiguration of that list member to be able to perform the necessary LTM-related procedures before the LTM cell switch.

-	Option 2: During LTM preparation, the UE receives a list of LTM candidate configs + IDs, none of which corresponds to the UE’s current serving cell group. The UE additionally receives a separate configuration update for its current serving cell group, which assigns a candidate ID to the current serving cell group and provides LTM-related configuration that the UE needs to perform the necessary LTM-related procedures before the LTM cell switch.

# 3	RRC configuration of L2 reset (RLC, PDCP)

RAN2#122 agreed that the UE always performs full MAC reset at LTM execution. The discussion below refers to RLC reestablishment and PDCP recovery for RLC AM.

RAN2#121 agreed that the UE determines whether to do L2 reset at LTM execution based on prior RRC configuration. This can be done as follows:

- Step 1: RRC provides partitioning of the candidate LTM configurations into sets during LTM preparation.

- Step 2: The LTM cell switch command indicates a candidate LTM configuration index.

- Step 3: The UE checks if the configuration index of the serving cell group and the configuration index of the candidate cell group belong to same or different sets as per the RRC configuration in Step 1:

  - If same set: the UE does NOT perform L2 reset.

  - If different sets: the UE performs L2 reset.

Proposal 2a: To determine whether to perform RLC reestablishment and PDCP recovery, RRC provides partitioning of the candidate LTM configurations into sets during LTM preparation. At LTM execution, the UE performs these steps if the source configuration and the target configuration fall into different sets. The UE skips these steps otherwise.

Having separate configurations per reset type is more flexible for the network and should be supported.

Proposal 2b: Separate RRC partitioning is configured to determine whether to perform RLC reestablishment or PDCP recovery.

# 4	Configuration of L1 measurement/reporting

## 4.1	Location of reporting configuration

For measurements, RAN2#122 agreed that the RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration.

For reporting, RAN2#121bis-e agreed that the reporting configuration is placed inside the ServingCellConfig of current serving cell(s). RAN2#122 further agreed that each C-DU receives the RS configuration of S-DU and/or other C-DUs to generate the corresponding “L1 configuration” for LTM. RAN2 should confirm that this L1 configuration includes the report configuration a UE would use after it switches to the corresponding LTM candidate cell, and that this configuration is carried as part of ServingCellConfig of the candidate cell (which after cell switch becomes the serving cell).

Proposal 3: RAN2 to confirm that the UE receives separate reporting configurations within the ServingCellConfig of each of the current serving cell and the candidate cells. The UE applies the reporting configuration of the current serving cell before the cell switch and that of the target cell after the cell switch.

## 4.2	Measurement/reporting of a deactivated serving SCell

In legacy behavior, UE reports L1 measurements of a serving SCell in activated state and stops these reports in the deactivated state. In case a cell is both a serving SCell and an LTM candidate cell, the UE would still need to submit L1 measurement reports of the cell in deactivated state for the sake of LTM or else LTM to the cell is never triggered. This implies that measurement of a cell that is both a serving SCell and an LTM candidate cell is independent of the activation status of that cell.

Proposal 4: LTM specific L1 measurements of an LTM candidate cell that is also a current serving SCell is independent of the activation status of that SCell.

# 5	Discussion on decoding and validity-check of LTM candidate configurations

RAN2#121bis-e agreed that the UE “may” perform early decoding and early validity check of the LTM candidate configurations. It is observed that this will be wasted effort if LTM execution is never triggered towards an LTM candidate for which the associated configuration had this early processing.

Observation 1: Decoding and validation of a configuration of an LTM candidate cell is wasteful if LTM is never triggered towards that cell.

This implies whether to do early decoding and validity check of an LTM candidate configuration should be the UE’s choice.

Proposal 5: The UE is not expected to receive a request from the network to perform early decoding and early validity check of an LTM candidate configuration.

Assuming the UE decodes an LTM candidate configuration and compliance check fails, it is not useful if the UE simply reports an error flag. On the other hand, providing a detailed report on which IEs in the LTM candidate configuration cause the compliance check failure may be an overwhelming task. Furthermore, the spec effort to determine how the UE identifies and reports compliance failure to portions of an RRC Reconfiguration message will be major.

Observation 2a: Simple error flagging of an LTM candidate configuration is not useful.

Observation 2b: The UE may be overchallenged to provide a detailed report of erroneous network configuration.

Observation 2c: The spec effort to determine how the UE identifies and reports IEs of an RRC Reconfiguration message for which compliance check fails is major.

Therefore, the simplest behavior of the UE is to re-establish RRC following legacy behavior.

Observation 2d: The simplest behavior of the UE is to re-establish its RRC connection at compliance check failure for an LTM candidate cell.

On the other hand, performing connection re-establishment for compliance check of an LTM candidate configuration is too costly if LTM execution is never triggered to the corresponding LTM candidate cell.

Observation 2e: Reestablishing connection due to compliance check failure for a configuration of an LTM candidate cell is too costly if LTM execution is never triggered towards that cell.

Therefore, connection re-establishment due to compliance check failure should only be performed if LTM execution is triggered towards the corresponding LTM candidate cell.

Proposal 6: The UE performs connection reestablishment due to compliance check failure for a candidate LTM configuration after LTM execution is triggered towards the corresponding LTM candidate cell.

# 6	Discussion on delta configuration

Figure 1: Efficient delta signaling in PCell/SCell-swap LTM scenario

RAN2#119bis-e agreed that a candidate LTM PCell may be a current serving SCell, and a candidate LTM SCell may be a current serving PCell. This implies that a PCell of one candidate LTM cell group be the same as an SCell of another candidate LTM cell group.

Observation 3a: A PCell of one candidate LTM cell group and an SCell of another candidate LTM cell group may be the same.

Figure 1 illustrates the PCell/SCell-swap LTM scenario where the UE switches b/w two candidate cell groups:

- Candidate CG 1 that comprises Cell1 as a PCell and Cell2 as an SCell

- Candidate CG 2 that comprises Cell2 as a PCell and Cell1 as an SCell

In this scenario, using a common reference cell configuration for delta signaling of the PCell config of candidate CG1 and the SCell config of candidate CG2 is the most efficient. Similarly, using a common reference cell configuration for delta signaling of the SCell config of candidate CG1 and the PCell config of candidate CG2 is the most efficient.

Observation 3b: If the same cell is a PCell in one candidate LTM cell group and an SCell in another candidate LTM cell group, using a common reference cell configuration to configure delta signaling for this cell is the most efficient.

Consequently, configuring a PCell with respect to a reference SCell configuration and configuring an SCell with respect to a reference SpCell configuration should be supported.

Proposal 7a: An SpCellConfig IE of the CellGroupConfig IE of the candidate RRC reconfiguration IE may use delta signaling wrt to either a reference SpCellConfig IE or a reference SCellConfig IE of the reference CellGroupConfig IE of the reference configuration.

Proposal 7b: An SCellConfig IE of the CellGroupConfig IE of the candidate RRC reconfiguration IE may use delta signaling wrt to either a reference SpCellConfig IE or a reference SCellConfig IE of the reference CellGroupConfig IE of the reference configuration.

# Conclusion

This contribution discussed various RRC configuration aspects of LTM including configuring a serving cell as candidate for subsequent LTM, L2 reset and L1 measurements/reporting. This contribution also discussed early decoding and delta configuration. The following observations and proposals have been made:

Current CG is candidate for subsequent LTM:

Proposal 1: RAN2 to discuss the following options to configure the UE’s current serving cell group as an LTM candidate for subsequent LTM:

-	Option 1: During LTM preparation, the UE receives a list of LTM candidate configs + IDs, one of which corresponds to the UE’s current serving cell group, where the UE immediately applies the RRCReconfiguration of that list member to be able to perform the necessary LTM-related procedures before the LTM cell switch.

-	Option 2: During LTM preparation, the UE receives a list of LTM candidate configs + IDs, none of which corresponds to the UE’s current serving cell group. The UE additionally receives a separate configuration update for its current serving cell group, which assigns a candidate ID to the current serving cell group and provides LTM-related configuration that the UE needs to perform the necessary LTM-related procedures before the LTM cell switch.

L2 reset:

Proposal 2a: To determine whether to perform RLC reestablishment and PDCP recovery, RRC provides partitioning of the candidate LTM configurations into sets during LTM preparation. At LTM execution, the UE performs these steps if the source configuration and the target configuration fall into different sets. The UE skips these steps otherwise.

Proposal 2b: Separate RRC partitioning is configured to determine whether to perform RLC reestablishment or PDCP recovery.

L1 measurement/reporting:

Proposal 3: RAN2 to confirm that the UE receives separate reporting configurations within the ServingCellConfig of each of the current serving cell and the candidate cells. The UE applies the reporting configuration of the current serving cell before the cell switch and that of the target cell after the cell switch.

Proposal 4: LTM specific L1 measurements of an LTM candidate cell that is also a current serving SCell is independent of the activation status of that SCell.

Early decoding:

Observation 1: Decoding and validation of a configuration of an LTM candidate cell is wasteful if LTM is never triggered towards that cell.

Proposal 5: The UE is not expected to receive a request from the network to perform early decoding and early validity check of an LTM candidate configuration.

Observation 2a: Simple error flagging of an LTM candidate configuration is not useful.

Observation 2b: The UE may be overchallenged to provide a detailed report of erroneous network configuration.

Observation 2c: The spec effort to determine how the UE identifies and reports IEs of an RRC Reconfiguration message for which compliance check fails is major.

Observation 2d: The simplest behavior of the UE is to re-establish its RRC connection at compliance check failure for an LTM candidate cell.

Observation 2e: Reestablishing connection due to compliance check failure for a configuration of an LTM candidate cell is too costly if LTM execution is never triggered towards that cell.

Proposal 6: The UE performs connection reestablishment due to compliance check failure for a candidate LTM configuration after LTM execution is triggered towards the corresponding LTM candidate cell.

Delta configuration:

Observation 3a: A PCell of one candidate LTM cell group and an SCell of another candidate LTM cell group may be the same.

Observation 3b: If the same cell is a PCell in one candidate LTM cell group and an SCell in another candidate LTM cell group, using a common reference cell configuration to configure delta signaling for this cell is the most efficient.

Proposal 7a: An SpCellConfig IE of the CellGroupConfig IE of the candidate RRC reconfiguration IE may use delta signaling wrt to either a reference SpCellConfig IE or a reference SCellConfig IE of the reference CellGroupConfig IE of the reference configuration.

Proposal 7b: An SCellConfig IE of the CellGroupConfig IE of the candidate RRC reconfiguration IE may use delta signaling wrt to either a reference SpCellConfig IE or a reference SCellConfig IE of the reference CellGroupConfig IE of the reference configuration.

# References

- Chairman Notes; TSG RAN WG2 Meeting #122

- Chairman Notes; TSG RAN WG2 Meeting #121bis-e

- Chairman Notes; TSG RAN WG2 Meeting #121