3GPP TSG-RAN WG2 Meeting #123        	          R2-2308627

Toulouse, France, 21 – 25 August, 2023

Agenda Item:	7.13.7

Source: 	Huawei, HiSilicon

Title: 	Discussion on SONMDT enhancements for NPN

Document for:	Discussion and decision

Introduction

In the last RAN2 #122 meeting [1], the following agreements for SON/MDT in NPN are achieved:

Agreements:

1	Include CAG ID(s) in the logged MDT area configuration.

Meanwhile, there are still some remaining issues, and in this paper, we shall provide our further concerns for these.

Discussion

Remaining issues for SON in NPN

One of the remaining issues is whether to introduce PNI-NPN ID to RLF/HOF report. In our concern, a PNI-NPN is a network deployed for non-public use which relies on network functions provided by a PLMN. In the view of network, PNI-NPN is a part of a PN network, while specified with CAG. For PNI-NPN, the mobility of the UE is based on the mobility restrictions in the UE context as legacy. The UE moves within cells which the UE is subscribed.

When HOF/RLF occurs during the mobility procedure, the UE stores the related cell information for RLF report forwarding. For non-RAN sharing case of a PNI-NPN network, when the cell information is provided, the network can deduce the related CAG ID. For RAN sharing case of PNI-NPN networks, if there are more than one CAGs within the same PLMN ID, the network will not be aware of the CAG ID to which the UE is connected. When performing mobility decision, the network will not distinguish the CAG to which the UE is connected. In this case, the RLF/HOF reports from UEs connecting to different CAGs share the same reference meaning for gNB mobility strategy optimization. Consequently, it makes no sense for the UE to report the CAG ID in the RLF report.

Observation 1: The network can deduce the PNI-NPN ID based on the cell information for non-RAN sharing case and for RAN sharing case, the network will not differ the CAG ID to which the UE is connected.

Proposal 1: No need to introduce PNI-NPN ID (e.g., CAG ID) in to RLF report.

Remaining issues for SON in SNPN

One remaining issue is to discuss how to include SNPN related ID in the RLF/HOF report and further which format (NID only or PLMN and NID) should be applied. Based on the agreements from RAN2 119b and 120 meeting, NID is introduced in RLF report for both HOF and RLF cases. As for the detail introduction method, we first refer to the existing ASN.1 for RLF report as the follows.

RLF-Report-r16 ::=                   CHOICE {

nr-RLF-Report-r16                    SEQUENCE {

[……]                                                                   OPTIONAL,

previousPCellId-r16                  CHOICE {

nrPreviousCell-r16                   CGI-Info-Logging-r16,

eutraPreviousCell-r16                CGI-InfoEUTRALogging

}                                                                    OPTIONAL,

failedPCellId-r16                    CHOICE {

nrFailedPCellId-r16                  CHOICE {

cellGlobalId-r16                     CGI-Info-Logging-r16,

pci-arfcn-r16                        PCI-ARFCN-NR-r16

},

eutraFailedPCellId-r16           CHOICE {

cellGlobalId-r16                 CGI-InfoEUTRALogging,

pci-arfcn-r16                    PCI-ARFCN-EUTRA-r16

}

},

[……]

...,

[……]

}

#### –	CGI-Info-Logging

The IE CGI-Info-Logging indicates the NR Cell Global Identifier (NCGI) for logging purposes (e.g. RLF report), the globally unique identity, and the TAC information of a cell in NR.

CGI-Info-Logging information element

-- ASN1START

-- TAG-CGI-INFO-LOGGING-START

CGI-Info-Logging-r16 ::=     SEQUENCE {

plmn-Identity-r16                    PLMN-Identity,

cellIdentity-r16                     CellIdentity,

trackingAreaCode-r16                 TrackingAreaCode               OPTIONAL

}

As listed below, for both IE failedPCellId and previousPCellId, they refer to the structure IE CGI-Info-Logging-r16, which doesn’t support any late non-critical extension at all. Therefore, to introduce the NID into RLF report, it cannot be introduced into the existing IE CGI-Info-Logging-r16. Instead, it is desirable to introduce a separate NID information related IE into the RLF report in the same paralleling with the cell information parameter.

On the other hand, according the TS 24.501, the equivalent SNPN is applied from Rel-18, where a UE is able to register and be served by a list of SNPNs. Therefore, to guarantee future support for equivalent SNPN, a reserved extension for PLMN should be kept in the aforesaid newly introduced NID information related ID.

Proposal 2: Introduce NID information related IE with reserved extension for both IE failedPCellId and previousPCellId.

Remaining issues for MDT in SNPN

For MDT in SNPN, one remaining issue is whether to include SNPN ID in the logged MDT area configuration. If the Rel-18 introduced equivalent SNPN is not considered, the UE operating in SNPN access mode can only moves within cells belonging to the current registered SNPN. When the UE is not operated in SNPN access mode or changed to another SNPN, the UE will be de-registered from the previously registered SNPN. In this case, the UE will delete any stored MDT configuration and related measurement results for previous registered SNPN. Correspondingly, if a UE is configured with logged MDT and the area configuration is included in the current registered SNPN network, this implies that the received logged MDT area configuration can only applied for the current registered SNPN scope. Therefore, there is no need to add SNPN ID (e.g., NID) in the logged MDT area configuration.

However, RAN3 is currently discussing how to support MDT in equivalent SNPNs. According to the latest RAN3 agreements, the MDT area scope has already introduced new choices for SNPN cell based MDT, SNPN TAI based MDT and SNPN based MDT, where among all the choices, NID lists are included. Correspondingly, we think RAN2 should also consider to include SNPN ID in the logged MDT area configuration.

- Add separate SNPN Cell Based MDT and SNPN TAI Based MDT area scope

Proposal 3: RAN2 to follow RAN3 discussion and include SNPN ID in the logged MDT area configuration.

Conclusion

In this contribution, we give our further opinions for remaining issues for SON/MDT in NPN based on RAN 2 aspects.

Observation 1: The network can deduce the PNI-NPN ID based on the cell information for non-RAN sharing case and for RAN sharing case, the network will not differ the CAG ID to which the UE is connected.

Proposal 1: No need to introduce PNI-NPN ID (e.g., CAG ID) in to RLF report.

Proposal 2: Introduce NID information related IE with reserved extension for both IE failedPCellId and previousPCellId.

Proposal 3: RAN2 to follow RAN3 discussion and include SNPN ID in the logged MDT area configuration.

References

- RAN2-122-SONMDT-HU_2023-05-26-1234

- RAN3_120_agenda_20230530_EOM1