3GPP TSG-RAN WG2 Meeting #123                                   R2-2308694

Toulouse, France, 21st August – 25th August, 2023

Agenda item:		7.2.4 (LPHAP)

Source:				Samsung

Title:					Discussion on SRS configuration in RRC_INACTIVE

Document for:		Discussion & Decision

1	Introduction

In the latest version of WID for R18 POS, the objectives for SRS configuration in RRC_INACTIVE are described as below [1].

- For UL and DL+UL positioning for UEs in RRC_INACTIVE state, specify SRS configuration enhancements based on SRS positioning validity area to avoid frequent RRC connection for SRS (re)configuration [RAN2, RAN1, RAN3]. - SRS for positioning configurations in multiple cells [RAN2, RAN1]. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations [RAN2, RAN3]. - SRS for positioning activation/request procedure(s) [RAN2, RAN1].

In RAN2#120, the following agreements were made regarding SRS configuration in RRC_INACTIVE:

Agreements: Proposal1 (modified): SRS positioning validity area for UL positioning in RRC_INACTIVE can avoid reconfiguration of SRS configuration upon cell reselection and is recommended for normative work from R2’s perspective if feasible from R1’s perspective  The solution should not require the gNB to monitor multiple SRS configuration simultaneously for a UE Proposal2 (modified): SRS configuration request can be discussed during normative work from RAN2 perspective.  Scenarios requiring SRS configuration request include:  Scenario1: During the UL positioning procedure, when the SRS configuration turns invalid, e.g., when the UE moves out of the SRS positioning validity area.  Scenario2: At the initiation of UL positioning procedure when an event is detected.  Detailed solution for the SRS update, e.g., with RRC message, UL MAC, or NG-AP message can be discussed in the WI phase Proposal3 (modified): Pre-configuration of multiple SRS configurations (e.g., for multiple SRS positioning validity areas) is feasible from RAN2 perspective and can be discussed in normative work.  The pre-configuration of multiple SRS configurations can be delivered to the UE either by dedicated signalling or SI broadcast

In RAN2#121, the following agreements were made regarding SRS configuration in RRC_INACTIVE:

Agreements:  When configured with SRS configuration along with SRS validity area, if the UE reselects to another cell within the SRS validity area during SRS transmission, the UE continues the SRS transmission, subject to validation for SRS transmission. Wait for RAN1 progress for the validation of SRS transmission with issues such as interference, timing advance and spatial relation information, etc.  RAN2 assume when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request. LS to RAN3 to confirm this.

In RAN2#121bis-e, the following agreements were made regarding SRS configuration in RRC_INACTIVE:

Agreement:  The SRS validity area configuration contains a list of cells in which it is valid. FFS validity timer or if we would depend only on explicit release by the network.  RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE.  SRS configuration request can be indicated via Msg3/MsgA transmission. FFS if the request is in the RRC message or an accompanying MAC CE.

In RAN2#122, the following agreements were made regarding SRS configuration in RRC_INACTIVE:

Agreements:  Define an SRS for positioning validity-area specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. - The UE starts/restarts the area-specific TA timer when it receives the TA command. - The UE stops the SRS transmission when the area-specific TA timer expires. - The UE stops the area-specific TA timer when it reselects to a cell out of the SRS validity area. - Other stop/restart conditions can be discussed.  RAN2 consider that the LMF should determine the area-specific SRS configuration. Details are up to RAN3. LS to RAN3 to indicate this conclusion, including RAN1 to prompt them for parameters. To be included in the LS from [AT122][415].  RAN2 will introduce an activation indication and/or request for preconfigured SRS using at least Msg3/MsgA; FFS if Msg1 would be supported also. FFS RRC signalling or MAC CE for the Msg3/MsgA case, as for the configuration request. This agreement does not imply that the UE will be allowed to transmit autonomously.

This contribution provides our views on some remaining issues on SRS configuration in RRC_INACTIVE.

2	Discussion

2.1	SRS configuration in multiple cells (i.e., validity area)

According to the objectives in WID, RAN2 needs to discuss the potential enhancement for SRS configuration in multiple cells. In our view, the common SRS configuration can be used within multiple cells. For that, the SRS configuration in RRC Release message can be associated with the validity area (i.e., the list of cells where the SRS configuration is valid).

Regarding this, at RAN2#121, RAN2 agreed the following:

- When configured with SRS configuration along with SRS validity area, if the UE reselects to another cell within the SRS validity area during SRS transmission, the UE continues the SRS transmission, subject to validation for SRS transmission.

- Wait for RAN1 progress for the validation of SRS transmission with issues such as interference, timing advance and spatial relation information, etc.

Based on the agreement above, it is understood that the UE uses a common SRS configuration, such as SRS-PosConfig-r17 including SRS-PosResource(Set), for all cells within a specific validity area. However, certain parameters in the SRS configuration, such as spatial relation information and pathloss reference, may need to be different in different cells.

Typically, the LMF determines a set of receiving TRPs to be used for positioning a target UE based on the serving gNB's location. The spatial relation information/pathloss reference of the SRS resource (set) is then configured as the DL RS from the selected receiving TRPs.

For SRS configuration in validity area, if the receiving TRPs can be changed according to UE’s serving cell within the validity area, the UE needs to have different spatial relation information/pathloss reference for different cells. However, in this case, the LMF needs to request multiple sets of TRPs (here, each TRP set can be the set of receiving TRPs for each cell within validity area) to measure the SRS from the target UE even though the UE can actually transmit the SRS toward only one set of TRPs. To avoid this kinds of inefficiency, we can consider additional signalling procedure for the UE to report its cell reselection within validity area, but this direction is not preferred since this will increase UE’s power consumption.

Alternatively, if a common set of receiving TRPs is assumed within the validity area, there is no need to configure different spatial relation information/pathloss reference for different cells within the validity area. The UE can continue to transmit the SRS using the common SRS configuration without additional signalling overhead to report its cell reselection to the network.

Regarding this issue, RAN1 has agreed the following until now.

Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state: - When the spatial relation information is absent in the configuration, the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources; - When the spatial relation information is provided in the configuration, it is applicable across the cells within the SRS positioning validity area. Further study the configuration details. - FFS: spatial relation information validity criteria, and whether/how to determine UE fallback behavior if validity criteria for spatial relation of the configured RS is not met. Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, when the spatial relation information is provided in the configuration, support: - Alt. 1-1: Reuse the configuration of spatial relation information in Rel-17. - When the UE determines that the configured RS for the spatial relation information cannot be accurately measured, the UE suspends the transmission of the SRS for positioning resource. Agreement For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is absent in the configuration, the UE determines the pathloss RS using a RS resource obtained from the SS/PBCH block of the camping cell that the UE uses to obtain MIB as the pathloss RS. Agreement For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is provided in the configuration, support: - Alt. 2-1 (modified): Reuse the configuration of pathloss RS in Rel-17; - FFS: A CD SSB or non-CD SSB can be configured as pathloss RS - If the UE determines that the pathloss RS cannot be accurately measured, pathloss may be calculated based on the RS resources obtained from SS/PBCH block of the new camping cell that the UE uses to obtain MIB.

Based on the agreement above, now RAN1 is considering two general cases below for both spatial relation and pathloss reference.

* Case 1: Spatial relation/pathloss RS is absent in the SRS configuration.

* Case 2: Spatial relation/pathloss RS is provided in the SRS configuration.

In case1, it is understood that the UE can determine spatial domain transmission filter/pathloss RS to use by itself without having additional signalling to report its cell reselection within the validity area.

In case 2, for spatial relation information, RAN1 agreed that the spatial relation information provided in the SRS configuration is applicable across the cells within the SRS positioning validity area (i.e., no need to update spatial relation information upon cell reselection within validity area). Based on this, it is understood that there will be only one set of receiving TRPs to be used for positioning a target UE within each validity area.

In case 2, regarding pathloss RS, RAN1 agreed that the path loss RS configuration in Rel-17 can be reused. It is also agreed that if the pathloss RS cannot be accurately measured, pathloss may be calculated based on the RS resources obtained from SS/PBCH block of the new camping cell.

According to the discussion above, there is no need of additional signalling procedure between UE and network to update pathloss RS upon cell reselection within the validity area.

Based on the observation above, we would like to propose the following.

Proposal 1: RAN2 confirm that for SRS transmission within validity area, the UE can keep transmitting SRS without additional signalling to update spatial relation information/pathloss RS upon cell reselection.

Meanwhile, the use of positioning validity-area specific TA timer is agreed at the last meeting. We understand that the UE starts the timer upon receiving RRCRelease message from NW and stops the SRS transmission when the area-specific TA timer expires. However, we need further discussion on the UE operation upon expiry of the area-specific TA timer. In Rel-17 SRS configuration for UE in RRC_INACTIVE state, the UE releases the SRS configuration when inactivePosSRS-TimeAlignmentTimer expires. On the other hand, in Rel-18 LPHAP, the motivation of introducing the validity area in SRS configuration is to allow the UE to continue SRS transmission in RRC_INACTIVE state while consuming minimum energy. Considering the motivation, it seems reasonable to allow the UE to keep the SRS configuration even after expiry of the area-specific TA timer and restart the SRS transmission after having a new TA value. By doing that, the UE can avoid additional energy consumption to receive a new SRS configuration.

Proposal 2: The UE does not release the SRS configuration when the area-specific TA timer expires.

Another issue on the area-specific TA timer is whether Rel-18 area-specific TA timer should be newly defined or the Rel-17 cell-specific TA can be reused. In our view, the Rel-17 TA configuration (i.e., inactivePosSRS-TimeAlignmentTimer-r17 in SRS-PosRRC-InactiveConfig-r17 IE) can be reused as the area-specific TA timer. If the SRS configuration in RRCRelease is associated with a validity area (i.e. a list of cells), the UE can regard the TA configuration as Rel-18 area-specific TA timer configuration. Otherwise, it can be regarded as Rel-17 cell-specific TA timer configuration.

Proposal 3: The existing inactivePosSRS-TimeAlignmentTimer-r17 can be reused as the area-specific TA timer in Rel-18.

In release 17, the UE uses the TA value associated with the PTAG for SRS transmission [2]. Thus, according to the legacy operation, the UE needs to update the TA value for SRS transmission upon any cell re-selection. With the SRS configuration in multiple cells, if the UE keeps using the old TA value even after the cell re-selection, it will cause the severe interference in uplink channels of the new serving cell due to the timing misalignment between OFDM symbols. Regarding this issue, RAN1 agreed the following and send the LS (R1-2306248) to ask RAN4 about the feasibility.

Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support the following to determine a valid TA: - The DL reference timing follows the DL timing of current camping cell. - By default, UE maintains the TA from the last serving cell. - UE can adjust its UL timing according to the change in DL reference timing. - If configured by the network, subject to UE capability, UE autonomously adjusts the TA, when cell-reselection happens. - Send LS to RAN4 asking about feasibility and necessary conditions (e.g. whether the above behaviour applies when the DL reference timing difference between the last camping cell and current camping cell exceeds a threshold and how UE adjusts it, or additional RRM procedure to obtain the timing difference).

Since RAN1 is still under discussion and waiting for the reply LS from RAN4, RAN2 can postpone the discussion on TA adjustment for SRS Tx within validity area.

Proposal 4: RAN2 to wait for RAN1 progress on TA update for SRS transmission by UE in RRC_INACTIVE within positioning validity area.

2.2 SRS configuration request

Regarding SRS configuration request, RAN2 has agreed the following until the last meeting.

- (At RAN2#121) RAN2 assume when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

- (At RAN2#121bis-e) SRS configuration request can be indicated via Msg3/MsgA transmission.  FFS if the request is in the RRC message or an accompanying MAC CE.

Based on the agreement above, now we can consider RRC message or MAC CE for the request message. In our view, we can just reuse RRCResumeRequest message for the request by introducing a new value (e.g., pos-SRS-Reconfig) in ResumeCause field as below.

ResumeCause ::=             ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, mo-VoiceCall, mo-VideoCall, mo-SMS, rna-Update, mps-PriorityAccess,

mcs-PriorityAccess, pos-SRS-Reconfig, spare2, spare3, spare4, spare5 }

In our view, when the UE requests the SRS configuration to the new serving gNB via RRCResumeRequest message, the gNB can request the LMF to provide a valid Requested SRS Transmission Characteristics for the target UE. Even though the current serving gNB can retrieve the Requested SRS Transmission Characteristics from the last serving gNB via the existing UE CONTEXT RETREIEVE procedure, the information from the last serving gNB can not be valid anymore especially for the case that the UE request SRS configuration upon cell reselection out of the validity area.

For MAC CE, we can not see the motivation of using MAC CE for SRS configuration request since there seems no additional information that the UE can provide to help the gNB configure SRS. However, we are open to discuss using MAC CE for activation of pre-configured SRS.

Based on the observation above, we would like to propose the following.

Proposal 5: RAN2 to confirm that RRCResumeRequest message can be used for SRS configuration request with a new resume cause value.

Meanwhile, in release 17, the UE is capable of transmitting SRS for positioning while in RRC_INACTIVE state, based on SRS configuration in RRCRelease (i.e., SRS-PosRRC-Inactive-r17). However, upon any cell reselection, the UE stops transmitting SRS and releases the SRS configuration since there is no way for the UE to request a new SRS configuration. Consequently, TRPs fail to measure the SRS from the UE and the LMF is notified of the failure. At this point, the LMF may initiate the procedure for SRS configuration again, and the gNB will page the UE to provide a new SRS configuration in RRC_CONNECTED state, which increases UE’s power consumption in the end.

In our view, to avoid the inefficient procedure described above, the SRS configuration request via RRC message can be also used for the case without validity area. Thus, we would like to propose the following:

Proposal 6: RAN2 to discuss whether the SRS configuration request via RRC message can be also used for the case without validity area.

2.3	Pre-configuration of one or multiple SRS

Regarding pre-configuration of one or multiple SRS, RAN2 agreed the following as a result of SI phase.

- Pre-configuration of multiple SRS configurations (e.g., for multiple SRS positioning validity areas) is feasible from RAN2 perspective and can be discussed in normative work. 
	The pre-configuration of multiple SRS configurations can be delivered to the UE either by dedicated signalling or SI broadcast

The expected procedure of using the pre-configuration of one or multiple SRS configuration can be briefly summarized as below.

Step 1. The LMF requests the pre-configuration of SRS from the gNB.

Step 2. One or more SRS configurations can be delivered to the target UE via RRC signalling (UE-dedicated or common) in advance.

Step 3. One of pre-configured SRS configurations can be activated via signalling between UE and gNB.

Step 4. The gNB informs the LMF of the activation of the pre-configured SRS for the target UE and LMF requests TRP(s) to measure the SRS.

Of the procedures outlined above, we will focus our discussion on steps 2 and 3, which fall under the scope of RAN2. The following issues can be discussed for each step 2/3.

In the step 2, the gNB can pre-configure one or multiple SRS via dedicated signalling or posSIB as in the figure 1 below.

Figure 1. Pre-configuration of SRS

Option 1, which involves UE-dedicated signaling, allows the gNB to deliver one or more UE-specific SRS configurations to a target UE. RRCReconfiguration or RRCRelease messages can be used for this purpose. Each SRS configuration can be associated with a cell/area, and the UE can use any of the pre-configured SRS configurations that is valid in the current cell/area. The UE can continue to transmit SRS without reconfiguration until there is no valid pre-configured SRS in the current cell/area.

Option 2, on the other hand, involves delivering multiple common SRS configurations to multiple UEs via (pos)SIB. When a particular UE needs to transmit SRS signals for positioning, one of the SRS configurations in (pos)SIB can be activated for that UE. Once the UE begins using the SRS configuration, it should be deleted from (pos)SIB. To achieve this, the gNB will trigger a (pos)SIB update procedure, which wakes up multiple UEs to receive the updated SIB, thereby consuming more power. Based on the observation above, we believe that there is no advantage of pre-configuring SRS via (pos)SIB in LPHAP scenarios.

Proposal 7: RAN2 to exclude pre-configuration of SRS via (pos)SIB for LPHAP.

From our understanding, pre-configured SRS means SRS configuration that is provided to UE in advance (in RRC_CONNECTED state) and activated later when positioning procedure starts (in RRC_INACTIVE state). Then, we think semi-persistent SRS configured in RRCRelease message can be reused for the pre-configured SRS.

Proposal 8: The existing semi-persistent SRS configured within RRCRelease can be reused for pre-configured SRS in Rel-18.

In the step3, one of the pre-configured SRS configurations can be activated via signalling between the UE and the gNB. In our view, there are two options to consider in this case.

Option 1. The UE requests the gNB to activate a specific SRS configuration.

Option 2. The UE just requests the activation of SRS and the gNB select a specific SRS configuration to activate.

Option 1 allows the UE to choose among multiple pre-configured SRS configurations for positioning. Once the UE has made its selection, it requests the gNB to activate the chosen SRS configuration, which is confirmed by the gNB. Option 2 allows the UE to simply request activation of SRS for positioning. The gNB then selects a specific SRS configuration to activate. Actually, this issue is related to whether there can be multiple pre-configured SRS configurations within one validity area. In our view, there is no clear motivation of having multiple pre-configured SRS configurations within one validity area. If we assume that the pre-configured SRS configuration is associated with a certain validity area, it is not desirable to reserve multiple SRS ResourceSets in all the cells within the validity area. Thus, we would like to propose the following.

Proposal 9: RAN2 to discuss whether there can be multiple pre-configured SRS configurations within a validity area.

3	Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: RAN2 confirm that for SRS transmission within validity area, the UE can keep transmitting SRS without additional signalling to update spatial relation information/pathloss RS upon cell reselection.

Proposal 2: The UE does not release the SRS configuration when the area-specific TA timer expires.

Proposal 3: The existing inactivePosSRS-TimeAlignmentTimer-r17 can be reused as the area-specific TA timer in Rel-18.

Proposal 4: RAN2 to wait for RAN1 progress on TA update for SRS transmission by UE in RRC_INACTIVE within positioning validity area.

Proposal 5: RAN2 to confirm that RRCResumeRequest message can be used for SRS configuration request with a new resume cause value.

Proposal 6: RAN2 to discuss whether the SRS configuration request via RRC message can be also used for the case without validity area.

Proposal 7: RAN2 to exclude pre-configuration of SRS via (pos)SIB for LPHAP.

Proposal 8: The existing semi-persistent SRS configured within RRCRelease can be reused for pre-configured SRS in Rel-18.

Proposal 9: RAN2 to discuss whether there can be multiple pre-configured SRS configurations within a validity area.

4	Reference

- RP-223549, ‘New WID on Expanded and Improved NR Positioning’

- 3GPP TS 38.321 v17.3.0, ‘Medium Access Control (MAC) protocol specification’