3GPP TSG-RAN WG2 Meeting #123	R2-2308719

Toulouse, France, 21st August – 25th August 2023

Agenda Item:		7.7.4.2

Source:				ASUSTeK

Title:		Discussion on moving cell reference location for CHO

Document for: 	Discussion and Decision

- Introduction

To reduce the rate of frequently updating reference location for earth-moving cell, it was agreed that the location-based CHO would follow the method applied for cell re-selection to derive serving cell’s reference location. In this contribution, we further discuss the agreement of maintaining earth-moving cell reference location for CHO.

- Discussion

The location-based CHO (i.e. CondEvent D1) is fulfilled when distance between UE and the reference location of serving cell (i.e. referenceLocation1) becomes larger than a first threshold distanceThreshFromReference1 and distance between UE and the reference location of target cell (i.e. referenceLocation2) becomes shorter than a second threshold distanceThreshFromReference2. To apply the location-based CHO for earth-moving cells, it has been agreed in RAN2 #121 meeting that the UE derives real-time serving cell’s reference location based on mechanism applied for cell re-selection. And it is FFS whether to use the same method for target cell’s reference location. The agreement is quoted as below:

- For location-based CHO for earth-moving cells we follow the solution being investigated for cell reselection to allow the UE to derive the serving cell’s reference locations as the cells move. FFS whether the same mechanism can also be used for the candidate cell’s reference location

Observation 1: For earth-moving cell, it was agreed that location-based CHO follows the method for location-based cell re-selection to derive up-to-date location of serving cell.

For cell reselection, the movement issue for serving cell has been discussed in previous RAN2 meetings, and it has been agreed that the UE derives the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epoch time. The related agreements are quoted as below:

In RAN2 #121bis:

- RAN2 understands that for earth-moving cell reselection, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time;

- For earth-moving cell, new IE is introduced to indicate the reference location of serving cell;

- For cell (re)selection in earth-moving system, a distance threshold is introduced for location-based measurement initiation, which reuses distanceThresh in SIB19.

In RAN2 #122:

- Re-use epochTime-r17 in ntn-Config IE to provide the time reference for an Earth moving cell reference location.

The UE could receive an earth-moving cell reference location and a distance threshold distanceThresh via SIB19. And the UE performs measurements for cell re-selection based on the cell reference location and distance threshold. The cell reference location is a new IE (i.e. movingReferenceLocation in current running RRC CR [1]) corresponds to epochTime in NTN-Config of the serving cell. The UE could derive the trajectory of the earth-moving cell based on serving satellite ephemeris and epoch time.

Observation 2: For location-based cell re-selection, the UE derives up-to-date location of an earth-moving cell based on corresponding ephemeris and epoch time.

Since the UE would have available NTN-Config for the serving cell and target cell when receiving a CHO command, it could be confirmed that the same mechanism is applied to both serving cell and target cell of earth-moving cells. The UE could receive NTN-Config for the serving cell from SIB19. The UE could receive NTN-Config for the target cell in the CHO command or from neighbour cell list in SIB19. That is to say, the UE has corresponding ephemeris and epoch time for the serving cell and target cell when receiving the CHO command. In addition, the cell reference location of serving cell could be associated with epochTime in NTN-Config of the serving cell, and the cell reference location of target cell could be associated with epochTime in NTN-Config of the target cell. Then the UE could derive real-time location for the serving cell based on serving cell’s satellite ephemeris and corresponding epoch time, and derive real-time location for the target cell based on target cell’s e satellite ephemeris and corresponding epoch time.

Proposal 1: To apply location-based CHO for earth-moving cells, the UE derives up-to-date cell reference location for serving cell and target cell based on corresponding ephemeris and epoch time.

- Conclusion

In this contribution, we discuss the earth-moving cell reference location for location-based CHO and have following observations and proposal:

Observation 1: For earth-moving cell, it was agreed that location-based CHO follows the method for location-based cell re-selection to derive up-to-date location of serving cell.

Observation 2: For location-based cell re-selection, the UE derives up-to-date location of an earth-moving cell based on corresponding ephemeris and epoch time.

Proposal 1: To apply location-based CHO for earth-moving cells, the UE derives up-to-date cell reference location for serving cell and target cell based on corresponding ephemeris and epoch time.

- Reference

- R2-2306964, “Stage 3 Running RRC CR for NR NTN Rel-18.”