3GPP TSG-RAN WG2 Meeting #123	R2-2308746

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.19.3

Source:	Nokia, Nokia Shanghai Bell

Title:	RA aspects

WID/SID:	NR_redcap_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#122, it was discussed how to handle the Msg4 with larger number of PRBs, which has not been concluded. In addition, Msg2 issue was raised in R2-2304722. In this paper, we present our view on them.

# 2	Discussions

## 2.1	Large Msg4

If Rel-18 RedCap UE detects a DCI scheduling a Msg4 PDSCH with a larger number of PRBs, Rel-18 RedCap UE cannot process it [R2-2304619]. There were two options discussed in RAN2#122:

Option 1: The UE keeps running the ra-ContentionResolutionTimer and considers the Contention Resolution not successful upon expiry of the ra-ContentionResolutionTimer. No specification change is required.

Option 2: The UE considers the Contention Resolution as not successful upon reception of such Msg4 and stops the ra-ContentionResolutionTimer immediately. Specify in TS 38.321.

It was argued that option 1 leads to unnecessary power consumption due to monitoring of PDCCH for the remaining time of ra-ContentionResolutionTimer, which can be resolved by option 2.

However, not stopping the ra-ContentionResolutionTimer does not lead to more power consumption than initially expected because the UE is supposed to monitor the PDCCH until the expiration of ra-ContentionResolutionTimer unless Contention Resolution is successful. Similarly, not stopping the ra-ContentionResolutionTimer does not cause any additional latency than originally anticipated because the UE is supposed to retransmit an RAP only after the Contention Resolution is considered not successful, i.e., when the ra-ContentionResolutionTimer expires. Thus, this issue is more like an optimization that applies only to certain RedCap UEs when such Msg4 is received than being a critical problem to be resolved.

In addition, the gNB can set the timer value to a reasonably short value if power consumption and latency are concerns for RedCap UE. In this case, the gain of stopping ra-ContentionResolutionTimer might not be significant because the remaining time wouldn’t be long.

Proposal 1. If Rel-18 RedCap UE detects a DCI scheduling a Msg4 PDSCH with a larger number of PRBs, the UE keeps running the ra-ContentionResolutionTimer and considers the Contention Resolution not successful upon expiry of the ra-ContentionResolutionTimer. No specification change is required.

## 2.2	Large Msg2

In RAN1#111, it was agreed that

For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation.

When the UE receives an RAR in larger PRBs than what the UE can process per slot, the UE transmits Msg3 if there is enough time to process the RAR before Msg3 transmission. In detail, if the time gap between RAR reception and Msg3 transmission is larger than a certain value, i.e., NT,1 + NT,2 + 0.5 + X ms, the UE transmits Msg3. In this case, the UE moves on to the contention resolution step by starting the ra-ContentionResolutionTimer.

If the time gap between RAR reception and Msg3 transmission is short, RAN1 decided to leave it up to UE implementation. It seems that there could be various implementations [R1-2212534]. For example, the UE processes only 5MHz portion of RAR or the UE does not transmit Msg3 at all. It was also considered in RAN1 that the UE may drop the Msg3 initial transmission but perform Msg3 retransmission. The issue is when the Msg3 is not transmitted, hence, ra-ContentionResolutionTimer is not started.

In MAC specification, the MAC entity does not consider whether RAR decoding succeeded early enough to transmit Msg3 or not. The RAR reception will be simply considered successful when RAR decoding is completed. The question would be how the UL grant received in RAR is handled if Msg3 to be transmitted on the received UL grant cannot be generated in time.

Strictly following the specification, the MAC does not check whether the UL grant can be used or not for Msg3 transmission. When RAR reception is considered successful, the MAC entity generates Msg3, stores it in the Msg3 buffer, and delivers the UL grant to the HARQ entity while the HARQ entity will trigger a new transmission. For new transmission, the Msg3 will be stored in the HARQ buffer and transmission will be instructed to the physical layers. At this point, the physical layer may notice that the Msg3 is received too late and cannot be transmitted, but MAC will start the ra-ContentionResolutionTimer as the MAC entity transmitted Msg3. Thus, the UE would continue RA procedure.

However, some UE implementation may check whether the Msg3 can be transmitted on the UL grant or not. In this case, the ra-ContentionResolutionTimer may not start as the MAC may ignore the UL grant and not generate Msg3. Then, the UE will remain trapped in the RA procedure without being able to exit.

Observation 1. If time gap between RAR reception and Msg3 transmission is short, the UE may not generate/transmit Msg3 and ra-ContentionResolutionTimer may not start depending on the implementation.

Three options were proposed [R2-2304722]:

Option 1: RAR is considered unsuccessful and PRACH retransmission is triggered.

Option 2: Ignore the received RAR and continue monitoring RAR till RAR window expires.

Option 3: Start contention resolution timer even if Msg3 is not transmitted.

In our view, option 1 or 2 is not desirable because the gNB may send such RAR again at the next RA attempt, which only adds unexpected latency to the RA procedure. Given that the UE already received an RAR, which can be decoded in the end, continuing the ongoing RA procedure by allowing transmission or retransmission of Msg3 would be helpful. For this, ra-ContentionResolutionTimer should be running, i.e., option 3. However, option 3 needs more discussion.

In NR-U, the MAC starts the ra-ContentionResolutionTimer even when Msg3 is not transmitted over the air due to LBT failure [R2-1914301]. It should be noted that, in the current specification, this timer is started when the MAC entity transmits the Msg3 [R2-2003050]. In NR-U, as the MAC entity instructs transmission of Msg3 with the UL grant to the physical layer (but transmission is not performed only in the physical layer), ra-ContentionResolutionTimer shall start. In RedCap, however, if the UE ignores the UL grant, the UE will not start ra-ContentionResolutionTimer as there is no Msg3 transmission at all from MAC perspective. Therefore, in order to start the ra-ContentionResolutionTimer, the UE should process the RAR and the received UL grant for transmission of Msg3 even though Msg3 cannot be transmitted in the physical layer. Otherwise, the UE will be stuck in the RA procedure.

Observation 2. In TS 38.321, it is specified that MAC entity shall start ra-ContentionResolutionTimer if Msg3 is transmitted from MAC perspective.

Observation 3. If the UE ignores the UL grant, the MAC entity will not start ra-ContentionResolutionTimer because the MAC does not consider the Msg3 is transmitted.

Proposal 2. The UE processes the RAR and generates Msg3 regardless of whether it can be transmitted on the UL grant received in the RAR or not in the physical layer. RAN2 discuss how/where to specify this behaviour.

With proposal 2, ra-ContentionResolutionTimer starts even if the UE receives a RAR PDSCH which is larger than the maximum number of unicast PRBs that the UE can process per slot, and the gap between the RAR reception and Msg3 transmission is small as agreed in RAN1 #111.

# 3	Conclusion

In this contribution, we discussed potential impact to RA procedure based on RAN1 agreement, and made proposals:

Proposal 1. If Rel-18 RedCap UE detects a DCI scheduling a Msg4 PDSCH with a larger number of PRBs, the UE keeps running the ra-ContentionResolutionTimer and considers the Contention Resolution not successful upon expiry of the ra-ContentionResolutionTimer. No specification change is required.

Observation 1. If time gap between RAR reception and Msg3 transmission is short, the UE may not generate/transmit Msg3 and ra-ContentionResolutionTimer may not start depending on the implementation.

Observation 2. In TS 38.321, it is specified that MAC entity shall start ra-ContentionResolutionTimer if Msg3 is transmitted from MAC perspective.

Observation 3. If the UE ignores the UL grant, the MAC entity will not start the ra-ContentionResolutionTimer because the MAC does not consider the Msg3 is transmitted.

Proposal 2. The UE processes the RAR and generates Msg3 regardless of whether it can be transmitted on the UL grant received in the RAR or not in the physical layer. RAN2 discuss how/where to specify this behaviour.