3GPP TSG-RAN WG2 #123	R2-2308814

Toulouse, France, 21st – 25th Aug 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on Cell barring for eRedCap

Document for:	Discussion and decision

Agenda Item:	7.19.3

- Introduction

A new WID on further NR RedCap UE complexity reduction (FS_NR_redcap_enh) was approved at the RAN#97-e meeting and revised at the RAN#98-e meeting [1]. The objectives of the WI are shown below.

The objective is to specify support for the following enhancements: Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2 and CT1 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99-e regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

In this contribution, we describe our view on cell barring for Rel-18 eRedCap.

- Discussion

In last RAN2 meeting (RAN2 #122), following agreement was made [2];

We introduce R18 versions of 1Rx and 2Rx barring bits and we don’t introduce a R18 version of the HD-FDD allowed-bit, i.e., the R17 HD-FDD allowed-bit is reused for and applied by R18 eRedCap UEs.

So RAN2 has confirmed that following 2 barring bits will be introduced.

cellBarred-eRedCap1Rx-r18,

cellBarred-eRedCap2Rx-r18

RAN2 also discussed further barring bit such as separate cell bar IE to identify [BW3+PR1] and [PR1 only]. But RAN2 has postponed this topic;

Proposal 5: No need to introduce separate cell bar IEs in SIB1 for Rel-18 eRedCap UEs with BW3+PR1 and Rel-18 eRedCap UEs with PR1 only.

Discussion P5:

- Ericsson thinks the NW should be allowed to allow e.g. only reduced peakrate UEs but not BB BW-limited UEs. Nokia and Sierra Wireless agrees. Huawei wants to ACK the agreement from plenary about initial access looking the same for Rel-18 eRedCap UEs with BW3+PR1 and Rel-18 eRedCap UEs with PR1 only.

- ZTE thinks that the point of separate barring bits would be to alleviate load, and if there is no such issue we shouldn’t add separate bits, Apple agrees with this view.

- Postponed

To discuss whether it would be beneficial to cell bar [BW3+PR1], [PR1 only] independently, it is better to discuss UE capability first to discuss how these two functions are related.

The reason is that how the two functions [BW3+PR1], [PR1 only] are put into the UE determines how the NW side that accommodates such UE has to provide resources for them. If it is identified that there are cases where it is necessary that the network side provides different resources for each function, it is preferred that the cell bar can be configured independently. On the other hand, if the two functions [BW3+PR1], [PR1 only] are dependent, or if the network has the option to apply the function or not, further control as a cell bar becomes unnecessary.

Proposal 2. 	RAN2 should first proceed with the UE capability discussion before concluding the cell bar discussion.

According to the features list from RAN1[3][4], it is mentioned that the followings about features for eRedcap.

- 2 feature groups are introduced;

  - FG48-1: RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1

  - FG48-2: RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1

- FG48-1 is prerequisite for FG48-2

- Add FG48-1 to the prerequisites for the two feature groups below

  - FG28-1a: RRC-configured DL BWP without CD-SSB or NCD-SSB

  - FG28-3: Half-duplex FDD operation type A for RedCap UE

Where FG48-1 means [BW3+PR1] and FG48-2 means [PR1 only]. Thus, all of the UEs supporting eRedcap will support a 5 MHz bandwidth limitation as FG48-1 and will optionally support a 20 MHz bandwidth as FG48-2.

Observation 	All eRedCap UEs support communication at [BW3+PR1], i.e., 5 MHz bandwidth. And in this situation, further barring bits are ineffective.

In this situation, the only case that can be created on the NW side by setting barring to separate [BW3+PR1] and [PR1 only] is to create a "cell that excludes BW3+PR1 UEs".

However, there is no particular benefit to the NW side from creating such a restriction, but only a drawback of reduced efficiency by increasing the number of useless SIB1 barring bits.

Therefore, only the last agreed 2 bits (cellBarred-eRedCap1Rx-r18, cellBarred-eRedCap2Rx-r18) are sufficient for adding Barring bits in eRedCap, and no further Barring bits are needed.

Proposal 2	Last agreed 2 bits (cellBarred-eRedCap1Rx-r18, cellBarred-eRedCap2Rx-r18) are sufficient for adding Barring bits in eRedCap, and no further Barring bits are needed.

- Summary and proposal

Proposal 1. 	RAN2 should first proceed with the UE capability discussion before concluding the cell bar discussion.

Observation 	All eRedCap UEs support communication at [BW3+PR1], i.e., 5 MHz bandwidth. And in this situation, further barring bits are ineffective.

Proposal 2	Last agreed 2 bits (cellBarred-eRedCap1Rx-r18, cellBarred-eRedCap2Rx-r18) are sufficient for adding Barring bits in eRedCap, and no further Barring bits are needed.

- Reference

[1] RP-223544, Enhanced support of reduced capability NR devices, Ericsson.

[2] R2-2306550, Report from eRedCap breakout session, Session chair (Ericsson)

[3] R2-2307013, LS on Rel-18 RAN1 UE features list for NR after RAN1#113, RAN1

[4] R2-2304611, LS on updated Rel-17 RAN1 UE features lists for NR after RAN1#112bis-e, RAN1