3GPP TSG-RAN WG2 Meeting #123		R2-2308834

Toulouse, France, 21th – 25th August, 2023

Agenda Item:	7.8.3

Source: 	vivo

Title:         	Discussion on flight path reporting

Document for: 	Discussion and Decision

- Introduction

In RAN2#122 meeting, RAN2 has reached the following agreements related to the flight path reporting for NR UAV [1]:

Agreements

- The network can configure the UE to trigger a flightpath update notification based on a configured delta time (when timestamp is configured to be reported) or distance configuration.

In this contribution, we will further discuss the remaining issues for NR UAV flight path reporting.

- Discussion

Triggering for updating flight path:

According to the agreement made in RAN2#122, RAN2 has agreed that the network can configure the UE to trigger a flightpath update notification based on a configured delta time or distance configuration. One straightforward way to specify the delta time and distance configuration in the specification is shown below. The network can configure the time and distance threshold in FlightPathInfoReportConfig.

FlightPathInfoReportConfig information element

-- ASN1START

FlightPathInfoReportConfig-r18 ::= SEQUENCE {

maxWayPointNumber-r18				INTEGER (1..maxWayPoint-r18),

includeTimeStamp-r18				ENUMERATED {true}					OPTIONAL,

wayPointThreshold-r18               ENUMERATED 	{0, value1, value2..valueN}	     OPTIONAL,

timeStampThreshold-r18              ENUMERATED {0, value1, value2..valueN}	     OPTIONAL

}

-- ASN1STOP

wayPointThreshold-r18/ timeStampThreshold-r18 can be optionally configured, both can contain value zero. For example, if timeStampThreshold-r18 is not configured, it means the UE is not allowed to report flight path change indication based on the change of time stamp. If timeStampThreshold-r18 is configure with value zero, it means the UE is allowed to report flight path change indication if any time stamp is changed.

Proposal 1: a waypoint/timestamp delta threshold with value zero can be optionally configured.

Based on the above example, for any waypoint/timestamp has changed more than the configured threshold except that 0 or infinity, the UE triggers flightpath update indication.

Proposal 2: if any waypoint/timestamp has changed more than the configured threshold except that 0 or infinity, the UE triggers flightpath update indication.

Flight path update indication:

Basically, if the UAV have a new plan, the entire path will be re-generated. From this point, delta reporting has few benefits. In addition, currently, delta signaling is not supported for UAI, which requires the network to store this information. And there is no reason to introduce the complexity just for UAV. Therefore, as a simple solution, there is no need to introduce delta signaling for flight path update. Thus, a single indication for both initial and updated flightpath availability is sufficient. Thus, the below proposal is given:

Proposal 3: Delta signaling for flight path report updating is not supported.

Proposal 4: Confirm a single indication is used for both initial and updated flightpath availability.

- Conclusion

In this paper, the following proposal are given:

Proposal 1: a waypoint/timestamp delta threshold with value zero can be optionally configured.

Proposal 2: if any waypoint/timestamp has changed more than the configured threshold except that 0 or infinity, the UE triggers flightpath update indication.

Proposal 3: Delta signaling for flight path report updating is not supported.

Proposal 4: Confirm a single indication is used for both initial and updated flightpath availability.

- References

[1] R2-2306543, Report from Session for on NES, UAV Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning.