3GPP TSG-RAN WG2 Meeting #123	R2- 2308879

Toulouse, France, 21-25 August, 2023

Title	Signalling aspects for Msg1 repetition

Source	LG Electronics

Document for	Discussion and Decision

Agenda Item	7.21.2

1 	Introduction

In NR coverage enhancements, it is agreed to support PRACH repetition, at least for same beams. The corresponding objective in WID [1] is as follows:

The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. The detailed objectives of the work item are as follows: - Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

This contribution continues the discussions on signalling aspects on PRACH repetition.

2	Discussion

PRACH partitioning to support PRACH repetition

In RAN2#121bis-e meeting [2], it is already agreed to reuse Rel-17 RACH partitioning framework in order to support a separate RACH resource for PRACH repetition. In addition, the RA resource separation with different repetition number is also supported:

Agreements - Msg1 repetition with different repetition number {2, 4, 8} are treated a separate feature, and a RACH partition is associated with a specific repetition number (Stage 3 details are FFS, e.g. we should not use all the spare values in the current IE)

In Rel-17 RACH partitioning framework, four spare fields are reserved in order to support the additional RACH partitioning functionalities for the future release. Reserved spare fields should be used for new feature indication in order to define the separate feature for PRACH repetition.

However, the separate RACH resource for different number of PRACH repetition should not be configured by the separate feature indication. Given that there are only four spare fields remained for additional feature indication, using the separate spare field for each number of PRACH repetitions would cause restriction to other WIs which need a spare field for RACH partitioning functionality. Therefore, for the feature indication for PRACH repetition, one common spare field should be used for PRACH repetition. An example feature indication can be defined as follows:

-- ASN1START -- TAG-FEATURECOMBINATION-START FeatureCombination-r17 ::= SEQUENCE { redCap-r17 ENUMERATED {true} OPTIONAL, -- Need R smallData-r17 ENUMERATED {true} OPTIONAL, -- Need R nsag-r17 NSAG-List-r17 OPTIONAL, -- Need R msg3-Repetitions-r17 ENUMERATED {true} OPTIONAL, -- Need R Msg1-Repetitions-r18 ENUMERATED {true} OPTIONAL, -- Need R spare3 ENUMERATED {true} OPTIONAL, -- Need R spare2 ENUMERATED {true} OPTIONAL, -- Need R spare1 ENUMERATED {true} OPTIONAL -- Need R } NSAG-List-r17 ::= SEQUENCE (SIZE (1.. maxSliceInfo-r17)) OF NSAG-ID-r17 -- TAG-FEATURECOMBINATION-STOP -- ASN1STOP

Proposal 1. One spare field is commonly used for each number of Msg1 repetitions.

Meanwhile, in RAN1#112[3], it is agreed that the RACH resource for different numbers of PRACH repetition would be separated.

Agreement For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions. - If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported. - FFS: details Agreement Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx beams.

In order to support the RACH resource differentiation for different number of PRACH repetition, it is proposed to configure the repetition number within the feature combination configuration for PRACH repetition (i.e., in FeatureCombinationPreambles IE with featureCombination = Msg1-repetition). For example, RA resource configuration within the RACH partition for Msg1 repetition can be configured as follows:

FeatureCombinationPreambles-r17 ::= SEQUENCE { featureCombination-r17 FeatureCombination-r17, startPreambleForThisPartition-r17 INTEGER (0..63), numberOfPreamblesPerSSB-ForThisPartition-r17 INTEGER (1..64), ssb-SharedRO-MaskIndex-r17 INTEGER (1..15) OPTIONAL, -- Need S groupBconfigured-r17 SEQUENCE { ra-SizeGroupA-r17 ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1}, messagePowerOffsetGroupB-r17 ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18}, numberOfRA-PreamblesGroupA-r17 INTEGER (1..64) } OPTIONAL, -- Need R separateMsgA-PUSCH-Config-r17 MsgA-PUSCH-Config-r16 OPTIONAL, -- Cond MsgAConfigCommon msgA-RSRP-Threshold-r17 RSRP-Range OPTIONAL, -- Need R rsrp-ThresholdSSB-r17 RSRP-Range OPTIONAL, -- Need R deltaPreamble-r17 INTEGER (-1..6) OPTIONAL, -- Need R [[ RA-ConfigForRepetition2 Msg1-repetitionConfig Optional, RA-ConfigForRepetition4 Msg1-repetitionConfig Optional, RA-ConfigForRepetition8 Msg1-repetitionConfig Optional, ]] ... } Msg1-repetitionConfig-r18 ::= SEQUENCE { startPreamble-r18 INTEGER (0..63), numberOfPreamblesPerSSB-r18 INTEGER (1..64), ssb-SharedRO-MaskIndex-r18 INTEGER (1..15) OPTIONAL, repNumTransMax-r18 ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200} OPTIONAL, ... }

Proposal 2. The separated RA resource for each repetition number is configured within the RACH configuration of Msg1 repetition.

3	Conclusion

In this paper, we discussed our views on signalling aspects to support the PRACH repetition. The discussion includes the following proposals:

Proposal 1. One spare field is commonly used for each number of Msg1 repetitions.

Proposal 2. The separated RA resource for each repetition number is configured within the RACH configuration of Msg1 repetition.

4	Reference

[1] RP-221858, Revised WID on further NR coverage enhancements

[2] RAN2#121bis-e Report from Break-out session on Further NR Coverage enhancements (ZTE Corporation)

[3] Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0