3GPP TSG-RAN WG2 #123	R2-2308897

Toulouse, France, August 21st – 25th 2023

Agenda Item:	7.13.5

Source:	Ericsson

Title:	Enhancements of SON reports for NR-U

Document for:	Discussion, Decision

# 1	Introduction

Given the progress in RAN2#123, only few open issues are left to be discussed in the context of NR-U.

In this paper, we discuss those few open issues that are partially addressed also in the email discussion [Post122][590][R18 SONMDT] Open issues of SON NR-U (Ericsson).

# 2	Discussion

### 2.1 Enhancements to the RA-InformationCommon

The first issue to be addressed on the RA-InformationCommon is related to the following agreement:

From RAN2#122:

On how to represent the preamble transmission attempts blocked by LBT,

Introduce a field (or reusing the existing field) that counts the number of preamble transmissions blocked by LBT per RA procedure, and a flag indicating transmission failures experienced right before beam switching. Details can FFS.

It was discussed whether the number of preamble transmissions should be represented by a new field, or if some existing field can be reused. Related to the latter approach, it was proposed that the fields numberOfPreamblesSentOnSSB and numberOfPreamblesSentOnCSI-RS could be modified, such that they account for all the preamble transmission attempts both the ones blocked by LBT and the successful ones. From our point of view, the main problem of this option is that currently the maximum  values for the numberOfPreamblesSentOnSSB and numberOfPreamblesSentOnCSI-RS is limited to 200. This value was dimensioned considering only the RA preambles actually transmitted. If now, this field also considers the preambles blocked by LBT, the total numberOfPreamblesSentOnSSB (numberOfPreamblesSentOnCSI-RS) might exceed 200. In our view this should be avoided, and the approach of introducing a new field is more robust from a technical perspective.

A new field is introduced to count the number of preamble transmissions blocked by LBT per RA procedure.

Proponents of the approach of reusing existing fields claim that an advantage of this approach is that the UE will always set the numberOfPreamblesSentOnSSB and numberOfPreamblesSentOnCSI-RS to a value. Otherwise, there is the problem that numberOfPreamblesSentOnSSB and numberOfPreamblesSentOnCSI-RS are mandatory fields and cannot be set to 0, which could happen in case of no preamble transmitted in an SSB. However, we note that the same issue affects also the perRAAttemptInfoList that should be always set. We note that leaving empty the entry of the perRAAttemptInfoList (i.e. avoid setting the fields in perRAAttemptInfoList) cannot be a viable solution, because there are other scenarios in which an entry of the perRAAttemptInfoList can be left empty (e.g. CFRA).

Hence, irrespective of the solution, RAN2 should discuss how to handle the numberOfPreamblesSentOnSSB/numberOfPreamblesSentOnCSI-RS and perRAAttemptInfoList. For example, one solution could be that it is clarified in the specification (e.g. in a NOTE) that in case all the successive LBT preamble transmissions in a beam are stopped by LBT failures, it is left to the implementation how to set the numberOfPreamblesSentOnSSB/numberOfPreamblesSentOnCSI-RS. RAN2 should then discuss how the UE should inform the network that all the preamble transmission attempts in that beam were blocked by LBT.

In case all the successive LBT preamble transmissions in a beam are stopped by LBT failures, it is left to the UE implementation how to set the numberOfPreamblesSentOnSSB/numberOfPreamblesSentOnCSI-RS.

RAN2 to discuss how the UE should inform the network that all the preamble transmission attempts in a beam were blocked by LBT.

Multiple RA procedures (performed as part of LBT recovery) may belong to multiple BWPs, hence it would be beneficial to include in the RA-Report the multiple RA information associated the random access attempts performed in the various BWPs, i.e. a list of RA-InformationCommon IEs.

Related to the other BWP, except the last one for which more detailed information will be included, RAN2 agreed the following:

From RAN#122:

For the other BWPs in which the UE experienced the consistent LBT failure, the UE logs in the RA-InformationCommon:

- a.	The locationAndBandwidth information of the BWP

- b.	The subcarrierSpacing information of the BWP

- c.	The absoluteFrequencyPointA information of the BWP ( How to log once for all the BWPs of the cell is FFS)

It seems natural for the UE to include all the information associated to all the BWPs in which consistent LBT failures were experienced prior to the successful completion of RA. This BWP information should be included in the chronological order of operation, so that the network can retrieve the order in which the UE performed the RA procedures.

The UE includes in the RA-Report all the BWP information in which the UE experienced the consistent LBT failures before the successful completion of the RA.

The various BWPs in which the UE experienced the consistent LBT failures prior the RLF/HOF/successful RA, are logged in the RA-InformationCommon in chronological order.

Related to the other BWPs where the UE performed random access, besides the minimum set of information already agreed, in our view it is beneficial that the UE also includes the number of LBT failures experienced in those other BWPs in which consistent LBT failure was declared. One may claim that this value would be the same as the consistent LBT failure counter (lbt-FailureInstanceMaxCount), however we note that the lbt-FailureInstanceMaxCount may be reset during the RA, e.g. upon lbt-FailureDetectionTimer expiry, hence the failures may be more than the configured lbt-FailureInstanceMaxCount.

For each BWP of the PCell, except the last BWP, in which the UE experienced consistent LBT failures, the UE logs the number of LBT failures experienced in the BWP during the RA procedure.

### 2.2 How to aid the energy detection configuration

The energy detection (ED) configuration, i.e. energyDetectionConfig, is provided to the UE as part of the channel access configuration for a serving cell. The ED configuration consists of a maximum ED threshold and of an ED threshold offset. A proper ED configuration is crucial for the UE performances in the unlicensed spectrum, because that impacts the likelihood of the UE to access the NR-U channel and at the same time it influences the coexistence of different devices in the unlicensed spectrum.

In fact, according to the TS 37.213, the UE compares detected power with the energy detection threshold (EDT) to access the channel.

- A channel access procedure is a procedure based on sensing that evaluates the availability of a channel for performing transmissions. The basic unit for sensing is a sensing slot with a duration Tsl=9us. The sensing slot duration Tsl is considered to be idle if an eNB/gNB or a UE senses the channel during the sensing slot duration, and determines that the detected power for at least 4us within the sensing slot duration is less than energy detection threshold XThresh. Otherwise, the sensing slot duration Tsl is considered to be busy.

As mentioned above UE performs channel access based on sensing that evaluates the availability of a channel for performing transmission. UE measures the detected power and determines if the channel is busy or free based on ED threshold. If the ED threshold is set to a too low value, then there it might be more difficult for a 3GPP UE to access the unlicensed spectrum in this cell, i.e. more LBT failures will be experienced, whereas if that is set to a too high value, the interference in the unlicensed spectrum may increase potentially impacting a fair coexistence with other devices.

A proper energy detection threshold configuration is crucial to ensure a fair coexistence of devices in the unlicensed spectrum, especially for the random access.

However, the network does not know the power detected by the UE. In our opinion, the information of the detected power and the applied EDT used by the UE at the time of LBT issue when executing a random-access procedure would be highly beneficial to pinpoint whether a random-access related issue was due a sub-optimal ED configuration used by the UE at the time of random-access procedure.

RAN2 includes some information per RA procedure to aid the network to properly configure the energy detection configuration.

Many different solutions have been proposed so far which were deemed to complicate for the UE. We believe that at least the UE could log information on whether the detected power is higher than the configured maximum ED threshold. In fact, an LBT failure may be due either to a too little value of the applied ED threshold, or to a too little value of the configured maximum ED threshold. For the former case, the network cannot do much, since the ED value applied by the UE may depend on the UE implementation. On the other hand, for the latter case, the LBT failure may depend on the network implementation, and the network may then decide to adjust the configuration of the maximum ED threshold (e.g. if too many LBT failures are experienced due to detected power higher than the maximum configured ED threshold).

An LBT failure may depend on the detected power being higher than the applied ED threshold and lower than the maximum configured ED threshold, or on the detected power being only higher than the maximum configured ED threshold.

The UE indicates whether the average detected power per RA procedure was higher than the configured maximum ED threshold (maxEnergyDetectionThreshold).

### 2.2 Enhancements to the RLF-Report

#### 2.2.1 RSSI reporting

Related to RSSI reporting in the RLF-Report, the following agreements were taken in RAN2#122:

From RAN2#122:

The UE logs the available RSSI measurement in the RLF-Report. FFS in which case.

8	The UE should log the following RSSI values in the RLF-Report:

a.	For RLF, the latest measured RSSI of the NR-U channel of the last serving cell if measRSSI-ReportConfig is configured for the corresponding frequency.

b.	FFS: For HOF, the latest measured RSSI of the NR-U channel of the source cell, and  the latest measured RSSI of the NR-U channel of the target cell, if measRSSI-ReportConfig is configured for the corresponding frequency.

Related to RLF case, in our view the RSSI measurements of the source cell should be considered also for the HOF. As in classical MRO, the network can use the RSSI information (together with the legacy RSRP/RSRQ/SINR) to compare the quality of the frequency of the source cell with the quality of the frequency of the target cell or neighbouring cells. For example, if the RLF-Report shows very bad RSSI on the target frequency, and still acceptable RSSI on the source frequency, then the NW can conclude that this HO was executed at a bad point in time, and the UE could have been kept longer time in the source frequency. Accordingly, the NW can adjust the thresholds on HO triggering.
Please note that this is exactly the same reasons for which the RSRP/RSRQ/SINR of the source as well as neighbouring cells is included in legacy.

The RSSI measurement results of the frequency associated to the last serving cell are included in the RLF-Report for the source PCell (in case of HO failure) and for the PCell (in case of RLF), if the measRSSI-ReportConfig is configured for such frequency.

Similarly, for neighbouring frequencies, if the RLF-Report shows very bad RSSI on the target frequency, but good RSSI on another neighbouring frequency, then the NW can conclude that this HO was executed to a wrong frequency, and hence it can adjust the thresholds on HO triggering.

The UE includes in the RLF-Report the available RSSI measurement results of the frequencies associated to the neighbouring cells other than the source PCell (in case of HO failure) or PCell (in case of RLF), if the measRSSI-ReportConfig is configured for such frequencies.

#### 2.2.2 RA-InformationCommon reporting in the RLF-Report

In RAN2#122, it was agreed to include the RA-InformationCommon in the RLF-Report as in legacy, i.e. in case of HOF and when the RLF cause is randomAccessProblem or beamFailureRecoveryFailure:

From RAN2#122: The UE logs RA-InformationCommon including LBT info in the RLF-Report, in case of HOF and when the RLF cause is randomAccessProblem or beamFailureRecoveryFailure (as in legacy).

Other reasons to include the RA-InformationCommon were discussed in RAN2. At least the case in which the RLF cause is set to ‘lbtFailure’ should be considered. In fact, in case the RLF is declared because of ‘lbtFailure’, it means that the UE has executed multiple random-access procedures in different BWPs of the PCell and finally when the UE declares RLF, that is because the RA were attempted in all BWPs configured with RACH with no success. That is obviously an RA issue for which RA-InformationCommon should be included in the RLF-Report.

The UE logs the RA-InformationCommon in the RLF-Report when the RLF cause is lbtFailure and the UE was performing random access at the moment of RLF.

Related to which BWP include in the RAInformationCommon within the RLF-Report, the following was agreed:

As baseline, RAN2 assumes the following:

a.	Enhancements discussed for the RA-InformationCommon for the RA-Report are applicable also to the RLF-Report

b.	The detailed “per RA attempt info” are only reported in the RLF-Report for the last RA procedure before RLF/HOF, FFS whereas limited information are reported for the other BWPs in which consistent LBT failure is detected

c.	The above bullets may be revisited case by case depending on future agreements.

We note that for the case of RA-Report, the UE includes the information associated to all BWPs in which consistent LBT failure was detected in the RA-Report. There seems to be no clear reason for not including the information about consistent LBT failures in the RA-InformationCommon included in the RLF-Report.

The benefit of this is quite clear. In NR-U systems, the reason for switching the BWP upon consistent LBT failure is because the UE is expected to experience different LBT performances in another BWP, i.e. LBT performed in different chunks of the NR-U band may lead to different LBT results.
Hence knowing the BWPs in which consistent LBT failure occurred prior to the RLF/HOF allows the NW to identify the problematic BWPs and optimize the usage of those. For example, the network can figure out that the consistent LBT failures are persistently experienced in a certain BWP, and then optimize the configuration/allocation of this BWP in the NR-U spectrum.

The UE includes in the RLF-Report the locationAndBandwidth, and the subcarrierSpacing of the BWPs in which the UE experienced the consistent UL LBT failures prior the RLF/HOF.

#### 2.2.3 On the inclusion of lbt-FailureRecoveryConfig in the RLF-Report

An LS [2] was sent to RAN3 asking the possibilities of keeping the LBT failure recovery configuration at the network side. In the reply [3] to LS, RAN3 has replied that although one similar mechanism exists in RAN3, it would be practically minimal to store such information as LBT recovery procedure for a long time since that limits the network functionality. This is shown on the following excerpt of RAN3 LS [2].

RAN3 observes though, that the above mechanism was designed with intention to retrieve UE context or the configuration information if the UE attempts reconnection and reports the failure right after connection failure; if the failure information is fetched from the UE hours after the failure, then the likelihood that the source and the last serving node can retrieve the needed information depends on RAN implementation and is practically minimal (this depends on RAN implementation, e.g., how long the gNB stores the UE context or how long that allocated C-RNTI is not reused by the RAN).

According to RAN3, a network-based solution to store LBT-Recovery config would be limiting network functionality and thus less practical in particular when re-establishment procedure fails.

In order to minimize the UE impact and the size of the RLF-Report, we propose a UE-based solution where UE can store the LBT-Recovery configuration information in the RLF report only in case of re-establishment procedure failure after the RLF.

Other solutions have been discussed to address this problem, e.g. in [2], such as introducing a configuration index that the gNB should provide to the UE in each RRCReconfiguration message. This index would then be provided back to the gNB in the RLF-Report, and the gNB would then be in charge of retrieving the corresponding configuration. We first note that when the UE context is still available at the network, the network can leverage on the existing information provided in the RLF report, such as the C-RNTI and the timeSinceFailure to retrieve the corresponding configuration previously stored. The approach proposed in [2] also implies that the network has to store a mapping between the configuration index and a configuration. Hence from a technical point of view, the approach proposed in [2] has to consider as an alternative to a network-based solution that the standard already allows. Given that the time left is short and stage-3 details (ASN.1 and procedural text) should be sorted out, we suggest postponing discussion on this optimization to a future release.

Introducing a configuration index to retrieve a NW configuration has to be considered as an optimization whose feasibility can be assessed in future releases, given that NW-based solutions already exist when the UE context is still available in the network.

UE logs the lbt-FailureRecoveryConfig in the RLF-Report, only in case of re-establishment procedure failure (i.e., when UE performs transition to RRC_IDLE state).

### 2.3 Enhancements to the SHR

In the current specification the UE logs the RA information in the SHR, only when the SHR is triggered due to T304 timer value becoming larger than a certain threshold. We note also for this case, that the UE may experience LBT failures or even trigger random access in multiple BWPs due to consistent UL LBT failures just before successfully completing the HO. The information associated to LBT failures during RA and all the possible random access procedures initiated just before the successful HO completion, may be of interest.

The ra-InformationCommon including the new Rel.18 information (i.e. the number of UL LBT failures during HO, the info on the multiple BWPs in which consistent UL LBT failures was triggered), are included in the SHR if T304 triggering conditions is fulfilled.

In addition, similar to the legacy SHR, RSSI measurements of source/target/neighbouring frequencies should be included. In SHR, the source/target/neighbouring frequencies are always included irrespective of the SHR triggering condition. That is because in order to optimize the HO, and reduce the probability of possible RLF or performance degradation, the network has to evaluate the quality of neighbouring frequencies. For example, if SHR is generated due to T310 triggering condition, it could be that the source cell quality was not optimal; however, in order to avoid this issue, e.g. taking the HO at an earlier point in time to another cell, the network has to know the quality of the target/neighbouring cells.

The SHR includes the RSSI measurements of the frequencies associated to the source/target/neighbouring cells, if the measRSSI-ReportConfig is configured for those frequencies.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A proper energy detection threshold configuration is crucial to ensure a fair coexistence of devices in the unlicensed spectrum, especially for the random access.

Observation 2	An LBT failure may depend on the detected power being higher than the applied ED threshold and lower than the maximum configured ED threshold, or on the detected power being only higher than the maximum configured ED threshold.

Observation 3	According to RAN3, a network-based solution to store LBT-Recovery config would be limiting network functionality and thus less practical in particular when re-establishment procedure fails.

Observation 4	Introducing a configuration index to retrieve a NW configuration has to be considered as an optimization whose feasibility can be assessed in future releases, given that NW-based solutions already exist when the UE context is still available in the network.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A new field is introduced to count the number of preamble transmissions blocked by LBT per RA procedure.

Proposal 2	In case all the successive LBT preamble transmissions in a beam are stopped by LBT failures, it is left to the UE implementation how to set the numberOfPreamblesSentOnSSB/numberOfPreamblesSentOnCSI-RS.

Proposal 3	RAN2 to discuss how the UE should inform the network that all the preamble transmission attempts in a beam were blocked by LBT.

Proposal 4	The UE includes in the RA-Report all the BWP information in which the UE experienced the consistent LBT failures before the successful completion of the RA.

Proposal 5	The various BWPs in which the UE experienced the consistent LBT failures prior the RLF/HOF/successful RA, are logged in the RA-InformationCommon in chronological order.

Proposal 6	For each BWP of the PCell, except the last BWP, in which the UE experienced consistent LBT failures, the UE logs the number of LBT failures experienced in the BWP during the RA procedure.

Proposal 7	RAN2 includes some information per RA procedure to aid the network to properly configure the energy detection configuration.

Proposal 8	The UE indicates whether the average detected power per RA procedure was higher than the configured maximum ED threshold (maxEnergyDetectionThreshold).

Proposal 9	The RSSI measurement results of the frequency associated to the last serving cell are included in the RLF-Report for the source PCell (in case of HO failure) and for the PCell (in case of RLF), if the measRSSI-ReportConfig is configured for such frequency.

Proposal 10	The UE includes in the RLF-Report the available RSSI measurement results of the frequencies associated to the neighbouring cells other than the source PCell (in case of HO failure) or PCell (in case of RLF), if the measRSSI-ReportConfig is configured for such frequencies.

Proposal 11	The UE logs the RA-InformationCommon in the RLF-Report when the RLF cause is lbtFailure and the UE was performing random access at the moment of RLF.

Proposal 12	The UE includes in the RLF-Report the locationAndBandwidth, and the subcarrierSpacing of the BWPs in which the UE experienced the consistent UL LBT failures prior the RLF/HOF.

Proposal 13	UE logs the lbt-FailureRecoveryConfig in the RLF-Report, only in case of re-establishment procedure failure (i.e., when UE performs transition to RRC_IDLE state).

Proposal 14	The ra-InformationCommon including the new Rel.18 information (i.e. the number of UL LBT failures during HO, the info on the multiple BWPs in which consistent UL LBT failures was triggered), are included in the SHR if T304 triggering conditions is fulfilled.

Proposal 15	The SHR includes the RSSI measurements of the frequencies associated to the source/target/neighbouring cells, if the measRSSI-ReportConfig is configured for those frequencies.

# 4	References

- R3-225241, LS on NR-U support for MRO, 3GPP TSG-RAN WG2 #117-e

- R2-2305424, Discussion on SON for NR-U, Nokia, Nokia Shanghai Bell