3GPP TSG-RAN WG2#123	R2-2308903

Toulouse, France, 21 –25 August 2023

Agenda Item:	7.25.4

Source:	Ericsson

Title:	Satellite IMT-2020 self-evaluation: CP latency

Document for:	Discussion, Decision

# Introduction

ITU-R has defined requirements for the satellite component of IMT-2020 [1]. The set of requirements and the corresponding evaluation guidelines are based on the terrestrial procedure, to which 3GPP has submitted TR 37.910 [2]. Similar to the terrestrial case, the requirements include a CP latency target which has been set to 40 ms [1] to take into account the longer propagation delay inherent to satellite communications.

In this contribution, we make an evaluation of the CP latency in NR NTN with different configurations and show how the target latencies are fulfilled.

# Discussion

As defined in ITU-R Report M.2514 [1], control plane latency refers to the transition time from a most “battery efficient” state (e.g., Idle state) to the start of continuous data transfer (e.g. Active state). During RAN2#122, an initial discussion took place and the following relevant agreements were made:

- Evaluate the control plane latency from RRC_INACTIVE to RRC_CONNECTED.

- Evaluate the control plane latency based on the 2-step RACH.

The CP procedure for the case agreed in RAN2#122 is illustrated in Figure 1. The CP latency as evaluated here encompasses steps 2 to 5. For step 1, the procedure for transition from a most “battery efficient” state has yet not begun, and hence this step is not relevant for the latency of the procedure. For step 10, the beginning of this subframe is considered to be "the start of continuous data transfer", hence this step is not relevant for the latency of the procedure, either.

Figure 1: Illustration of CP signalling during transition from RRC_INACTIVE to RRC_CONNECTED mode.

## Evaluation assumptions

Besides considering 2-step RACH, the most significant difference with respect to the terrestrial evaluation documented in TR 37.910 [2] is the addition of the satellite’s long propagation delay. As discussed during RAN2#122, the scenario assumption is the optimal case supported in Rel-17 NR NTN which the shortest round-trip time (RTT): a transparent LEO satellite at 600 km altitude with both UE and gateway at 90 degrees elevation angle and resulting in a one-way delay (BS-UE) of 4 ms.

In terms of sub-carrier spacing (SCS), the evaluation is for SCS of 15, 30, and 60 kHz. Note that a SCS of 120 kHz is not supported in NR NTN.

The evaluation presented here is for UL data transfer. It is understood that the evaluation results for DL data transfer gives a lower value because the UE processing delay in Step 5 does not need to handle the reception of the UL grant, and the total delay would be reduced compared to the case of UL data transfer.

In addition, the following assumptions apply to the evaluation:

- The transmission duration of steps 2 and 4 cannot be crossing the boundary of a slot.

- Because the latency will be dominated by the propagation delay, we focus on the (slightly slower) resource mapping type A, different from the terrestrial evaluation in TR 37.910.

A detailed description the remaining assumptions is collected in Table 1. As a clarification, the processing delays in steps 3 and 5 are assumed to be the same as in steps 7 and 9 in Table 5.7.2.1-1 of TR 37.910 [2].

Table 1: Detailed assumptions for CP latency

| Step | Description | CP latency for UL data transfer |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period (1TTI) | 0 |
| 2.1 | Transmission of MsgA (RACH Preamble + PUSCH) | Ts (the length of 1 slot / non-slot) |
| 2.2 | Propagation delay UE -> BS | 4 ms |
| 3 | Preamble detection and processing in gNB | 3 ms |
| 4.1 | Transmission of MsgB (RA response) | Ts (the length of 1 slot / non-slot) |
| 4.2 | Propagation delay BS -> UE | 4 ms |
| 5 | UE Processing Delay | 7 ms |
| 6 | Transmission of RRC Resume Complete and data | 0 |

## Results

The CP latency is the sum of the time durations of steps 2.1 to 5 presented in Table 1. The resulting values for different configurations are shown in Table 2. The ITU requirement of 40 ms can be fulfilled with all evaluated configurations.

Table 2: CP latency results [ms]

| Resource mapping type | Slot/non-slot duration | UE capability 1 & 2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 15kHz SCS | 30kHz SCS | 60kHz SCS |
| Type A | M =4 (4OS non-slot) | 14.57 | 14.29 | 14.14 |
|  | M =7 (7OS non-slot) | 15 | 14.5 | 14.25 |
|  | M =14 (14OS slot) | 16 | 15 | 14.5 |

NR NTN can achieve a CP latency of around 15 ms and thus fulfill the ITU requirement of 40 ms with all evaluated configurations.

# Conclusion

In section 2 we made the following observations:

- NR NTN can achieve a CP latency of around 15 ms and thus fulfill the ITU requirement of 40 ms with all evaluated configurations.

# References

ITU-R Report M.2514, Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020, 09/2022.

3GPP TR 37.910, Study on self evaluation towards IMT-2020 submission, Release 17

3GPP TS 38.214, Physical layer procedures for data, Release 15