3GPP TSG-RAN WG2 Meeting #123	R2-2309021

Toulouse, France, 21 – 25 August 2023

Title:	[DRAFT] Response LS on SHR and SPR

Response to:	R3-233380/ R2-2307030

Release:	Release 18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG3

Cc:

Contact Person:

#### Name:	György Wolfner

E-mail Address:	gyorgy<dot>Wolfner<at>nokia<dot>com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN3 for their LS on SHR and SPR. RAN2 discussed the issue, and provide the following answers to the questions:

Q1: Whether RAN2 sees any issues in defining a solution for “Configuration Information” as described above?

Q2: For SHR/SPR, is there any issue to include this “Configuration Information” in the RRC Reconfiguration message with sync containing Handover Command or PSCell change command?

Answer to Q1 and Q2:

RAN2 concluded that it is feasible to specify this type of “Configuration Information” solution from RAN2 specification perspective, however, concerns on the solution were raised by some companies.

RAN2’s understanding is that the new “Configuration Information” is provided for SHR/SPR whenever a new RRC configuration is sent to the UE. This creates extra overhead as it requires to include this “Configuration Information” in the RRCReconfiguration message, however, RAN2 has not investigated whether it imposes significant overhead in the RRCReconfiguration message containing Handover Command/PSCell change command or not.

Some companies have some general concerns on the solutions, commented that there is an existing solution based on “C-RNTI and time since event” that can leverage on existing reports to retrieve the configuration information (not the UE context), and not convinced that a new solution is needed given that the overhead impact in RRC messages should be limited to supported SON use cases. It was also noted that the “Configuration information” based solution may create backward compatibility issue if it is applied in a case where the “C-RNTI and time since event” approach is already used in Rel-17, but backward compatibility is not an issue in SPR and SHR cases.

Q3: In cases when this “Configuration Information” is not configured by the network to the UE, RAN3 discussed whether UE can include the source cell C-RNTI and the time between the event that triggered the report and the sending of the report to the network. RAN3 wants to check with RAN2 if it’s feasible in the above scenario?

Answer to Q3:

RAN2 concluded that the UE can include the source cell C-RNTI and the time between the event that triggered the report and the sending of the report to the network.

Some companies think if the “Configuration information” based solution is specified for a report, then adding this solution as an alternative is not desired, as it increases implementation and specification complexity. Other companies believe that this implies that the gNB may need to implement both CRNTI based solution (for legacy UEs) and configuration “Configuration information” based solution for Rel-18 UEs.

Q4: RAN3 kindly asks RAN2 to confirm RAN3’s understanding on CGI of the PCell and update their specifications if feasible, and also take the above information for indication on whether the PSCell change was MN-initiated or SN-initiated into account and provide feedback.

Answer to Q4:

RAN2 confirms RAN3’s understanding on CGI of the PCell reporting.

RAN2 has realized that in the case of legacy PSCell change the UE is not aware whether a PSCell change was MN-initiated or SN-initiated without an explicit indication received from the network. RAN2’s view is that in the other scenarios sending the indication is feasible without additional indication from the network.

It requires further discussion in RAN2 if the UE sends implicit or explicit indication.

2. Actions:

To RAN WG3

ACTION: 	RAN2 respectfully asks RAN3 to take the above into account.

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#123bis	from 2023-10-09	to 2023-10-13		Xiamen, CN

RAN2#124	from 2023-11-13	to 2023-11-17		Chicago, US