3GPP TSG-RAN WG2 Meeting #123bis                                                                                        R2-2309558

Xiamen, China, October 9th – 13th, 2023

Source:	CATT, CBN

Title:	Remaining UP Issues for Multicast reception in RRC_INACTIVE

Agenda Item:	7.11.2.2

Document for:	Discussion and Decision

1	Introduction

In RAN2#123 meeting, the following agreements have been reached on user plane for multicast reception in RRC_INACTIVE as in [1].

NW indicates which multicast service can be received in INACTIVE in suspendConfig of RRC Release. FFS how exactly this is indicated Unless blocking issues are identified, UE behaviour is not to suspend corresponding multicast MRBs and to keep using them in INACTIVE For “non-synchronised“ cell (in terms of PDCP COUNT), upon cell reselection, UE sets the initial PDCP count of the MRB for the multicast reception in RRC_INACTIVE state based on the same mechanism as R17 MBS broadcast. One cell can indicate "synchronized", if by implementation, it follows a common QoS flow to MRB mapping rule and at the same time PDCP COUNT is set according to the MBS QoS Flow SN. FFS how the UE is indicated about cells being synchronized (i.e. what information the NW needs to provide to the UE) Solutions which require COUNT broadcasting via MCCH are not considered SPS is not supported for multicast reception in RRC_INACTIVE. RAN2 enables RRC_INACTIVE UE receiving multicast to also receive possible PTM retransmissions initiated by UEs receiving multicast in RRC_CONNECTED. Allow configuration of drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM for INACTIVE UEs (38.331). UE receiving MBS multicast in RRC_INACTIVE should start drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM when reception of the transport block has not been successful. FFS the details, e.g. when the timers are started exactly. This is optional UE capability

In this contribution, we discuss the remaining issues. The contribution is organized as the following. In section 2, we discussed remaining issues on the following aspects,

- CFR configuration

- L2 operation during state transitions and mobility

- MAC operation

- RAN1 related impacts

Our proposals are summarized in Section 3.

2	Discussion

## 2.1 CFR configuration

For the relation between Multicast CFR in RRC_INACTIVE and broadcast CFR (i.e., whether one CFR is completely contained within the other), it was discussed previously in RAN2#121bies-e meeting but not concluded. A FFS is left in the agreements as in [1],

Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. FFS whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise).

Some one company think one of the two CFRs need to be fully contained (or overlapping) with the other CFR and it enables UE to just monitor the larger CFR and to receive both services without BWP switch.

However, there is no issue if one CFR is not completely contained within the other. First of all, BWP switching is not needed as anyway the two CFRs are partial overlapped due to that both the CFRs need to contain CORSET#0. UE only need to monitor a large region that covers both the two CFRs without BWP/CFR switching.

Then, even in case one CFR is completely contained within the other, UE cannot work only on one CFR (e.g., the larger CFR). UE needs to monitor PDCCH DCIs in separate CFRs for multicast and broadcast and to decode PDSCH in different CFRs according to the content of DCI. The reason is that downlink resource allocation for multicast/broadcast is calculated according to the related CFR, the details can be found in TS 38.214.

Therefore, it is proposed that,

Proposal 1: If multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, there is no need to restrict that one CFR is completely contained within the other.

## 2.2 L2 operation during state transitions and mobility

MRBs handling during state transition

In RAN2#123 meeting, there are some agreements on the continuation of the MRBs in CONNECTED during state transition from RRC_CONNECTED to RRC_INACTIVE as below,

NW indicates which multicast service can be received in INACTIVE in suspendConfig of RRC Release. FFS how exactly this is indicated Unless blocking issues are identified, UE behaviour is not to suspend corresponding multicast MRBs and to keep using them in INACTIVE

Regarding the network indication mentioned in above agreement, it is worth to clarify that it is used for UE to determine whether to suspend the MRBs of the active multicast session. A simple way is to indicate it on a per multicast session basis.

Therefore, it is proposed that,

Proposal 2: NW indicates whether the corresponding multicast MRBs configured in CONNECTED for an active multicast session can be continued in INACTIVE.

If P2 is agreed, then the following FFS in RRC running CR can be resolved,

FFS the details of MRB handling in case MRB in RRC_CONNECTED cannot be used in RRC_INACTIVE. …… FFS whether MRB ID needs to be configured.

That is, if an active multicast session is indicated to suspend the corresponding multicast MRBs of the multicast session upon transiting to RRC_INACTIVE, then UE suspends all the corresponding MRBs in CONNECTED and use the PTM configuration of the multicast session in RRCRelease or MCCH. With this, there is no need to include MRB ID in MCCH.

Therefore, it is proposed that,

Proposal 3: For an active multicast session is indicated to not continue the corresponding multicast MRBs configured in CONNECTED, UE suspends the corresponding MRBs and uses the PTM configuration of the multicast session in RRCRelease or MCCH.

Proposal 4: MRB ID is not needed for the PTM configuration on MCCH or RRCRelease message.

How the UE is indicated about cells being synchronized

It is a FFS how to indicate UE whether the cell is synchronized,

FFS how the UE is indicated about cells being synchronized (i.e. what information the NW needs to provide to the UE)

To address this issue, it is better to consider a simple solution considering the TU and timeline. e.g., we should avoid RAN3 impacts at this stage due to that it is an optimization and we are approaching feature frozen timeline.

Observation 1: To support PDCP continuity during mobility in INACTIVE, RAN3 impacts should be avoided.

In R17 MBS, the same QoS flows of one broadcast session can be mapped to different MRB ID in source cell and target cell. So this case should be considered if we consider mechanism for PDCP continuity during mobility in INACTIVE.

Observation 2: To support PDCP continuity during mobility in INACTIVE, we need to consider the case that MRB IDs for a multicast session in source cell and target cell are different.

In R17 MBS, derivation of the PDCP COUNT values by means of a DL MBS QFI Sequence Number is optionally supported. Based on this, The gNB can inform the UE that which QoS flow(s) of the multicast session maps to which MRB. If PDCP COUNT values are derived from a DL MBS QFI Sequence Number provided on NG-U. Then, based on the mapping relationship, the UE can determine whether one MRB is mapped from same QoS flows on the source and target cell. If it is, UE can treat this cell as synchronized.

Proposal 5: To determine whether cells are synchronized, the mapping between QoS flow(s) to the MRB can be provided by cells to UE.

## 2.3 MAC operation

The time point to start the drx-HARQ-RTT-TimerDL-PTM

In RAN2#123 meeting, it was agreed to configure PTM related HARQ RTT Timer and DRX Retransmission Timer for multicast in RRC_INACTIVE and the the timepoint to start the drx-HARQ-RTT-TimerDL-PTM is still FFS,

RAN2 enables RRC_INACTIVE UE receiving multicast to also receive possible PTM retransmissions initiated by UEs receiving multicast in RRC_CONNECTED. Allow configuration of drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM for INACTIVE UEs (38.331). UE receiving MBS multicast in RRC_INACTIVE should start drx-HARQ-RTT-TimerDL-PTM and drx-RetransmissionTimerDL-PTM when reception of the transport block has not been successful. FFS the details, e.g. when the timers are started exactly. This is optional UE capability

To determine the time point to start the drx-HARQ-RTT-TimerDL-PTM, one straightforward way may be the time when UE is supposed to transmit the HARQ feedback if it were enabled. UE can follow the legacy mechanism (i.e., according to PDSCH-to-HARQ_feedback timing indicator in DCI 4_1) to the time when UE is supposed to transmit the HARQ feedback if it were enabled.

Observation 3: The UE in RRC_INACTIVE state can follow the legacy mechanism (i.e., according to PDSCH-to-HARQ_feedback timing indicator in DCI 4_1) to determine the time when UE is supposed to transmit the HARQ feedback.

Proposal 6: The time point to start the drx-HARQ-RTT-TimerDL-PTM is the time when UE is supposed to transmit the HARQ feedback by assuming the HARQ feedback was enabled.

Whether DRX Command MAC CE is applicable for inactive multicast DRX operation

In MAC running CR [2], whether DRX Command MAC CE is applicable for inactive multicast DRX operation is kept as FFS,

Editor Note 1: FFS on whether DRX Command MAC CE is applicable for inactive multicast DRX operation.

In Rel-17 MBS, DRX Command MAC CE scheduled by G-RNTI can be used for multicast in CONNECTED. It seem feasible to reuse it for UEs which are receiving multicast in INACTIVE.

Proposal 7: DRX Command MAC CE addressed to G-RNTI can be used for multicast reception in RRC_INACTIVE.

Multicast MCCH RNTI

The value of the multicast-MCCH-RNTI is kept FFS in the MAC running CR [2],

Editor Note: FFS on the value of the multicast-MCCH-RNTI

For the value of the multicast-MCCH-RNTI, the first issue is whether it is a configurable or fixed. we do not see a clear motivation to make it configurable.it is workable to following the R17 broadcast way (i.e., using a fixed value for MCCH RNTI).To differentiate broadcast MCCH and multicast MCCH, a value which is different from the broadcast MCCH RNTI should be introduced.

Proposal 8: A fixed value (e.g., FFFB) is introduced as the value of the multicast-MCCH-RNTI.

The LCID of multicast MCCH

The value of LCID multicast MCCH is supposed to be defined in Table 6.2.1-1 but the value is still FFS, according to the open issue list for MBS [4],

Which value(e.g., 0 or new value) is defined for Which value(e.g., 0 or new value) is defined for the LCID of multicast MCCH in Table 6.2.1-1 in MAC spec in Table 6.2.1-1 in MAC spec

Since we will introduce a new RNTI for multicast MCCH and it is sufficient to use it to differentiate broadcast MCCH from multicast MCCH. So there is no need to define a new value for the LCID of multicast MCCH. That is, the LCID value (i.e., 0) for broadcast MCCH can be reused.

Proposal 9: Value 0 is used as the LCID of the multicast MCCH.

## 2.4 RAN1 related impacts

Issue on support DCI format 4-1 in beam sweeping way

In RAN2, we have assumed to reuse the same DCI format of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE, And agreed to support the multicast transmission RRC_INACTIVE in the beam sweeping way.

The multicast transmission RRC_INACTIVE is performed via beam sweeping based on SSB index like broadcast MBS (i.e. beam information is not need in DCI. For MTCH, RAN2 assumes to reuse the same DCI format of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for MCCH scheduling, DCI format 4-0 is used. We will ask RAN1 to confirm whether it is feasible and whether both 4-1 and 4-2 are needed.

Then in the RAN1 reply LS [5] it is confirmed DCI format 4_1 can be reused for multicast MTCH reception in RRC_INACTIVE.

- Question 1: Is the following RAN2 assumption feasible? If feasible, whether both DCI format 4-1 and DCI format 4-2 are needed? - For MTCH, RAN2 assumes to reuse the same DCI formats of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for multicast MCCH scheduling, DCI format 4-0 is used. Reply: From RAN1’s perspective, the above RAN2 assumption is feasible. For the detailed DCI formats, DCI format 4_0 can be reused for multicast MCCH reception in RRC_INACTIVE. DCI format 4_1 can be reused for multicast MTCH reception in RRC_INACTIVE. There is no consensus to support DCI format 4_2 for multicast MTCH reception in RRC INACTIVE in RAN1.

Based on above conclusion, one thing to clarify is that whether there is any additional impact to support DCI format 4-1 in beam sweeping way. Taking the DCI format 4_0 (which are also transmitted in beam sweeping) for reference, it seems what have been captured (i.e., using similar mechanism as broadcast MCCH/MTCH) in RRC running CR [3] is sufficient from RAN2 perspective. And whether there is a need to clarify something (e.g., a Type3-PDCCH CSS can support the Search Space used for multicast in RRC_INACTIVE for DCI formats with CRC scrambled by G-RNTI) in RAN1 spec can be up to RAN1.

Observation 4: There is no additional RAN2 spec impact to support DCI format 4-1 in beam sweeping way.

Observation 5: It is up to RAN1 on RAN1 spec impact to support DCI format 4-1 in beam sweeping way.

CSS for multicast in INACTIVE

In another reply LS from RAN1[6] ,RAN1 replies that network can configure the same or different search space ID for multicast MTCH in RRC_INACTIVE and multicast MTCH in RRC_CONNECTED.And further RAN1 indicate that RAN1 has not decided which CSS type(s) will be used for multicast MTCH.

- Question 3: Is it feasible to reuse the following Rel-17 CSS design for multicast MTCH and multicast MCCH? - 3.1) Reusing the same CSS or the same CSS type for multicast MTCH in RRC_INACTIVE (same as multicast MTCH in RRC_CONNECTED). Reply: For Question 3.1, from RAN1’s view, it is feasible to reuse the same CSS or the same CSS type for multicast MTCH in RRC_INACTIVE as the multicast MTCH in RRC_CONNECTED. It is up to the network to configure the same or different search space ID for multicast MTCH in RRC_INACTIVE and multicast MTCH in RRC_CONNECTED. RAN1 considers both Type 3-PDCCH CSS and Type 0/0B-PDCCH CSS can be used for multicast MTCH in RRC_INACTIVE. But both will require additional RAN1 work to enhance these search spaces to meet RAN2 relevant agreements. RAN1 has not decided which CSS type(s) will be used for multicast MTCH. RAN1 will continue discussing the detailed specification impacts regarding the CSS type.

Regardless whether the same or different search space ID for multicast MTCH in RRC_INACTIVE and multicast MTCH in RRC_CONNECTED, There should be a way for UE in RRC_INACTIVE to get the search space configuration multicast MCCH and MTCH from SIB/MCCH. And it seems which CSS type(s) is chosen by RAN1 has no RAN2 impacts.

Observation 6: RAN2 work has no dependency on which CSS type(s) is to chosen by RAN1 for multicast MCCH/MTCH in INACTIVE.

In Rel17 MBS, the search space for broadcast MCCH and MTCH are indicated in the IE PDCCH-ConfigCommon as below,

PDCCH-ConfigCommon ::= SEQUENCE { controlResourceSetZero ControlResourceSetZero OPTIONAL, -- Cond InitialBWP-Only commonControlResourceSet ControlResourceSet OPTIONAL, -- Need R searchSpaceZero SearchSpaceZero OPTIONAL, -- Cond InitialBWP-Only commonSearchSpaceList SEQUENCE (SIZE(1..4)) OF SearchSpace OPTIONAL, -- Need R …… searchSpaceMCCH-r17 SearchSpaceId OPTIONAL, -- Need R searchSpaceMTCH-r17 SearchSpaceId OPTIONAL, -- Need S

Similarly, it seems straightforward to also indicate the search space for multicast MCCH and MTCH are configured in PDCCH-ConfigCommon.

Proposal 10: MCCH/MTCH search space configuration for multicast in INACTIVE is to be defined as part of PDCCH-ConfigCommon.

3	Conclusion

Based on the discussions, we have the following observations and proposals for UP for Multicast reception in RRC_INACTIVE,

Observation 1: To support PDCP continuity during mobility in INACTIVE, RAN3 impacts should be avoided.

Observation 2: To support PDCP continuity during mobility in INACTIVE, we need to consider the case that MRB IDs for a multicast session in source cell and target cell are different.

Observation 3: The UE in RRC_INACTIVE state can follow the legacy mechanism (i.e., according to PDSCH-to-HARQ_feedback timing indicator in DCI 4_1) to determine the time when UE is supposed to transmit the HARQ feedback.

Observation 4: There is no additional RAN2 spec impact to support DCI format 4-1 in beam sweeping way.

Observation 5: It is up to RAN1 on RAN1 spec impact to support DCI format 4-1 in beam sweeping way.

Observation 6: RAN2 work has no dependency on which CSS type(s) is to chosen by RAN1 for multicast MCCH/MTCH in INACTIVE.

CFR configuration

Proposal 1: If multicast CFR in RRC_INACTIVE and broadcast CFR are configured differently, there is no need to restrict that one CFR is completely contained within the other.

L2 operation during state transitions and mobility

Proposal 2: NW indicates whether the corresponding multicast MRBs configured in CONNECTED for an active multicast session can be continued in INACTIVE.

Proposal 3: For an active multicast session is indicated to not continue the corresponding multicast MRBs configured in CONNECTED, UE suspends the corresponding MRBs and uses the PTM configuration of the multicast session in RRCRelease or MCCH.

Proposal 4: MRB ID is not needed for the PTM configuration on MCCH or RRCRelease message.

Proposal 5: To determine whether cells are synchronized, the mapping between QoS flow(s) to the MRB can be provided by cells to UE.

MAC operation

Proposal 6: The time point to start the drx-HARQ-RTT-TimerDL-PTM is the time when UE is supposed to transmit the HARQ feedback by assuming the HARQ feedback was enabled.

Proposal 7: DRX Command MAC CE addressed to G-RNTI can be used for multicast reception in RRC_INACTIVE.

Proposal 8: A fixed value (e.g., FFFB) is introduced as the value of the multicast-MCCH-RNTI.

Proposal 9: Value 0 is used as the LCID of the multicast MCCH.

RAN1 related impacts

Proposal 10: MCCH/MTCH search space configuration for multicast in INACTIVE is to be defined as part of PDCCH-ConfigCommon.

4	Reference

[1] Draft_R2_123_meeting_report_v1, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_123/Report

[2] R2-2309036, MAC running CR for eMBS, Apple

[3] R2-2309033, RRC running CR for eMBS, Huawei, HiSilicon

[4] R2-23xxxx, Open issue list for NR MBS enhancements

[5] R2-2307015/R1-2306243, Reply LS on multicast reception in RRC_INACTIVE

[6] R1-2308612, Reply LS on multicast reception in RRC_INACTIVE