3GPP TSG-RAN WG2 Meeting #123bis	R2-2309886

Xiamen, China, 9th – 13th October 2023

Agenda Item:	7.9.2

Source:	ASUSTeK

Title:	Remaining issues on PC5 radio link failure and PC5 link release

Document for:	Discussion and Decision

1	Introduction

In RAN2#119bis, the following agreement was made:

Relay reselection triggers include at least 1) Upper layer trigger; 2) PC5-RLF detection at the remote UE; 3) PC5-RLF indication received from the relay; 4) PC5 signal strength conditions; 5) PC5 link release message from relay to remote.

RAN2#121 further made the following agreement:

When the remote UE receives PC5-RLF indication from the U2U relay UE, it would inform upper layers and rely on upper layers to trigger relay reselection (or not).  FFS if there would be any constraints on the remote UE implementation behaviour to keep or release the PC5 link with the relay UE.

According to the above agreements, we have:

- the U2U Remote UE triggers relay reselection if a PC5-S link release message is received from the U2U Relay UE; and

- the U2U Relay UE notifies the U2U Remote UE if PC5-RLF is detected on the PC5 link with the peer U2U Remote UE.

These two agreements were captured in Stage 2 Running CR clause 16.12.4 [1] and RRC Running CR clause 5.8.9.10 [2] as quoted in Annex A and Annex B.

In this contribution, we discuss the remaining issues on U2U Relay UE’s behaviors in response to PC5 radio link failure and PC5 link release with a U2U Remote UE.

2	Discussion

2.1 	Content of NotificationMessageSidelink message

The NotificationMessageSidelink message sent by the U2U Relay UE to inform the U2U Remote UE of the PC5 RLF with the peer U2U Remote UE is specified in the RRC Running CR [2] as below:

NotificationMessageSidelink-v18xy-IEs ::=        SEQUENCE {

sl-IndicationType-r18                        ENUMERATED {relayUE-PC5-RLF. FFS}     OPTIONAL,  -- Need N

nonCriticalExtension                    SEQUENCE {}                        OPTIONAL

}

As shown above, the NotificationMessageSidelink message only includes a sl-IndicationType to indicate relayUE-PC5-RLF. Since a U2U Remote UE may connect or communicate with multiple peer U2U Remote UEs via one U2U Relay UE (according to clause 6.7.1.1 in TS 23.304 [3]), there is a need for the U2U Relay UE to include information in the NotificationMessageSidelink message for identifying the concerned peer U2U Remote UE with which the PC5 RLF is detected so that the U2U Remote UE can initiate relay reselection for the concerned peer U2U Remote UE. The information for identifying the concerned peer U2U Remote UE could be for further study (FFS).

- A U2U Remote UE may connect or communicate with multiple peer U2U Remote UEs via one U2U Relay UE and thus the U2U Relay UE needs to identify the peer U2U Remote UE when notifying the U2U Remote UE of the PC5 RLF with the peer U2U Remote UE.

- Upon PC5 RLF with a peer U2U Remote UE, the U2U Relay UE includes an ID of the peer U2U Remote UE in the NotificationMessageSidelink message sent to the U2U Remote UE. The ID of the peer U2U Remote UE is FFS.

2.2 	U2U Relay UE releases the PC5 link with the peer Remote UE

Clause 6.4.3.7.3 in TS 23.304 [3] specifies the following:

When the PC5 link between a 5G ProSe Layer-3 End UE and the 5G ProSe Layer-3 UE-to-UE Relay is released, the 5G ProSe Layer-3 UE-to-UE Relay may initiate the PC5 link release to the peer 5G ProSe Layer-3 End UE(s) or notify the peer 5G ProSe Layer-3 End UE(s) the peer PC5 link is released.

To align with what specified in TS 23.304 [3], we suggest the following proposal:

- Upon release of a PC5 link with a peer U2U Remote UE, the U2U Relay UE may notify the U2U Remote UE.

2.3 	U2U Relay UE detects PC5 RLF with the peer Remote UE

According to the U2U Relay UE behaviours upon PC5 link release specified in TS 23.304 [3] and the U2U Relay UE behaviours upon PC5 RLF specified in the RRC Running CR [2], we have the following summary:

| Events | PC5 link release with the peer Remote UE [3] | PC5 RLF with the peer Remote UE [2] |
| --- | --- | --- |
| Behaviours of U2U Relay UE | - Notify the Remote UE; or - Initiate PC5 link release with the Remote UE. | - Notify the Remote UE |

As we can see, there are two possible actions specified for PC5 link release in TS 23.304 [3] while there is only one action specified for PC5 RLF in the RRC Running CR [2].

According to clause 5.8.9.3 in TS 38.331 [4], the UE considers the PC5-RRC connection is released upon PC5 RLF is detected, which implies the consequence of PC5 RLF is equivalent to PC5 link release. This suggests the U2U Relay UE behaviour in response to PC5 RLF should be the same as the U2U Relay UE behaviour in response to PC5 link release.

- The PC5-RRC connection is released upon PC5 RLF is detected.

- The U2U Relay UE behaviour in response to PC5 RLF with a peer U2U Remote UE should be the same as the behaviour in response to PC5 link release with the peer U2U Remote UE.

In addition, clause 5.3.10.3 in TS 38.331 [4] specifies U2N Relay behaviour in response to Uu RLF as quoted below:

A L2/L3 U2N Relay UE shall:

1>	upon detecting radio link failure:

2>	either indicate to upper layers (to trigger PC5 unicast link release) or send NotificationMessageSidelink to the connected L2/L3 U2N Remote UE(s) in accordance with 5.8.9.10.

- Upon detecting Uu RLF, the U2N Relay UE shall either indicate to upper layers to trigger PC5 unicast link release or send NotificationMessageSidelink to the connected U2N Remote UE(s).

To align with the U2U Relay UE behaviour upon PC5 link release and the U2N Relay UE behaviour upon Uu RLF, we have the following proposal:

- Upon detecting PC5 RLF with the peer U2U Remote UE, the U2U Relay UE may either initiate PC5 unicast link release with the U2U Remote UE or send a notification to the U2U Remote UE.

3 	Conclusion

In this contribution, we observe and propose the following:

- A U2U Remote UE may connect or communicate with multiple peer U2U Remote UEs via one U2U Relay UE and thus the U2U Relay UE needs to identify the peer U2U Remote UE when notifying the U2U Remote UE of the PC5 RLF with the peer U2U Remote UE.

- The PC5-RRC connection is released upon PC5 RLF is detected.

- The U2U Relay UE behaviour in response to PC5 RLF with a peer U2U Remote UE should be the same as the behaviour in response to PC5 link release with the peer U2U Remote UE.

- Upon detecting Uu RLF, the U2N Relay UE shall either indicate to upper layers to trigger PC5 unicast link release or send NotificationMessageSidelink to the connected U2N Remote UE(s).

- Upon PC5 RLF with a peer U2U Remote UE, the U2U Relay UE includes an ID of the peer U2U Remote UE in the NotificationMessageSidelink message sent to the U2U Remote UE. The ID of the peer U2U Remote UE is FFS.

- Upon release of a PC5 link with a peer U2U Remote UE, the U2U Relay UE may notify the U2U Remote UE.

- Upon detecting PC5 RLF with the peer U2U Remote UE, the U2U Relay UE may either initiate PC5 unicast link release with the U2U Remote UE or send a notification to the U2U Remote UE.

4 	References

[1] R2-2309339, Introduction of NR sidelink relay enhancements (TS38.300 running CR).

[2] R2-2309308, Introduction of NR sidelink U2U relay (TS38.331 running CR).

[3] TS 23.304-i20, Proximity based Services (ProSe) in the 5G System (5GS) (Release 18)

[4] TS 38.331-h50, RRC protocol specification (Release 17)

5 	Annex A: Text captured in Stage 2 Running CR [1]

16.12.4	Relay Selection/Reselection

…

The U2U Remote UE triggers U2U Relay reselection in the following cases:

-	The PC5 signal strength of the current U2U Relay UE is below a (pre)configured signal strength threshold;

-	When U2U Remote UE detects PC5 RLF;

-	When U2U Remote UE receives the PC5 RLF indication from the U2U Relay UE;

-	When U2U Remote UE receives a PC5-S link release message from U2U Relay UE;

-	Indicated by the upper layer of the U2U Remote UE.

6 	Annex B: Text captured in Stage 3 Running CR [2]

#### 5.8.9.10	Notification Message

##### 5.8.9.10.1	General

Figure 5.8.9.8.1-1: Notification message in sidelink

This procedure is used by a U2N Relay UE to send notification to the connected U2N Remote UE. This procedure is also used by a U2U Relay UE to send notification to the connected U2U Remote UE.5.8.9.10.2	Initiation

The Relay UE may initiate the procedure when one of the following conditions is met:

1>	If the UE is acting as U2N Relay UE:

2>	upon Uu RLF as specified in 5.3.10;

2>	upon reception of an RRCReconfiguration including the reconfigurationWithSync;

2>	upon cell reselection;

2>	upon L2 U2N Relay UE's RRC connection failure including RRC connection reject as specified in 5.3.3.5 and 5.3.13.10, and T300 expiry as specified in 5.3.3.7, and RRC resume failure as specified in 5.3.13.5;

1>	If the UE is acting as U2U Relay UE:

2>		upon detection of PC5 RLF with U2U Remote UE as specified in 5.8.9.3;

##### Editor Note:	FFS the remote UE in previous agreement “When the remote UE receives PC5-RLF indication from the U2U relay UE, it would inform upper layers and rely on upper layers to trigger relay reselection (or not).” applies to both source and target remote UEs or not, applies to both L2 and L3 U2U relay or not.

##### 5.8.9.10.3	Actions related to transmission of NotificationMessageSidelink message

The Relay UE shall set the indication type as follows:

1>	If the UE is acting as U2N Relay UE;

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RLF:

3>	set the indicationType as relayUE-Uu-RLF;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to reconfiguration with sync:

3>	set the indicationType as relayUE-HO;

2>	else if the UE initiates transmission of the NotificationMessageSidelink message due to cell reselection:

3>	set the indicationType as relayUE-CellReselection;

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to Uu RRC connection establishment/Resume failure:

3>	set the indicationType as relayUE-Uu-RRC-Failure;

2>	submit the NotificationMessageSidelink message to lower layers for transmission;

1>	If the UE is acting as U2U Relay UE:

2>	if the UE initiates transmission of the NotificationMessageSidelink message due to PC5 RLF:

3>	set the sl-IndicationType as relayUE-PC5-RLF.

##### 5.8.9.10.4	Actions related to reception of NotificationMessageSidelink message

Upon receiving the NotificationMessageSidelink, the Remote UE shall:

1>	if the UE is acting as U2N Remote UE:

2>	if the indicationType is included:

3>	if the UE is L2 U2N Remote UE in RRC_CONNECTED:

4>	if T301 is not running, initiate the RRC connection re-establishment procedure as specified in 5.3.7;

3>	else (the UE is L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE):

4>	if the PC5-RRC connection with the U2N Relay UE is determined to be released:

5>	indicate upper layers to trigger PC5 unicast link release;

4>	else (i.e., maintain the PC5 RRC connection):

5>	if the UE is L2 U2N Remote UE and the indicationType is relayUE-HO or relayUE-CellReselection:

6>	consider cell re-selection occurs;

NOTE 1:	For L3 U2N Remote UE, or L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, it is up to Remote UE implementation whether to release or keep the PC5 unicast link.

NOTE 2:	The L2 U2N Remote UE may ignore the NotificationMessageSidelink if it does not release the PC5 unicast link in source side yet during an indirect-to-direct path switch, i.e. T304 is running.

1>	if the UE is acting as U2U Remote UE:

2>	if sl-IndicationType is relayUE-PC5-RLF:

3>	indicate upper layers to trigger reselection of NR sidelink U2U Relay UE;

Editor Note:	FFS if there would be any constraints on the Remote UE implementation behaviour to keep or release the PC5 link with the relay UE.