3GPP TSG-RAN WG2 Meeting #123bis                           	        R2-2310401

Xiamen, China, October 9th – 13th, 2023

Agenda Item:	7.16.2.3

Source: 	MediaTek Inc.

Title:  	Discussion on AI/ML Model Transfer/Delivery

Document for:	Discussion and decision

Introduction

In RAN2#121 meeting, RAN2 made following agreements for model transfer/delivery:

We Use the wording “model transfer/delivery”

model delivery that serves the use cases in the SI is within RAN2 scope, regardless other aspects.

Agreed:

- Aim to at least analyze the feasibility and benefits of model/transfer solutions based on the following:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 2a: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

- Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 2b: CN (except LMF) can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4: Server (e.g. OAM, OTT) can transfer/delivery AI/ML model(s) to UE (e.g. transparent to 3GPP).

In RAN2#123 meeting, RAN2 made following agreements for model transfer/delivery initiation:

Model transfer/delivery can be initiated in following two ways:

- Reactive model transfer/delivery: an AI/ML model is downloaded when it is needed due to changes in scenarios, configurations, or sites.

- FFS: Proactive model transfer/delivery: AI/ML models are pre-download to UE, and a model switch is performed when changes in scenarios, configurations, or sites occur.

In this contribution, we continue to discuss the tangible issues that need resolution and evaluate which options are better suited to address these issues.

Discussion

Proactive and Reactive Model Transfer/Delivery

Typically, model transfer/delivery is the process that facilitates the availability of an AI/ML model at the UE side. The model can be selected or activated for inference when it's applicable to specific scenarios, configurations, or sites that it's designed for. Model transfer/delivery is thus necessary when there's no existing AI/ML model at the UE applicable to the relevant scenario, configuration, or site. In such instances, the UE needs to download the AI/ML model trained for the specific scenario, configuration, or site.

In RAN2#123 meeting, RAN2 agreed that model transfer/delivery can be initiated in a reactive way, i.e., an AI/ML model is downloaded when it is needed due to changes in scenarios, configurations, or sites. The proactive model transfer/delivery is FFS. If UE is able to store multiple AI/ML models, UE can pre-download the models. A model switching is performed when changes in scenarios, configurations, or sites occur. This depicts a proactive method of model transfer/delivery. The proactive model transfer/delivery is a practical approach, particularly for collaboration level 'y'. In this scenario, model delivery from the OTT server to the UE may be realized through a software update, which does not have a latency requirement. For collaboration level 'z', it allows the UE more flexibility to convert or compile the model into a runtime image due to the relaxed latency requirement. Therefore, the proactive model transfer/delivery approach should also be supported.

Proposal 1: Proactive model transfer/delivery is supported as one approach to initiate model transfer/delivery, i.e., AI/ML models are pre-download to UE, and a model switch is performed when changes in scenarios, configurations, or sites occur.

In RAN1#114 meeting, RAN1 made following observations for model transfer/delivery:

Observation - Scenario/configuration specific (including site-specific configuration/channel conditions) models may provide performance benefits in some studied use cases (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites). - At least, when UE has limitation to store all related models, model delivery/transfer, if feasible, to UE may be beneficial, at the cost of overhead/latency associated with model delivery/transfer. - Note: On-device Finetuning/retraining, if feasible, of a single model may be an alternative to model delivery/transfer. - Note: a single model may generalize well in some studied use cases. - Note: Model transfer/delivery to UE may also face challenges, e.g., proprietary issues /burdens in some scenarios Observation - Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE.

Based on the observation, RAN1 is also considering the case that a single model cannot generalize well to multiple scenarios/configurations/sites and UE has limitation to store all related models. In this case, UE downloads the model only when necessary - a reactive approach to model transfer/delivery. Just as the observation made by RAN1, the model transfer/delivery is beneficial at the cost of overhead/latency associated with model transfer/delivery.

In the previous RAN2 meeting, some companies think that even if model transfer/delivery is initiated reactively due to changes in configuration, conditions, or site, there is no latency requirement for model transfer/delivery. This is because the UE can always be fallback to non-AI operation. But in our understanding, we categorize this case as proactive model transfer/delivery because it does not necessitate the immediate use of the newly downloaded AI/ML model.

The purpose of reactive model transfer/delivery is to immediately utilize the newly downloaded AI/ML model adaptive to the changes in configuration, conditions, or site. Therefore, for reactive model transfer/delivery, the latency requirement for model transfer/delivery should be taken into account. This process should be swift enough to adapt to changes in scenarios, configurations, or sites. If not, the AI/ML model may become outdated by the time it is available at the UE. For instance, in the extreme case where the AI/ML model is cell-specific, reactive model transfer/delivery does have a latency requirement when the UE moves among different cells.

Proposal 2: RAN2 assumes that the objective of reactive model transfer/delivery is to promptly utilize the newly downloaded model for the applicable configuration, condition, or site. Consequently, there should be a latency requirement for reactive model transfer/delivery.

For case y, the model is transferred from the OTT server to the UE. Concerning proactive model delivery, the exact method used for triggering the model delivery procedure falls outside of RAN2's scope. For instance, the OTT server may initiate the model delivery process when certain events occur, such as the availability of a new or updated model on the OTT server that needs to be downloaded by the UE. However, in the case of reactive model delivery, the challenge lies in ensuring that the OTT server is aware of the associated radio access network configuration, scenario, and site, and is able to deliver the relevant model to the UE in a timely manner.

In case z, the model resides within the 3GPP network. Regardless of whether the model is trained on the UE side or the network side, the network must be knowledgeable about the model's meta information, including the applicable configuration, scenario, and site. As such, it is fundamental to consider network-triggered model transfer as the baseline. Once the model is available, a switch to the new model occurs in response to changes in the configuration, scenario, or site.

Proposal 3: RAN2 assumes that when the AI/ML model is stored within the network, the network is informed of the model's meta information, including the relevant configuration, scenario, and site.

Proposal 4: RAN2 assumes that model transfer is triggered by the network and should be executed prior to the selection, activation, deactivation, or switching of the model.

Model Transfer/Delivery Latency

If we assume that the size of the model is substantial, we must take into account the total latency from the initiation of model transfer/delivery to the point where the AI/ML model is executable at the UE. This consideration is particularly crucial for the reactive model transfer/delivery approach, just as explained in proposal 2. This is due to the need for minimizing the duration from the onset of model transfer/delivery to the moment when the AI/ML model is ready for execution at the UE. Ensuring promote model transfer/delivery is one important aspect to ensure adaptability regarding any changes to the scenario, configuration, and sites.

The total latency may consider end-to-end (E2E) latency, transport latency, and UE processing latency for the AI/ML model. The transport latency is contingent on factors such as model size, number of RRC messages required (CP-based solution) or the data rate (UP-based solution). The processing latency at the UE for the AI/ML model is specific to UE implementation, depending also on the format in which the model is delivered, and the necessary steps (for instance, convert, compile) to make the model executable.

For existing model transfer/delivery options, we provide an initial analysis in Table 1, assuming the latency between gNB and AMF/UPF is 0.5ms and the latency between AMF and LMF is 0.5ms.

Table 1 Comparison of different model delivery/transfer solutions

| Solution | Terminated entity | Allowed payload size | E2E latency (one way) | Transport latency | Estimated overall latency |
| --- | --- | --- | --- | --- | --- |
| Solution 1a | gNB | 45Kbytes or 144kbyte (with16 segments) | 1ms | 5*10; or 16+15*10ms | <200ms |
| Solution 1b | gNB | No limitation | 1ms; or 2ms | Model size/ data rate | TBD |
| Solution 2a | CN | 45Kbytes or 144kbyte (with16 segments) | 1.5ms | Similar as solution 1a | <200ms |
| Solution 2b | CN | No limitation | 1.5ms | Model size/ data rate | TBD |
| Solution 3a | LMF | 45Kbytes | 2ms | 5*10ms | <200ms |
| Solution 3b | LMF | No limitation | 2ms | Model size/ data rate | TBD |
| Solution 4 | OTT server | No limitation | 1s [1] | Model size/ data rate | TBD |

Proposal 5: RAN2 takes table 1 as starting point to evaluate the different model transfer/delivery solutions.

With reference to the latency requirement for AI/ML application model download in Solution 4 [1], the end-to-end (E2E) latency accounts for 1 second, forming a dominant influence on the overall model delivery latency. In conjunction with the transport latency and the User Equipment (UE) processing latency, the total latency is estimated to reach 2 seconds. If the model delivery is carried out reactively, the UE may not obtain the AI/ML model promptly upon alterations in the scenarios, configurations, or sites, causing potential issues. Therefore, Solution 4 proves effective primarily with proactive model delivery. The specifics on how the UE and the server converse to determine the initiation of the model delivery procedure are implementation-oriented and might be outside the 3GPP scope. But, if model monitoring and control occur at the network, anytime the UE downloads new or updated models from the server, it ought to update the model information with the network.

For alternatives to Solution 4, we anticipate the latency for model transfer to be significantly less. The dominant factor in the total latency of model transfer should be transport latency, which depends on the size of the model and the data rate. Assuming the model size is 30 Mbits and the data rate is 100 Mbps, the transport latency would be around 300 ms. Model transfer latency from RAN nodes is generally less than from CN nodes. UP-based model transfer solutions typically offer shorter latencies than CP-based solutions. We expect the total model transfer latency for options 1a/1b, 2a/2b, and 3a/3b to be in the range of hundreds of milliseconds.

To fully understand the overall latency, we require RAN1's input regarding the typical latency requirements (either specific values or a range) for the reactive model transfer/delivery approach. Additionally, awareness of the model size can enable RAN2 to calculate an estimation of transmission latency. This understanding becomes a primary factor in choosing between CP-based solutions and UP-based solutions. Moreover, we need RAN1 to provide typical model sizes (either specific values or a range), facilitating an evaluation of the suitability of different model transfer/delivery solutions.

Proposal 6: Ask RAN1 to provide input on the typical latency requirement (value or value range) to transfer an AI/ML model in reactive manner. Additionally, RAN1 should provide input on the typical model size (value or value range), allowing RAN2 to assess the applicability of different model transfer/delivery solutions.

Proposal 7: RAN2 assumes that there is no latency requirement on proactive model transfer/delivery.

For CP-based solutions - that is, solutions 1a, 2a, 3a - transferring large-sized models is impractical due to the immense signalling overhead and extended processing latency. CP-based solutions are apt for transferring models with small sizes, such as less than 45 Kbytes or 144 Kbytes. If the model size exceeds the capacity permitted by the CP-based solution, it's advisable to use a User Plane (UP)-based solution.

Proposal 8: RAN2 assumes CP-based solutions are better suited for transferring models of small size (less than 45Kbytes or 144Kbytes), whereas UP-based solutions are more effective for large-size model transfers.

Conclusion

Proposal 1: Proactive model transfer/delivery is supported as one approach to initiate model transfer/delivery, i.e., AI/ML models are pre-download to UE, and a model switch is performed when changes in scenarios, configurations, or sites occur.

Proposal 2: RAN2 assumes that the objective of reactive model transfer/delivery is to promptly utilize the newly downloaded model for the applicable configuration, condition, or site. Consequently, there should be a latency requirement for reactive model transfer/delivery.

Proposal 3: RAN2 assumes that when the AI/ML model is stored within the network, the network is informed of the model's meta information, including the relevant configuration, scenario, and site.

Proposal 4: RAN2 assumes that model transfer is triggered by the network and should be executed prior to the selection, activation, deactivation, or switching of the model.

Proposal 5: RAN2 takes table 1 as starting point to evaluate the different model transfer/delivery solutions.

Proposal 6: Ask RAN1 to provide input on the typical latency requirement (value or value range) to transfer an AI/ML model in reactive manner. Additionally, RAN1 should provide input on the typical model size (value or value range), allowing RAN2 to assess the applicability of different model transfer/delivery solutions.

Proposal 7: RAN2 assumes that there is no latency requirement on proactive model transfer/delivery.

Proposal 8: RAN2 assumes CP-based solutions are better suited for transferring models of small size (less than 45Kbytes or 144Kbytes), whereas UP-based solutions are more effective for large-size model transfers.

Reference

[1] 3GPP TR 22.874 V18.2.0, Study on traffic characteristics and performance requirements for AI/ML model transfer in 5GS,