3GPP TSG-RAN WG2 Meeting #123-bis		R2-2311119

Xiamen, China: 9th October – 13th October 2023

Source:	Qualcomm Incorporated

Title:	Open issues in functionality to physical entities mapping

Agenda Item:	7.16.2.1 Architecture and General

Document for:	Discussion and Decision

# Introduction & Background

In the RAN2#123 meeting [1], RAN2 discussed the mapping of functions to physical entities based on post RAN2#122 email discussion [Post122][060][AIML] Mapping of functions to physical entities [2].

In this contribution, we discuss FFS on mapping of functions to physical entities.

# Mapping of functionality to entities

Based on post RAN2#122 email discussion [Post122][060][AIML] Mapping of functions to physical entities [2], in RAN2#123 meeting, RAN2 made the following agreements:

P1-P6 are agreed, it is expected that FFS items for which support is not increased will be removed.

In RAN1#112 meeting, RAN1 made the following agreement

-------------------------------

Agreement

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.

Note: The Case definition is NOT intended to introduce sub-levels of Level z.

Note: Other cases may be included further upon interest from companies.

FFS: Z4 and Z5 boundary

-------------------------------

## 2.1 	Model training at the UE-side and neutral side

From the above RAN1#112 meeting agreement, RAN1 agreed that model training can be performed on the UE-side or neutral side. However, RAN2 has incorrectly assumed that the UE-side or neutral-side training can only be performed at the OTT server. UE-side or neutral-side training can be performed at a server within the 3GPP network or outside the 3GPP network. Furthermore, in the LS reply R1-2308730, the RAN1 note states that different possible network entities where the training can be performed can be gNB, CN, LMF, or OAM. RAN1 did not confirm these network entities for training, stating it is outside RAN1's expertise. Note that it is outside RAN2's expertise to confirm if CN can be an appropriate entity for performing training. Therefore, servers within or outside the 3GPP network can be possible entities for performing training. RAN2 should leave the decision of whether the model training can be performed at the server within or outside the 3GPP network to the SA working groups.

Observation 1: In the RAN1#112 meeting agreement, RAN1 agreed that model training can be performed on the UE-side or neutral side.

Observation 2: RAN2 has incorrectly assumed that the UE-side or neutral-side training can only be performed at the OTT server (without 3GPP signaling impact).

Observation 3: the UE-side or neutral-side training can be performed at

- A server within the 3GPP network, and/or

- A server outside the 3GPP network (e.g., OTT server).

Observation 4: in the LS reply R1-2308730, the RAN1 note states that different possible network entities where the training can be performed can be gNB, CN, LMF, or OAM. RAN1 did not confirm these network entities for training, stating it is outside RAN1's expertise.

Observation 5: Confirming whether the UE-side or neutral-side training can or cannot be performed at a server within the 3GPP network is outside RAN2 expertise. RAN2 leaves the decision of whether the UE-side or neutral-side training can be performed at the server within the 3GPP network to the SA WGs.

## 2.2 	AI/ML model transfer/delivery methods

Similar to the discussions above for the UE-side or neutral-side training at a server within or outside the 3GPP network, it is outside the RAN2 expertise to confirm whether a server within the 3GPP network can or cannot be appropriate for model delivery/transfer to the UE. Therefore, RAN should leave the decision of whether the model delivery/transfer can be performed from a server within the 3GPP network to the UE to the SA working groups.

Observation 6: Confirming whether a server within the 3GPP network can or cannot be used for model delivery/transfer to the UE is outside RAN2 expertise. RAN leaves the decision of whether the model delivery/transfer can be performed from a server within the 3GPP network to the UE to the SA WGs.

## 2.3 	RAN2 agreements on mapping of functions to the physical entities

Considering the above discussion that model training can be performed at a server within or outside the 3GPP network and the model can be transferred/delivered from a sever within or outside the 3GPP network, we propose to modify the UE-side OTT server to the UE-side server. Furthermore, we propose to modify note 4 to whether/how the UE-side training can be performed at the server within or outside the 3GPP network is to be involved may need to consult SA2.

Proposal 1: For proposals P1, P2, and P4 in R2-2308286 modify the “OTT server” and “UE-side OTT server” to “UE-side Server”.

Proposal 2: Modify note 4 in proposals P1, P2, and P4 in R2-2308286 to “whether/how a UE-side Server within or outside the 3GPP network is to be involved may need to consult RAN3, SA2.”

# Conclusion

Observation 1: In the RAN1#112 meeting agreement, RAN1 agreed that model training can be performed on the UE-side or neutral side.

Observation 2: RAN2 has incorrectly assumed that the UE-side or neutral-side training can only be performed at the OTT server (without 3GPP signaling impact).

Observation 3: the UE-side or neutral-side training can be performed at

- A server within the 3GPP network, and/or

- A server outside the 3GPP network (e.g., OTT server).

Observation 4: in the LS reply R1-2308730, the RAN1 note states that different possible network entities where the training can be performed can be gNB, CN, LMF, or OAM. RAN1 did not confirm these network entities for training, stating it is outside RAN1's expertise.

Observation 5: Confirming whether the UE-side or neutral-side training can or cannot be performed at a server within the 3GPP network is outside RAN2 expertise. RAN2 leaves the decision of whether the UE-side or neutral-side training can be performed at the server within the 3GPP network to the SA WGs.

Observation 6: Confirming whether a server within the 3GPP network can or cannot be used for model delivery/transfer to the UE is outside RAN2 expertise. RAN leaves the decision of whether the model delivery/transfer can be performed from a server within the 3GPP network to the UE to the SA WGs.

Proposal 1: For proposals P1, P2, and P4 in R2-2308286 modify the “OTT server” and “UE-side OTT server” to “UE-side Server”.

Proposal 2: Modify note 4 in proposals P1, P2, and P4 in R2-2308286 to “whether/how a UE-side Server within or outside the 3GPP network is to be involved may need to consult RAN3, SA2.”

# Reference

- RAN2#123 meeting report

- R2-2308286, [Post122][060][AIML] Mapping of functions to physical entities

# Appendix: R2-2308286 Proposals for Reference

Proposals for agreements:

For CSI feedback enhancement:

Proposal 1: The Table 1 can be used as starting point for discussion on mapping of AI/ML functions to physical entities for CSI compression with two-sided model.

Table 1: The mapping of functions to physical entities for CSI compression with two-sided model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training(offline training) | gNB, OAM, OTT server, UE, [FFS: CN] |
| b) | Model transfer/delivery | For training Type 1: gNB->UE, or OAM->gNB&UE, or OTT server->gNB&UE, or UE->gNB, [FFS: CN->gNB&UE] For training Type 3: - For UE part of two-sided model: OTT server->UE, [FFS: CN->UE]; - For NW part of two-sided model: OAM->gNB, [FFS: CN->gNB]; |
| c) | Inference | NW part of two-sided model: gNB UE part of two-sided model: UE |
| d) | Model/functionality monitoring | NW-side: NW monitors the performance UE-side: UE monitors the performance and may report to NW |
| e) | Model/functionality control (selection, (de)activation, switching, updating, fallback) | gNB, [FFS: UE] |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how CN is to be involved may need to consult RAN3, SA2.

For beam management:

Proposal 2: The Table 2 can be used as starting point for discussion on mapping of AI/ML functions to physical entities for beam management with UE-side model.

Table 2: The mapping of AI/ML functions to physical entities for beam management with UE-side model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training(offline training) | UE-side OTT server, UE, [FFS: gNB, OAM, CN] |
| b) | Model transfer/delivery | UE-side OTT server->UE, [FFS: gNB->UE, or OAM->UE, or CN->UE] |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE (UE monitors the performance, and may report to gNB), gNB (gNB monitors the performance) |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB if monitoring resides at UE or gNB, UE if monitoring resides at UE |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how CN is to be involved may need to consult RAN3, SA2.

Proposal 3: The Table 3 can be used as starting point for discussion on mapping of AI/ML functions to physical entities for beam management with NW-side model.

Table 3: The mapping of functions to physical entities for beam management with NW-side model

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | gNB, OAM, [FFS: CN, OTT server] |
| b) | Model transfer/delivery | OAM->gNB, [FFS: CN->gNB, OTT server->gNB] |
| c) | Inference | gNB |
| d) | Model/functionality monitoring | gNB |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how CN is to be involved may need to consult RAN3, SA2.

For Positioning accuracy enhancement:

Proposal 4: The Table 4 can be used as starting point for discussion on mapping of AI/ML functions to physical entities for positioning with UE-side model (case 1 and 2a).

Table 4: The mapping of functions to physical entities for positioning with UE-side model (case 1 and 2a)

| Use case | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | UE-side OTT server, UE, [FFS: LMF, OAM, CN] |
| b) | Model transfer/delivery | UE-side OTT server->UE, [FFS: LMF->UE, OAM->UE, CN->UE] |
| c) | Inference | UE |
| d) | Model/functionality monitoring | UE, LMF |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | UE if monitoring resides at UE, LMF if monitoring resides at UE or LMF |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how CN/LMF is to be involved may need to consult RAN3, SA2.

Proposal 5: The Table 5 can be used as starting point for discussion on mapping of AI/ML functions to physical entities for positioning with LMF-side model (case 2b and 3b).

Table 5: The mapping of functions to entities for positioning with LMF-side model (case 2b and 3b)

|  | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | LMF |
| b) | Model transfer/delivery | N/A |
| c) | Inference | LMF |
| d) | Model/functionality monitoring | LMF |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | LMF |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: Whether/how LMF is to be involved may need to consult RAN3, SA2.

Proposal 6: The Table 6 can be used as starting point for discussion on mapping of AI/ML functions to physical entities for positioning with gNB-side model (case 3a).

Table 6: The mapping of AI/ML functions to entities for positioning with gNB-side model (case 3a)

| Use case | AL/ML functions (if applicable) | Mapped entities |
| --- | --- | --- |
| a) | Model training (offline training) | gNB, OAM, [FFS: LMF] |
| b) | Model transfer/delivery | OAM->gNB, [FFS: LMF->gNB] |
| c) | Inference | gNB |
| d) | Model/functionality monitoring | gNB, [FFS: LMF] |
| e) | Model/functionality control (selection, (de)activation, switching, fallback) | gNB, [FFS: LMF] |

Note 1: For a), only data collection part may be further discussed, how to perform the model training is up to implementation.

Note 2: For b), no model transfer/delivery is expected if the entity for model training and model inference is the same one.

Note 3: Whether/how OAM is to be involved may need to consult RAN3, SA5.

Note 4: Whether/how LMF is to be involved may need to consult RAN3, SA2.