3GPP TSG-RAN WG2 #124								R2-2311797

Chicago, USA, November 2023

Agenda Item:	5.1.3.2

Source:	OPPO

Title:	Left issues on UE MRTD/MTTD performance requirement in ENDC and NEDC

Document for:	Discussion, Decision

Introduction

This is to discuss the left issue from R4-2317401 and R4-2317402.

Discussion

Given the RP request to introduce a BS signaling, for TDD-TDD case of inter-band ENDC scenario, to differentiate type-1/2 UE behavior, and now the extension of intra-band ENDC async capability flag to inter-band case and NE-DC case, it seems helpful to summarize all related cases together to have an overview.

The information comes from TS 38.133, and latest CR of R4-2317400 (together with the incoming LSs).

Table 1 UE capability and performance requirement for intra-/inter-band EN/NE-DC and for different duplex combinations

|  |  | EN-DC |  |  |  |  | NE-DC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Question | How to diff 1/3 |  | How to diff 1/2/3 | How to diff 1/2 |  | How to diff 1/2/3 | How to diff 1/2 | How to diff 2/3 |
|  |  | Intra-band, FDD-FDD | Intra-band TDD-TDD | Inter-band FDD-FDD with overlapping frequency | Inter-band FDD-TDD, TDD-FDD, FDD-FDD, TDD-TDD, without overlapping frequency | Inter-band TDD-TDD with overlapping frequency | Inter-band FDD-FDD, with overlapping frequency | FDD-TDD, TDD-FDD, FDD-FDD, TDD-TDD, without overlapping frequency | Inter-band TDD-TDD, with overlapping frequency |
| 1 | Async MTTD: Table 7.5.2-1 Table 7.5.5-1 MRTD Table 7.6.2-1 Table 7.6.5-1 | Capability-based Support | Not support | Capability-based Support | Mandatory support | Not support | Capability-based Support | Mandatory support | Not support |
| 2 | Sync, Type-2 MTTD: Table 7.5.2.1-1 Table 7.5.5.1-1 MRTD: Table 7.6.2.1-1 Table 7.6.5.1-1 | Not Support | Not Support | Capability-based Support | Mandatory support | Capability-based Support | Capability-based Support | Mandatory support | Capability-based Support |
| 3 | Sync, Type-1 MTTD: Table 7.5.3-1 MRTD: Table 7.6.3-1 | Mandatory Support | Mandatory Support |  | Not support |  |  | Not Support |  |

Issue-1: How to extend asyncIntraBandENDC

R4 requested that

The signaling “asyncIntraBandENDC” was introduced for intra-band EN-DC, while it is further extended to cover inter-band EN-DC with overlapping frequency.

[…]

Further, the signaling “asyncIntraBandENDC” now only cover EN-DC UE reporting. RAN4 would like to ask RAN2 to update this signaling description to cover NE-DC reporting.

Currently, asyncIntraBandENDC is for FDD-FDD only, while

asyncIntraBandENDC

Indicates whether the UE supports asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. If asynchronous FDD-FDD intra-band (NG)EN-DC is not supported, the UE supports only synchronous FDD-FDD intra-band (NG)EN-DC.

This capability applies to:

[…]

-	Inter-band (NG)EN-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]).

While the interBandMRDC-WithOverlapDL-Bands-r16 covers both FDD-FDD and TDD-TDD

interBandMRDC-WithOverlapDL-Bands-r16

Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE).

So the firstly since inter-band ENDC, FDD-FDD with overlapping has been somehowcovered for the case that

“where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band”,

but did not mention the case of partial overlapping, e.g., for DC_20A_n28A (included in Table 5.5B.4.1-1 of TS 38.101-3, where the DL of band 20 and n28 is partially overlapping during 791MHz – 803MHz range), yet considering the NOTE that

NOTE 8:	The frequency range in band n28 / 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. This restriction also apply for any band combinations when DC_20_n28/ DC_28_n20/ CA_20-28/ CA_n20-n28 is a subset of a higher order band combination.

It seems not clear whether there is a need to consider this case as “FDD-FDD with overlapping range” (i.e., whether the overlapping is defined in frequency range of band-level or CC-level), and even it is not clear whether the legacy capability bit has already covered that case (if valid).

- For R4-2317402, before capturing in R2 CR, R2 discuss to cover the case of partial overlapping, e.g., DC_20A_n28A for ENDC.

For R4-2317402, before capturing in R2 CR, R2 confirm what R4 requested for EN-DCis to additionally cover inter-band ENDC, FDD-FDD with “partial” overlapping frequency.

Secondly, for NE-DC, the left issue is

- whether the intention is to extend the capability to both TDD-TDD and FDD-FDD, i.e., the cases where overlapping may happen.

- whether the intention is to extend the capability to NEDC for inter-band case only, or intra-band case as well.

- For R4-2317402, before capturing in R2 CR, R2 confirm what R4 requested for NE-DC is to additionally cover  inter-band NEDC, FDD-FDD with overlapping frequency.

Another issue is whether to use separate IE to do the extension, for NE-DC-only BC list, there is no concern, since this IE is per-BC IE, but then for the BC which is capable of both EN-DC and NE-DC, separate IE may be motivated if the UE can support one but not the other. However, it seems not very possible given it is a R4 capability.

- For R4-2317402, before capturing in R2 CR, R2 discuss whether to use a separate capability bit to do the extension for NEDC.

Issue-2: How for UE to decide which performance requirement to follow

- Only for intra-band, TDD-TDD, EN-DC, UE just needs to support a single performance request (synchronous, type-1). Otherwise, for other scenarios, UE has to support more than one performance requirements.

- For scenarios of 1) intra-band, EN-DC, FDD-FDD, 2) inter-band, EN-DC, FDD-FDD/FDD-TDD/TDD-FDD and TDD-TDD without overlapping, and 3) inter-band, NE-DC, FDD-FDD/FDD-TDD/TDD-FDD and TDD without overlapping, it is not clear how for UE to select between asynchronous and synchronous requirements.

Based on RP LS

For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands

This new BS signalling is expected to be applicable for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands provided the UE has indicated “interBandMRDC-WithOverlapDL-Bands-r16”.

- Only for inter-band, EN-DC, TDD-TDD with overlapping frequency, a BS signaling is introduced to guide UE selection between two performance requirements (synchronous, type-1 vs. type-2).

- For scenarios of 1) inter-band, EN-DC, FDD-FDD with overlapping, and 2) inter-band, NE-DC, FDD-FDD/TDD-TDD with overlapping, it is not clear how for UE to select between synchronous type-1 and synchronous type-2 requirements.

Given R2 is to introduce signaling for inter-band, EN-DC, TDD-TDD with overlapping case, it would helpful to clarify companies understanding on the similar issue for other deployment scenarios.

- For R4-2317401, before capturing in R2 CR, R2 discuss for Rel-18 scenarios of 1) Inter-band, ENDC, FDD-FDD with overlapping frequency, 2) Inter-band, NEDC, FDD-FDD with overlapping frequency, and 3) Inter-band, NEDC, TDD-TDD with overlapping frequency, R2 discuss how for UE to select to follow a) synchronous type-2, or b) synchronous type-1 requirement, e.g., whether can extend the newly introduced BS signaling to FDD-FDD with overlapping, and to NEDC case as well.

And it may be helpful to clarify that UE how for UE to decide whether to apply the asynchronous requirement. E.g., when the new BS siganling is absent, whether it means collocated scenario only, or it could be asynchronous scenario as well, and it relies on UE itself to judge.

Since the incoming LS mainly concerns

- Inter-band, ENDC, FDD-FDD with overlapping frequency

- Inter-band, NEDC, FDD-FDD with overlapping frequency

- Inter-band, NEDC, TDD-TDD with overlapping frequency

It might be easier to start from these 3 cases, and maybe easier to clarify it for Rel-18.

- For R4-2317401, before capturing in R2 CR, R2 discuss for Rel-18 scenarios of 1) Inter-band, ENDC, FDD-FDD with overlapping frequency, and 2) Inter-band, NEDC, FDD-FDD with overlapping frequency, how for UE to select to follow asynchronous requirement or not.

Conclusion

We have the following proposals:

Proposal 1	For R4-2317402, before capturing in R2 CR, R2 discuss to cover the case of partial overlapping, e.g., DC_20A_n28A for ENDC.

Proposal 2	For R4-2317402, before capturing in R2 CR, R2 confirm what R4 requested for NE-DC is to additionally cover  inter-band NEDC, FDD-FDD with overlapping frequency.

Proposal 3	For R4-2317402, before capturing in R2 CR, R2 discuss whether to use a separate capability bit to do the extension for NEDC.

Proposal 4	For R4-2317401, before capturing in R2 CR, R2 discuss for Rel-18 scenarios of 1) Inter-band, ENDC, FDD-FDD with overlapping frequency, 2) Inter-band, NEDC, FDD-FDD with overlapping frequency, and 3) Inter-band, NEDC, TDD-TDD with overlapping frequency, R2 discuss how for UE to select to follow a) synchronous type-2, or b) synchronous type-1 requirement, e.g., whether can extend the newly introduced BS signaling to FDD-FDD with overlapping, and to NEDC case as well.

Proposal 5	For R4-2317401, before capturing in R2 CR, R2 discuss for Rel-18 scenarios of 1) Inter-band, ENDC, FDD-FDD with overlapping frequency, and 2) Inter-band, NEDC, FDD-FDD with overlapping frequency, how for UE to select to follow asynchronous requirement or not.