3GPP TSG-RAN WG2 Meeting #124		 R2-2312571

Chicago, USA, 13th Nov. – 17th Nov. 2023

Source:	vivo (Rapporteur)

Title:	TP for TR conclusion on high layer aspects

Agenda Item:	7.22.1

Document for:	Discussion and Decision

- Introduction

In this contribution, Rapporteur would like to share some views on the conclusions for the study on the LP-WUS/WUR SI in [1].

- Discussion

  - General

According to the study and agreements in RAN2 meeting from #121bis, rapporteur suggests to capture the below conclusion in the TR:

---------------------------------------------------------------------Start of TP-------------------------------------------------------------

In conclusion of the study, the following high layer aspects are recommended: For RRC_IDLE/INACTIVE mode - RAN2 has studied the procedure and concluded the feasibility for LP-WUS to control paging monitoring. Details are to be decided in WI RAN2 has studied and concluded the subgrouping methods for LP-WUS, including CN assigned and/or UE_ID based subgrouping, which depends on the payload of LP-WUS. - RAN2 has studied the entry/exit condition of using LP-WUS, and concluded the condition(s) is based on the measurement on LR and/or MR for at least serving cell quality. - RAN2 has studied and concluded the feasibility for MR RRM measurement relaxation (including no measurement) for serving cell and neighboring cell. Details are to be decided in WI. - TBD: whether/how to mention SI maintenance or further depends on the discussion and conclusion in RAN2#124. For RRC_CONNECTED mode - RAN2 has studied the procedure, and concluded the feasibility for LP-WUS to control PDCCH monitoring in RRC_Connected mode with and without C-DRX. Detailed design is to be decided in WI. Note: Both duty cycle and continuous monitor for LP-WUS could be further discussed in WI phase for both RRC_IDLE/INACTIVE and RRC_CONNECTED mode.

----------------------------------------------------------------------End of TP-------------------------------------------------------------

Proposal: RAN2 to discuss and agree the above TR conclusion for the high layer study on LP-WUS/WUR.

- Conclusion

In this contribution, rapporteur provides the initial views on the TR conclusion for the study on LP-WUS/WUS, and provides the following proposal:

Proposal: RAN2 to discuss and agree the above TR conclusion for the high layer study on LP-WUS/WUR.

- References

- RP-232672, Revised SID: Study on low-power Wake-up Signal and Receiver for NR.