3GPP TSG-RAN WG2 #124	R2-2312607

Chicago, US, 13th – 17th Nov 2023

Agenda Item:	7.5.4.3 Configured Grant enhancements for XR

Source:	NEC

Title:	CG enhancement for XR

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss which factors can be taken into account to set UTO-UCI.

# Discussion

UTO-UCI will be a bitmap and included in PUSCH transmission at Physical layer, But the decision of UTO can only be made by MAC entity based on availability of L2 data for transmission.

We think at least there are two cases that MAC can predicate the usage of future CG transmission occasion:

Case 1: receiving end of data burst indication

When UE receive the last PDU of a data burst (e.g., an end of data burst mark from UE upper layer is received together), based on total amount of data available in L2 buffer for transmission, MAC can determine how many TOs are needed to transmit all these buffered data and then how many TOs will be skipped. Following figure shows an example where all bits in UTO-UCI are set as “NOT unused” till “end of data burst” is received. At the moment of receiving “end of data burst” UE is able to calculate that TO3 and TO4 will be used to transmit the buffered data and TO5-TO7 can be marked as “unused”:

Figure 3: Case 1 receiving end of data burst indication

Case 2: Empty buffer size

when there will have no buffered data, UE can indicate the following TOs within the periodicity/defined window as “unused”

in detailed, there could be two sub-cases

2-a) late arrival of a data burst. if multiple CG TOs are configured to cope with jitter, then configured multiple CG TOs spread out around the reference arrival timing point to cover the jitter window, so it is possible that either first or last TOs will not be used depends on the packet arrival timing

2-b) all buffered data has been scheduled/discarded.

Figure 4: Case 2 Empty buffer size

Proposal 1 RAN2 consider to specify below cases where UE can mark a future TO as “unused “:

Case 1: receiving end of data burst indication

Case 2: Empty buffer size

# Summary

This contribution provided our initial opinion on how to set UTO-UCI:

Proposal 1 RAN2 consider to specify below cases where UE can mark a future TO as “unused “:

Case 1: receiving end of data burst indication

Case 2: Empty buffer size