3GPP TSG-RAN WG2 Meeting #124															R2-2312719

Chicago, USA, Nov. 13th – 17th, 2023

Agenda item:		7.11.3

Source:				Samsung

Title:					Remaining Issues for Shared Processing

Document for:		Discussion & Decision

# 1 Introduction

This contribution discusses the remaining issues for shared processing for unicast and broadcast reception.

# 2 Discussion

### 2.1 Rel-18 MII - ReconfigurationWithSync and Reestablishment scenarios

In legacy Rel-17 RRC spec (see excerpts below), robustness of MII transmission is ensured for the ReconfigurationWithSync and Reestablishment procedures and MII transmission is initated again, for the cases when MII was initiated during 1 second preceding reception of the RRCReconfiguration message or RRCReestablishment message, or after receiving RRCReconfiguration applied due to conditional reconfiguration execution.

It is quite spontaneous logic to have a common or similar behaviour as well for Rel-18 MII transmission (i.e. related to shared processing for unicast and broadcast reception). As per Rel-18 eMBS WID, UE receives unicast on serving cell and receives MBS broadcast on non-serving cell and MII essentially provides UE’s dynamic baseband /hardware processing capability due to simultaneous reception of unicast and broadcast.

For Rel-18 MII, gNB for serving cell is not required to provide any broadcast service, and it is natural & reasonable that SIB21 may not be broadcasted by the PCell. However, SIB1 including non-ServingCellMII is essential requirement for the PCell to allow Rel-18 MII transmission. It is important as gNB for the serving cell interprets Rel-18 MII for the non-serving cell information to use for unicast configuration and scheduling. Therefore, we need to consider non-servingCellMII provided in SIB1 by the PCell for initiating MII during ReconfigurationWithSync and Reestablishment scenarios.

Proposal 1A: When non-servingCellMII is provided in SIB1 by the PCell, UE initiates transmission of MII during ReconfigurationWithSync and Reestablishment scenarios (i.e. for the cases when MII was initiated during 1 second preceding reception of the RRCReconfiguration message or RRCReesablihment message, or after receiving RRCReconfiguration applied due to conditional reconfiguration execution).

Proposal 1B: Adopt text proposal TP1 and TP2 as provided.

Text Proposal: TP1

5.3.5.3 Reception of an RRCReconfiguration by the UE The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO, CPA or CPC): .....<Omitted Text>........ 1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; or, 1> if sl-PathSwitchConfig was included in reconfigurationWithSync included in spCellConfig of an MCG, and when successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE): .....<Omitted Text>........ 2> if reconfigurationWithSync was included in masterCellGroup and the target cell provides SIB21or provides SIB1 including non-ServingCellMII: 3> if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding reception of this RRCReconfiguration message; or 3> if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE has initiated transmission of an MBSInterestIndication message after having received this RRCReconfiguration message: 4> initiate transmission of an MBSInterestIndication message in accordance with clause 5.9.4;

Text Proposal: TP2

5.3.7.5 Reception of the RRCReestablishment by the UE The UE shall: .....<Omitted Text>........ 1> if SIB21 is provided by the PCell; or 1> if nonServingCellMII is provided in SIB1 by the PCell: 2> if the UE initiated transmission of an MBSInterestIndication message during the last 1 second preceding detection of radio link failure: 3> initiate transmission of an MBSInterestIndication message in accordance with 5.9.4;

# 3 Conclusion

Request RAN2 to discuss and agree to the following proposals:

Proposal 1A: When non-servingCellMII is provided in SIB1 by the PCell, UE initiates transmission of MII during ReconfigurationWithSync and Reestablishment scenarios (i.e. for the cases when MII was initiated during 1 second preceding reception of the RRCReconfiguration message or RRCReesablihment message, or after receiving RRCReconfiguration applied due to conditional reconfiguration execution).

Proposal 1B: Adopt text proposal TP1 and TP2 as provided.