3GPP TSG-RAN WG2 Meeting #124	R2-2312834

Chicago, USA, 13 - 17 November 2023

Agenda item:	7.8.5

Source: 	Huawei, HiSilicon

Title: 	Further discussion on UAV remote identification broadcast

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #123-bis, the following agreements have been reached regarding UAV ID broadcast for NR UAVs [1]:

Agreements:

- Configure the separate resource pools one-level higher on SL-BWPPoolConfigCommon/SL-BWPPoolConfig. Adopt the TP for 38.331.

- Add additional flag to indicate whether the pool is for BRID, DAA, or both (check if this introduces a problem)

In this contribution, we analyze the potential issue when separate resource pool is supported.

- Discussion

In the last meeting, RAN2 agreed to introduce a dedicated resource pool for BRID and DAA broadcasts of UAV. There is an issue that if the NW does not configure the dedicated resource pool for UAV, whether the UAV can use the normal resource pool needs to be confirmed. In Rel-17, sidelink relay discovery resource pool was introduced for relay discovery. The UE uses the sidelink relay discovery resource pool to send discovery message when the discovery resource pool is configured. Otherwise, the UE can use the sidelink normal resource pool for discovery message.

We think the A2X dedicate resource pool can follow the mechanism of sidelink relay discovery resource pool. The UAV uses the dedicated resource pool for A2X service if the A2X dedicated resource pool is configured. Otherwise, the UAV can use the “normal” resource pool for A2X service.

Proposal 1: RAN to confirm that the UAV uses the dedicated resource pool for A2X service if the A2X dedicated resource pool is configured. Otherwise, the UAV can use the “normal” resource pool for A2X service.

Till now, RAN2 has discussed the dedicated resource pool for DAA and BRID. However, SA2 has also defined C2 communication service for UAV besides BRID and DAA services. Thus, RAN2 should discuss that whether the C2 communication can use the dedicated resource pool.

Proposal 2: RAN2 should discuss that whether the C2 communication of UAV can use the dedicated resource pool.

In the last meeting, RAN2 agreed to add additional flag to indicate whether the pool is for BRID, DAA or both. Then, the UAV needs to distinguish the service type when the service data is coming. According to the current spec, one solution is that the service type can be distinguished by the PQI defined in SA2 [1]. However, we are not sure whether the UE MUST use the PQI defined by SA2 for DAA/BRID .

Proposal 3: RAN2 should ask SA2 whether the UE must use the PQI for DAA/BRID that defined by SA2.

If the UE uses the non-standardized PQI or uses the legacy PQI for BRID or DAA service, the UE cannot correctly indicate to the gNB which resource pool should be configured. For example, we assume that the UE enters RRC_Connected state and has DAA/BRID service data to be transmitted but the corresponding resource pools are not configured. At this moment, the UE needs to request the corresponding resource pool. Then, it sends the SUI to the NW which includes PQI. The NW will configure the corresponding resource pool based on the PQI. Unfortunately, the configuration may be wrong because the UE uses the non-standard PQI or uses the legacy PQI for BRID or DAA service. Thus, an indication should be included in SUI message to indicate the service type of the BRID or DAA.

Proposal 4: An indication should be included in SUI message to indicate the service type of the BRID or DAA.

- Conclusion

Proposal 1: RAN to confirm that the UAV uses the dedicated resource pool for A2X service if the A2X dedicated resource pool is configured. Otherwise, the UAV can use the “normal” resource pool for A2X service.

Proposal 2: RAN2 should discuss that whether the C2 communication of UAV can use the dedicated resource pool.

Proposal 3: RAN2 should ask SA2 whether the UE must use the PQI for DAA/BRID that defined by SA2.

Proposal 4: An indication should be included in SUI message to indicate the service type of the BRID or DAA.

- Reference

- S2-2311428_was10251_23256_PQI_v3