3GPP TSG-RAN WG2 Meeting #124	R2-2313425

Chicago, USA, 13 – 17 November 2023

Agenda item:	7.0.2

Source:	Nokia, Nokia Shanghai Bell, InterDigital

Title:	LCID extension for CCCH

WID/SID:	NR_redcap_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In the previous RAN2#123-Bis meeting, LCID values extension was discussed with the following conclusions:

Agreements:

-	Solutions that increase the msg3 size are excluded (e.g. eLCID cannot be used as a solution for this purpose)

-	RAN2 will discuss and find a solution in Rel-18

-	Use first R bit for LCID extension. It is only applied to UL, and for now only CCCH/CCCH1 and enabled by network.   FFS on details

-	An explicit indication from network will be added to enable this feature.  FFS on the details of signaling.

-	A single CR will capture the extension and LCID value to be used.  Only the need for LCID value usage will be agreed by each individual session.  Combinations can be discussed in individual session and can be brought up to common session for discussion only if need.   MAC rapporteur will provide the CRs.

This TDoc discusses the design details further.

# 2	Discussion

## 2.1	MAC subheader design

MAC UL/DL-SCH subheader types including the LCID are the following (TS 38.321):

Figure 6.1.2-1: R/F/LCID/L MAC subheader with 8-bit L field Figure 6.1.2-2: R/F/LCID/L MAC subheader with 16-bit L field Figure 6.1.2-3: R/LCID MAC subheader

When the first (leftmost) R bit is used to indicate the new LCID space, even though the intention is (for now) to use this only for CCCH/CCCH1, it means that the R bit is effectively used for any MAC UL-SCH type as the NW cannot know if it is CCCH or another SDU for CONNECTED mode UE so it is impossible to define R bit for other purpose later, which was exactly the same reason why the second R bit was not adopted in the previous meeting. Hence, in case the new LCID field is also 6 bits long, there is no reserved bit for future use other than for R/LCID MAC subheader type. This would not allow extending all the subheader types with a specific feature requiring a single bit. On the other hand, half of the current LCID space is already used for Logical Channels whereas only half of the space is used for MAC CEs or early indications with CCCH/CCCH1. Consequently, it seems sufficient to extend the LCID space for CCCH/CCCH1 use with 32 LCID values (5 bits) for now and enable the further extension possibility for any UL-SCH MAC subheader type – by means of an additional R bit. This R bit could be, e.g., the 3rd leftmost bit or the rightmost bit of the first byte in the MAC subheader.

Proposal 1: When LCID extension is indicated by the leftmost R bit in the MAC subheader, a new R bit is introduced for each UL-SCH MAC subheader type which includes the new LCID field. This R bit can be the 3rd leftmost bit or the rightmost bit of the first byte in the MAC subheader.

Since the early indications are generally always applied with both of the CCCH and CCCH1, it seems a bit clumsy to allocate two separate LCID value always to indicate the CCCH size of 48 or 64 bits. Instead, one bit of the MAC subheader could be used to indicate whether CCCH or CCCH1 is used when the LCID extension is indicated by the leftmost R bit in the MAC subheader.

Proposal 2: Assign one bit in the MAC subheader to indicate whether CCCH or CCCH1 is transmitted as MAC SDU.

With the above approach, the resulting UL-SCH MAC subheader types including the LCID extension would be as follows. It should be noted that the new fields need to be illustrated only with the MAC subhdear for fixed sized MAC SDU/CE given the extension is only applied for CCCH/CCCH1 for this version of the specification. However, the LX field needs to be included for all subheader types given it cannot be used for any other purposes later on. For simplicity, only the MAC subheaders with LCID are illustrated, however, similarly the first R bit would need to be assigned an LX field for the MAC subheaders with eLCID.

Figure 6.1.2-1: LX/F/LCID/L MAC subheader with 8-bit L field Figure 6.1.2-2: LX/F/LCID/L MAC subheader with 16-bit L field Figure 6.1.2-3: LX/R/LCID MAC subheader Figure 1: LX/CL/R/sLCID MAC subheader

In the figures above, LX is used to indicate the LCID extension, new R bit is allocated as the 3rd leftmost bit, CL bit is used to indicate the CCCH length (either CCCH/48bits or CCCH1/64bits), and sLCID is the new short LCID field with 32 values usable for early indications.

Proposal 3: Agree the above MAC subheader types for UL-SCH.

Proposal 4: Agree on the TP in the Annex A.

## 2.2	Network indication to enable the feature

It was agreed that the NW provides explicit indication to the UEs that it supports the new LCID extension feature – this is evidently required. However, it seems to us that the NW needs to enable each feature in a cell it can employ those early indications sent by the UE (e.g., eRedCap, Msg4 HARQ feedback repetition, MUSIM indication, etc.). Hence, no separate explicit indication seems to be required but the UE deduces whether the LCID extension can be used based on whether the NW indicates support for a feature that require the LCID extension bits to be used.

Proposal 5: UE deduces from the features indicated by the NW in the system information whether the NW supports the LCID extension.

## 2.3	Whether to use legacy LCID or new LCID in Rel-18

Since the legacy LCID space still have a few reserved LCID values, it needs to be discussed if the new early indications specified in Rel-18 features use the legacy LCID space or the new LCID space. Since the new LCID space is meant for CCCH/CCCH1 for now, and all the indications agreed to be specified for Rel-18 are also meant for CCCH/CCCH1, it seems reasonable to use the new LCID space for the Rel-18 new early indications. The legacy LCID space can then be used for everything else not only used for CCCH/CCCH1, like any new MAC CEs in the future that require the lowest possible header overhead.

Proposal 6: The Rel-18 early indications for CCCH to be specified (e.g., for eRedCap, NTN, and MUSIM) use the new LCID space.

# 3	Conclusion

Proposal 1: When LCID extension is indicated by the leftmost R bit in the MAC subheader, a new R bit is introduced for each UL-SCH MAC subheader type which includes the new LCID field. This R bit can be the 3rd leftmost bit or the rightmost bit of the first byte in the MAC subheader.

Proposal 2: Assign one bit in the MAC subheader to indicate whether CCCH or CCCH1 is transmitted as MAC SDU.

Proposal 3: Agree the above MAC subheader types for UL-SCH.

Proposal 4: Agree on the TP in the Annex A.

Proposal 5: UE deduces from the features indicated by the NW in the system information whether the NW supports the LCID extension.

Proposal 6: The Rel-18 early indications for CCCH to be specified (e.g., for eRedCap, NTN, and MUSIM) use the new LCID space.

# Annex A: TP for TS 38.321:

First Modified Subclause

### 6.1.2	MAC PDU (DL-SCH and UL-SCH except transparent MAC and Random Access Response)

A MAC PDU consists of one or more MAC subPDUs. Each MAC subPDU consists of one of the following:

-	A MAC subheader only (including padding);

-	A MAC subheader and a MAC SDU;

-	A MAC subheader and a MAC CE;

-	A MAC subheader and padding.

The MAC SDUs are of variable sizes.

Each MAC subheader corresponds to either a MAC SDU, a MAC CE, or padding.

A MAC subheader except for fixed sized MAC CE, padding, and a MAC SDU containing UL CCCH consists of the header fields LX/F/LCID/(eLCID)/L. A MAC subheader for fixed sized MAC CE and padding consists of the header fields LX/R/LCID/(eLCID). A MAC SDU containing UL CCCH consists of either the header fields LX/R/LCID or LX/CL/R/sLCID which is determined based on the Tables 6.2.1-2 and 6.2.1-2c.

Figure 6.1.2-1: R/F/LCID/(eLCID)/L MAC subheader with 8-bit L field

Figure 6.1.2-2: R/F/LCID/(eLCID)/L MAC subheader with 16-bit L field

Figure 6.1.2-3: R/LCID/(eLCID) MAC subheader

Figure 6.1.2-3a: LX/CL/R/LCID MAC subheader

MAC CEs are placed together. DL MAC subPDU(s) with MAC CE(s) is placed before any MAC subPDU with MAC SDU and MAC subPDU with padding as depicted in Figure 6.1.2-4. UL MAC subPDU(s) with MAC CE(s) is placed after all the MAC subPDU(s) with MAC SDU and before the MAC subPDU with padding in the MAC PDU as depicted in Figure 6.1.2-5. The size of padding can be zero.

Figure 6.1.2-4: Example of a DL MAC PDU

Figure 6.1.2-5: Example of a UL MAC PDU

A maximum of one MAC PDU can be transmitted per TB per MAC entity.

Next Modified Subclause

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1, 6.2.1-1c and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The size of the LCID field is 6 bits. If the LCID field is set to 34, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field. If the LCID field is set to 33, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field;

NOTE 1:	For MBS broadcast, a logical channel is identified based on G-RNTI and LCID if the same LCID is allocated for logical channels corresponding to different G-RNTIs.

-	eLCID: The extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

NOTE 2:	The extended Logical Channel ID space using two-octet eLCID and the relevant MAC subheader format is used, only when configured, on the NR backhaul links between IAB nodes or between IAB node and IAB Donor, or for multicast MTCHs.

-	sLCID: The short Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU as described in table 6.2.1-2c. The size of the sLCID field is either 5 bits.

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	LX: The LCID extension field indicates whether the MAC subheader includes an LCID or an sLCID. The size of the LX field is 1 bit. The value 0 indicates that MAC subheader includes an LCID and the value 1 indicates that MAC subheader includes an sLCID. In this version of the specification, the LX can be set to 1 only when the sLCID identifies UL CCCH, otherwise, it is set to 0.

-	CL: CCCH length field indicates whether the CCCH of size 48 bits or CCCH1 of size 64 bits is transmitted when the sLCID identifies UL CCCH. The value 0 indicates CCCH of size 48 bits and the value 1 indicates CCCH1 of size 64 bits.

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel of DCCH, DTCH and multicast MTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35–46 | Reserved |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-1a Values of two-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-1b Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 226 | 64 to 290 | Reserved |
| 227 | 291 | Serving Cell Set based SRS TCI State Indication MAC CE |
| 228 | 292 | SP/AP SRS TCI State Indication MAC CE |
| 229 | 293 | BFD-RS Indication MAC CE |
| 230 | 294 | Differential Koffset |
| 231 | 295 | Enhanced SCell Activation/Deactivation MAC CE with one octet Ci field |
| 232 | 296 | Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field |
| 233 | 297 | Unified TCI States Activation/Deactivation MAC CE |
| 234 | 298 | PUCCH Power Control Set Update for multiple TRP PUCCH repetition MAC CE |
| 235 | 299 | PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE |
| 236 | 300 | Enhanced TCI States Indication for UE-specific PDCCH |
| 237 | 301 | Positioning Measurement Gap Activation/Deactivation Command |
| 238 | 302 | PPW Activation/Deactivation Command |
| 239 | 303 | DL Tx Power Adjustment |
| 240 | 304 | Timing Case Indication |
| 241 | 305 | Child IAB-DU Restricted Beam Indication |
| 242 | 306 | Case-7 Timing advance offset |
| 243 | 307 | Provided Guard Symbols for Case-6 timing |
| 244 | 308 | Provided Guard Symbols for Case-7 timing |
| 245 | 309 | Serving Cell Set based SRS Spatial Relation Indication |
| 246 | 310 | PUSCH Pathloss Reference RS Update |
| 247 | 311 | SRS Pathloss Reference RS Update |
| 248 | 312 | Enhanced SP/AP SRS Spatial Relation Indication |
| 249 | 313 | Enhanced PUCCH Spatial Relation Activation/Deactivation |
| 250 | 314 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 251 | 315 | Duplication RLC Activation/Deactivation |
| 252 | 316 | Absolute Timing Advance Command |
| 253 | 317 | SP Positioning SRS Activation/Deactivation |
| 254 | 318 | Provided Guard Symbols |
| 255 | 319 | Timing Delta |

Table 6.2.1-1c Values of LCID for MBS broadcast on DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | MCCH |
| 1–32 | Identity of the logical channel of broadcast MTCH |
| 33–63 | Reserved |

Table 6.2.1-2 Values of LCID for UL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]), except for a RedCap UE |
| 1–32 | Identity of the logical channel of DCCH and DTCH |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one-octet eLCID field) |
| 35 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) for a RedCap UE |
| 36 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) for a RedCap UE |
| 37–42 | Reserved |
| 43 | Truncated Enhanced BFR (one octet Ci) |
| 44 | Timing Advance Report |
| 45 | Truncated Sidelink BSR |
| 46 | Sidelink BSR |
| 47 | Reserved |
| 48 | LBT failure (four octets) |
| 49 | LBT failure (one octet) |
| 50 | BFR (one octet Ci) |
| 51 | Truncated BFR (one octet Ci) |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]), except for a RedCap UE |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

Table 6.2.1-2a Values of two-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to (216 – 1) | 320 to (216 + 319) | Identity of the logical channel |

Table 6.2.1-2b Values of one-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 228 | 64 to 292 | Reserved |
| 229 | 293 | Enhanced Multiple Entry PHR for multiple TRP (four octets Ci) |
| 230 | 294 | Enhanced Multiple Entry PHR for multiple TRP (one octets Ci) |
| 231 | 295 | Enhanced Single Entry PHR for multiple TRP |
| 232 | 296 | Enhanced Multiple Entry PHR (four octets Ci) |
| 233 | 297 | Enhanced Multiple Entry PHR (one octets Ci) |
| 234 | 298 | Enhanced Single Entry PHR |
| 235 | 299 | Enhanced BFR (one octet Ci) |
| 236 | 300 | Enhanced BFR (four octet Ci) |
| 237 | 301 | Truncated Enhanced BFR (four octet Ci) |
| 238 | 302 | Positioning Measurement Gap Activation/Deactivation Request |
| 239 | 303 | IAB-MT Recommended Beam Indication |
| 240 | 304 | Desired IAB-MT PSD range |
| 241 | 305 | Desired DL Tx Power Adjustment |
| 242 | 306 | Case-6 Timing Request |
| 243 | 307 | Desired Guard Symbols for Case 6 timing |
| 244 | 308 | Desired Guard Symbols for Case 7 timing |
| 245 | 309 | Extended Short Truncated BSR |
| 246 | 310 | Extended Long Truncated BSR |
| 247 | 311 | Extended Short BSR |
| 248 | 312 | Extended Long BSR |
| 249 | 313 | Extended Pre-emptive BSR |
| 250 | 314 | BFR (four octets Ci) |
| 251 | 315 | Truncated BFR (four octets Ci) |
| 252 | 316 | Multiple Entry Configured Grant Confirmation |
| 253 | 317 | Sidelink Configured Grant Confirmation |
| 254 | 318 | Desired Guard Symbols |
| 255 | 319 | Pre-emptive BSR |

Table 6.2.1-2c Values of sLCID for UL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 to 15 | Reserved |

End of Changes