3GPP TSG-RAN WG2 Meeting #124	R2-2313632

# Chicago 13th – 17th Nov, 2023

Source: 			ZTE Corporation, Sanechips, vivo

Title: 	Proposal on the MN-SN Coordination for the MUSIM

Agenda item:	7.17.2

Document for: 	Discussion and Decision

- Introduction

During on-line discussion, the MN-SN coordination for the MUSIM topic was touched and would be further discussed in the CB session. In this paper, we just give some proposals that maybe agreed by companies based on some offline discussion.

- Discussion

MN-SN coordination on the temporary capability restriction

For the MN-SN coordination on the temporary Capability Restriction, based on the online discussion, there are 2 solutions:

- Solution 1: The MN can indicate the proactive/reactive temporary capabilities restriction (e.g. musim-Ca           pRestriction-r18) to the SN together with the musim-candidateBandList-r18.

- Solution 2: MN Indicate the forbidden/affected band entry for each allowed BC that included in the

legacy allowedBC-ListMRDC to the SN.

In the solution 1, for that in the current structure of musim-CapRestriction-r18, the index was used to indicate the band Entry in the musim-candidateBandList-r18, thus the musim-candidateBandList-r18 would be also indicated for the SN to determine the dedicated band indicator. The solution 2 is tightly coupled with the legacy allowedBC-ListMRDC, and for the case that the temporary capability only affects the SN side, the MN may only forward the MUSIM temporary capability restriction info to the SN, it doesn't need to include the legacy allowedBC-ListMRDC to the SN. Thus, the solution 1 is preferred by more companies.

WF 1: The MN can indicate the proactive/reactive temporary capabilities restriction (e.g. musim-CapRestriction-r18) to the SN together with the musim-candidateBandList-r18.

MN-SN coordination on “keep” solution

For the “keep” solution, during the on-line discussion, it has been agreed that

=>After UE indicates its preference for gap priority “keep” solution option, NW can configure UE to use “keep” solution option or not.

Furthermore, some papers also proposed to transfer the keep solution from the MN to the SN.

R2-2311802 Procedures and signalling for MUSIM temporary capability restriction OPPO [1] Proposal 9: MN indicates SN to use the keep solution, i.e. to keep all colliding MUSIM gaps irrespective of the priority of the MUSIM gaps. R2-2312818 Discussion on remaining open issues on capability restriction Ericsson [2] Proposal 7: The “Keep solution indication” should be transferred from the MN to the SN node in the CG-ConfigInfo inter-node message. R2-2313420 Further discussion on UE capabilities and MN-SN coordination Samsung [3] Proposal 2: For keep solution indication, the MN can indicate keep solution if configured to the SN.

Thus, maybe we can suggest to also discuss the below Way Forward during the CB-Session:

WF 2: The “Keep solution indication” should be transferred from the MN to the SN node in the CG-ConfigInfo inter-node message.

Other:

During offline discussion, companiey also discussed whether the emporary capability restriction on measurement gap requirement shall also be coordinated on the MN-SN. However as in [4] considering that the measurement gap requirement reporting by RRC Reconfiguration Complete message is not supported in NR-DC, the MUSIM temporary capability restriction on measurement gap requirement is not supported in NR-DC, and thus no impact to the MN-SN interface.

#### – NeedForGapsInfoNR The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.

WF 3: MUSIM temporary capability restriction reporting on measurement gap requirement is not supported in NR-DC.

- Conclusion

With the above analysis, we have the following Way Forwards:

WF 1: The MN can indicate the proactive/reactive temporary capabilities restriction (e.g. musim-CapRestriction-r18) to the SN together with the musim-candidateBandList-r18.

WF 2: The “Keep solution indication” should be transferred from the MN to the SN node in the CG-ConfigInfo inter-node message.

WF 3: MUSIM temporary capability restriction reporting on measurement gap requirement is not supported in NR-DC.

- Reference

[1]R2-2311802    Procedures and signalling for MUSIM temporary capability restriction   OPPO

[2]R2-2312818      Discussion on remaining open issues on capability restriction  Ericsson

[3]R2-2313420      Further discussion on UE capabilities and MN-SN coordination     Samsung

[4]R2-2312729	Discussion on MUSIM temporary capability restriction in NR-DC	Huawei, HiSilicon