3GPP TSG-RAN WG2 Meeting#125	   R2-2400127

Athens, Greece, 26th February-1st March 2024

Agenda item:		7.21.3

Source:	vivo

Title:	Remaining UP Issues on eCovEnh

Document for:		Discussion and Decision

1 Introduction

In this contribution, we would like to provide our views on the following 2 remaining UP issues on eCovEnh,

Issue 1: Whether Msg1 repetition is applicable in NTN network? If applicable, when to start RAR window and how to capture it in MAC?

Issue 2: Whether DWS and mTRP can be configured together? Even if the answer is "Yes", RAN2 assumes there is no RAN1 or RAN4 impact

2 Discussion

Msg1-repetition is introduced for UL coverage enhancement, which is largely beneficial in the NTN case due to the large free space pathloss. So, it is necessary to support Msg1 repetition in NTN network. Moreover, from the technical point of view, no new functional enhancement is needed to realize this goal considering the NTN band is independent of the TN band. That is if the NTN network would like to adopt Msg1 repetition, it can directly reuse the already specified Msg1 repetition framework by configuring the parameters applicable to NTN case (e.g. the RSRP threshold value and preamble target power are specific to the NTN case).

Observation: Supporting Msg1 repetition helps to alleviate the negative impact of free space pathloss in NTN case.

Proposal 1: RAN2 confirms that Msg1 repetition is applicable in NTN network.

Next, it comes to how to support this in the MAC spec. As mentioned the MAC rapporteur, we also agree that the remaining issue is when to start the RAR window and how to model it in MAC spec. In NTN network, the UE delays the starting point of RAR window as specified in TS 38.213 due to the large UE-gNB RTT time. It can also seen that this operation is not limited to normal Msg1 transmission but is also applicable to Msg1 repetition transmission. In this sense, the MAC spec does not need to explicitly capture how to delay the RAR window starting point. That is the PHY can consider the additional UE-gNB RTT time.

TS 38.213 subclause 8.2 Random access response - Type-1 random access procedure In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by kmac or kmac=0 if kmac is not provided. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

Based on the above, the corresponding TP is given as follows,

TP for 38.321-i00 5.1.4 Random Access Response reception Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall: 1> if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity: 2> if the contention-free Random Access Preamble for beam failure recovery request was transmitted on a non-terrestrial network: 3> start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the PDCCH occasion as specified in TS 38.213 [6]. 2> else: 3> start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission. 2> monitor for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while ra-ResponseWindow is running. 1> else: 2> if the Random Access Preamble was transmitted on a non-terrestrial network: 3> if the Random Access Preamble is transmitted with repetitions: 4> start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] after the end of the last Random Access Preamble transmission (within all repetitions). 3> else: 4> start the ra-ResponseWindow configured in RACH-ConfigCommon at the PDCCH occasion as specified in TS 38.213 [6]. 2> else if the Random Access Preamble is transmitted with repetitions: 3> start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion from the end of all repetitions of the Random Access Preamble transmission as specified in TS 38.213 [6]. 2> else: 3> start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission. 2> monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running. <the unchanged part is omiited>

Proposal 2: For Msg1 repetition in NTN, RAN2 assumes PHY layer handles the UE-gNB RTT for RAR window starting (No spec change is required).

Proposal 3: For Msg1 repetition in NTN, MAC starts the ra-ResponseWindow at the first PDCCH occasion (specified in TS 38.213) after the end of the last Random Access Preamble transmission.

Next, we would like to provide our view on whether DWS and mTRP can be configured together. In our understanding, regarding the combination of multiple-TRP PDCCH or multiple-TRP PDSCH and DWS, it seems feasible considering that the DL reception is independent of UL transmission. So, the open issue looks intended for the combination of multi-TRP PUSCH and DWS case. Then the first question that comes to us is whether twoPHRMode (in case of multi-TRP PUSCH repetition) can be configured together with dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 or dynamicTransformPrecoderFieldPresenceDCI-0-2-r18. From the use case perspective, the combination of those 2 features indeed helps to enhance the UL coverage considering the multi-TRP PUSCH repetition mode brings diversity gain. Moreover, according to the current specification, it seems there is no restriction for the simultaneous configuration of twoPHRMode and R18 DWS configuration. Further, the potential RAN2 spec impact may be defining new MAC CEs for reporting two PH and Pcmax for assumed PUSCH.

Proposal 4: RAN2 confirms that twoPHRMode can be configured along with R18 DWS configuration.

3 Conclusion

In this contribution, we have discussed the remaining UP issues eCovEnh. All the observation and proposals are summarized below:

Observation: Supporting Msg1 repetition helps to alleviate the negative impact of free space pathloss in NTN case.

Proposal 1: RAN2 confirms that Msg1 repetition is applicable in NTN network.

Proposal 2: For Msg1 repetition in NTN, RAN2 assumes PHY layer handles the UE-gNB RTT for RAR window starting (No spec change is required).

Proposal 3: For Msg1 repetition in NTN, MAC starts the ra-ResponseWindow at the first PDCCH occasion (specified in TS 38.213) after the end of the last Random Access Preamble transmission.

Proposal 4: RAN2 confirms that twoPHRMode can be configured along with R18 DWS configuration.