3GPP TSG-RAN WG2 Meeting #125	R2-2400136

Athens, Greece, 26th February – 1st March, 2024

Agenda Item:	7.12.1

Source:	Huawei, HiSilicon

Title:	Reply LS on UE RACH-less handover for mobile IAB

Document for: Discussion

1	Introduction

RAN2 received an LS from RAN3 in the last meeting [1], which informed RAN2 three issues from RAN3 that:

(1) During DU migration, UE handovers may not only occur from the source logical DU’s cell but also from other cells to the co-located target logical DU’s cell. RAN3 assumes that RACH-less handover can only be applied to those UEs that are handed over from the source logical DU’s cell. The co-located target logical DU therefore needs to be able to derive from the information it receives during UE handover preparation, whether the UE is presently connected to the source logical DU. (2) When the target logical DU configures the UE’s beam to be used in the target cell for RACH-less handover based on network-implementation-specific knowledge (i.e., without measurement report), it needs to identify the beam configuration this UE presently uses in the source logical DU’s cell. For this purpose, the target logical DU needs to know a UE ID the logical source DU uses for this UE in order to identify the UE in the source logical DU during the UE handover preparation procedure. (3) When the target logical DU configures the UE’s beam to be used in the target cell for RACH-less handover based on legacy measurements, it needs to be able to obtain the beam information the UE reported to the source logical DU’s CU in the measurement report.

RAN3 requested RAN2 to take the above issues into account, and to consider whether the existing RRC container (i.e., HandoverPreparationInformation) contains sufficient information to address the above issues, and to provide feedback if needed.

In this contribution, we provide our views on the RAN3’s LS.

2	Discussion

For issue 1, it is reasonable the target logical DU needs to know the UE performing handover is from the source logical DU. From RAN2 perspective, we see the current HandoverPreparationInformation IE includes the RRM-Config IE; the RRM-Config includes the measurement results; and in the measurement result, the PCI for the measured cell is included. The source F1 terminating CU can include the measurement result of the source cell together with the measurement result of the cells under the target F1 terminating CU in the HandoverPreparationInformation, and the target logical DU knows the UE is from the source logical DU according to the PCI of the source cell. The solution above is based on the assumption the PCI collision is avoided. If RAN3 still has concerns on whether the PCI can uniquely identify the source cell, the CGI of the source cell may need to be added in the handover request explicitly.

Proposal 1a:	For issue 1, if no PCI collision issue during the RACH-less HO is considered, the target logical DU can know whether the UE is from the source logical DU, based on the PCI in the measurement result included in the current HandoverPreparationInformation. Otherwise, the CGI of the source cell needs to be explicitly added in the handover request (up to RAN3).

For issue 2, the current HandoverPreparationInformation IE includes the sourceConfig IE, which means the radio resource configuration as used in the source cell. The sourceConfig contains all the rrcreconfiguration information including the beam configuration in the source cell, to let the target cell make the delta configuration. The target logical DU can identify the beam configuration used in the source logical DU’s cell and determine the target beam according to the sourceConfig if the network-specific implementation is adopted. Thus, no additional UE source ID is needed. It is proposed:

Proposal 1b:	For issue 2, the sourceConfig included in the current HandoverPreparationInformation is sufficient for the target logical DU to know the source beam configuration and to decide the target beam, if the network-specific implementation is adopted. No additional UE ID in source logical DU is needed.

For issue 3, the current measurement result included in the HandoverPreparationInformation already includes the beam information, and there is no further spec impact.

Proposal 1c:	For issue 3, the current measurement results included in the HandoverPreparationInformation already provides the beam information.

It is also proposed:

Proposal 2:	RAN2 reply the LS to RAN3 with the above Proposals 1a/1b/1c.

# 3	Conclusion

This contribution analysis the issues raised by the RAN3 LS, and makes the following proposals:

Proposal 1a:	For issue 1, if no PCI collision issue during the RACH-less HO is considered, the target logical DU can know whether the UE is from the source logical DU, based on the PCI in the measurement result included in the current HandoverPreparationInformation. Otherwise, the CGI of the source cell needs to be explicitly added in the handover request (up to RAN3).

Proposal 1b:	For issue 2, the sourceConfig included in the current HandoverPreparationInformation is sufficient for the target logical DU to know the source beam configuration and to decide the target beam, if the network-specific implementation is adopted. No additional UE ID in source logical DU is needed.

Proposal 1c:	For issue 3, the current measurement results included in the HandoverPreparationInformation already provides the beam information.

Proposal 2:	RAN2 reply the LS to RAN3 with the above Proposals 1a/1b/1c.

# 4	Reference

- R3-238048, Reply LS on UE RACH-less handover for mobile IAB