3GPP TSG-RAN WG2 Meeting #125	R2-2400171

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Title:	Draft Reply LS on R17 DC location signaling

Response to:	R2-2400059_R4-2321950

Release:	Release 17

Work Items:	NR_RF_FR2_req_enh2-Core

Source:	RAN2

To:	RAN4

CC:

Contact Person:

Name:		Xiaodong Yang

E-mail Address:	yangxiaodong5g@vivo.com

# 1	Overall description

RAN2 would like to thank RAN4 for their LS on R17 DC location signaling.

RAN2 has concluded to modify the conditions for using offsetValue and offsetlist in offsetToDefault for DC location report, the specific changes are as follows.

offsetToDefault Indicates the DC location offset to the default DC location derived from defaultDC-Location. The lowest SCS in the CC group is used as the offset granularity. Value 0 respresents no offset. offsetValue is used in case DefaultDC-Location is not changed due to carrier activation or deactivation and BWP activation or deactivation. offsetlist is used in case DefaultDC-Location is changed due to carrier activation or deactivation and BWP activation or deactivation. Each entity in this list corresponds to the entry in carriers combination in IntraBandCC-CombinationReqList of the intra-band CA component. For each CC group, the UE shall include the same number of entries, and listed in the same order as in CC-CombinationList. If DefaultDC-Location is set the activeCarrier, same offsetValue is signalled for all requested carriers combinations with same active carriers states(regardless of the active BWP index).

# 2	Actions

To RAN4

RAN2 kindly requests RAN4 to take the above information into account during the following work.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #125bis	15th Apr. – 19th Apr. 2024            Changsha, Hunan Province , CN

TSG RAN WG2 Meeting #126	20th May – 24th May 2024            Fukuoka City, Fukuoka , JP