3GPP TSG-RAN WG2 Meeting #125	R2-2400294

# Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	7.15.3

Source: 	Xiaomi

Title:  	Correction on TS 38.321 for SL

Document for:	Discussion and Decision

1	Introduction

In this contribution, we would like to discuss some corrections related to TS 38.321 for SL and have corresponding proposals/TPs.

2	Discussion

2.1 Issue 3

Based on the issue list from rapporteur, one remaining issue is whether to consider the HARQ attribute during carrier selection. Actually in R16 V2X, during resource pool selection, UE should select the resource pool configured with PSFCH resources if the LCH is set to HARQ-enabled.

1> if the MAC entity has selected to create a selected sidelink grant corresponding to transmissions of multiple MAC PDUs, and SL data is available in a logical channel: 2> if the MAC entity has not selected a pool of resources allowed for the logical channel: … 3> else if sl-HARQ-FeedbackEnabled is set to enabled for the logical channel: 4> select any pool of resources configured with PSFCH resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 3> else: 4> select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured.

In addition, there is restriction on NW configuration to configure at least one RP with PSFCH if at least one LCH is configured as HARQ-enabled LCH.

NOTE 2: The MAC entity expects that PSFCH is always configured by RRC for at least one pool of resources in sl-TxPoolSelectedNormal and for the resource pool in sl-TxPoolExceptional in case that at least a logical channel configured with sl-HARQ-FeedbackEnabled is set to enabled.

When it comes to SL, we think similar principle should be adopted, i.e., HARQ attribute should be considered during carrier selection, e.g., if the LCH triggering carrier selection is HARQ-enabled, as long as the per-carrier CBR satisfying the threshold condition and the carrier having at least one RP configured with PSFCH, can the carrier be considered as candidate carrier. In addition, from NW configuration point of view, NW should configure at least one carrier having at least one RP with PSFCH in case at least one LCH is HARQ-enabled. Another alternative is that we can restrict NW to configure each carrier to have at least one RP with PSFCH, in which case, HARQ attribute may not be considered during carrier selection.

Proposal 1: RAN2 confirms HARQ attribute should be considered during carrier selection.

2.2 Issue 1

Regarding procedure’s structure as mentioned by rapporteur, we think the existing text procedure correctly captures the order, i.e., UE selects carrier during which both HARQ attribute and CBR are considered, and then UE selects resource pool on the selected carrier and create SL grant on the selected resource pool. During carrier selection, UE needs to determine the per-carrier CBR to justify whether the carrier can be considered as candidate carrier or not, in this case, UE needs to select a RP for CBR measurement during carrier selection.

Proposal 2: RAN2 confirms UE selects carrier first and then selects RP on the selected carrier.

2.3 Issue 2

Regarding the decouple of “pool selection for CBR measurement” and “pool selection for SL grant”, since we have agreed to reuse LTE principle to determine the per-carrier CBR, therefore, it remains unclear whether the resource pool associated with the per-carrier CBR is the final selected resource pool or not and whether UE can only select single resource pool on the selected carrier or not.

Agreements on per-carrier CBR 1: Confirms the working assumption “Same principle as LTE V2X CA is applied to determine per-carrier CBR” as an agreement.

- Alternative 1: couple “pool selection for CBR measurement” and “pool selection for SL grant”

For this option, companies think as long as one carrier is selected (e.g., based on CBR), the resource pool selected to determine the per-carrier CBR should be the final selected resource pool and UE can only select one resource pool. In this case, during carrier selection procedure, when checking the per-carrier CBR, UE needs to select a resource pool that should also satisfy the HARQ requirement, i.e., if the LCH is HARQ enabled, UE needs to select a resource pool configured with PSFCH to determine the per-carrier CBR.

- Alternative 2: decouple “pool selection for CBR measurement” and “pool selection for SL grant”

For this option, companies think during carrier selection procedure, when checking the per-carrier CBR, it should be up to UE implementation to select any resource pool to determine the per-carrier CBR, e.g., no need to consider the HARQ attribute. As long as the carrier is selected (e.g., based on CBR), UE performs resource pool selection on the selected carrier, based on e.g., HARQ attribute. In this case, the RP for CBR measurement and the RP for SL grant can be different RPs. Even the LCH is HARQ-enabled, the RP for CBR measurement can be configured without PSFCH while the RP for grant creation should be configured with PSFCH.

From our point of view, we think the intention of the agreement is to reuse the single resource pool CBR as the per-carrier CBR, but not to limit the final selected resource pool as the resource pool associated with the per-carrier CBR and not to limit to select single resource pool. Even in R16, multiple resource pools can be selected, it does not make sense to limit to single in R18. Therefore, we think alternative 2 is more reasonable and aligned with the existing framework.

Proposal 3: RAN2 confirms the “pool selection for CBR measurement” and “pool selection for SL grant” are decoupled.

According to the running CR, during RP selection procedure, there is a condition 3> that if single carrier frequency is configured, then the following RP selection follows the legacy. While if multiple carrier frequencies are configured, then UE should trigger carrier reselection. However, for multiple carrier case, after UE selects one or multiple carrier according to 5.22.1.11, the following RP selection procedure on the selected carrier(s) is missing. Since as long as the carrier(s) are selected, how to select the RP on the selected carrier(s) should follow legacy, we suggest to agree with the following TP.

1> if the MAC entity has selected to create a selected sidelink grant corresponding to transmissions of multiple MAC PDUs, and SL data is available in a logical channel: 2> if the MAC entity has not selected a pool of resources allowed for the logical channel: 3> if single carrier frequency is configured or if multiple carrier frequencies are configured and on each selected carrier: 4> if SL data is available in the logical channel for NR sidelink discovery: … 4> else: 5> select any pool of resources among the pools of resources except the pool(s) in sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon, if configured. 3> else (i.e. multiple carrier frequencies are configured): 4> trigger the TX carrier (re-)selection procedure as specified in clause 5.22.1.11.

Proposal 4: RAN2 to agree with the above TP on resource pool selection.

2.4 Issue on enhanced LCP

As mentioned by rapporteur, according to the existing specification, enhanced LCP is performed only when both sl-BWP-DiscPoolConfig and sl-BWP-DiscPoolConfigCommon are not configured. Since SL-U can be applied when sl-BWP-DiscPoolConfig or sl-BWP-DiscPoolConfigCommon is configured, enhanced LCP should also apply to the data transmission on this kind of BWP. Therefore, corresponding enhanced LCP procedure should be added to data transmission on BWP configured with common or dedicated discovery pools.

Proposal 5: RAN2 confirms enhanced LCP also applies to data transmission on BWP configured with common or dedicated discovery pools.

2.5 Resource selection

According to the running CR, when triggers TX resource (re)selection as the result of TX resource (re)selection check, UE should further determine the order of the selected carriers and performs the resource selection procedure on each selected carrier according to the order.

2> if the TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check: 3> In case of dynamic co-channel coexistence of LTE sidelink and NR sidelink as specified in TS 38.214 [7], 4> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier; 4> when SCS of NR SL is (pre-)configured as μ=1, select the time and frequency resources in the first of NR SL slots overlapping with an LTE SL subframe. 5> may additionally select the time and frequency resources in the subsequent NR SL slot overlapping with the LTE SL subframe. 3> if sl-lbt-FailureRecoveryConfig is configured in the SL BWP: 4> indicate to the physical layer RB set information for which Sidelink consistent LBT failure was detected as specified in clause 5.31.2.

However based on the existing specification, TX resource (re)selection check is performed on the selected resource pool. As long as UE selects one or multiple resource pool(s) during resource pool selection procedure, UE can perform TX resource (re)selection check on the selected resource pool(s) and when resource reselection is triggered, UE perform resource selection as in legacy. Therefore, since TX carrier selection is already considered and performed during resource pool selection, there is no need to further check if one or multiple carriers are selected and/or determine the order of the selected carriers during resource selection procedure.

Proposal 6: RAN2 to agree with the above TP on resource selection.

2.6 SL consistent LBT recovery timer

According to the running CR, it is stated as long as the SL BWP is deactivated, UE should stop the sl-lbt-FailureDetectionTimer for all RB sets in the SL BWP, if running. Similarly, when the SL BWP is deactivated, UE should stop the SL consistent LBT failure recovery timer for all RB sets in the SL BWP, if running.

1> if the BWP is deactivated: 2> not transmit SL-BCH on the BWP, if configured; 2> not transmit S-PSS and S-SSS on the BWP, if configured; 2> not transmit PSCCH on the BWP; 2> not transmit SL-PRS on the BWP; 2> not transmit SL-SCH on the BWP; 2> not receive PSFCH on the BWP, if configured; 2> not receive SL-BCH on the BWP, if configured; 2> not receive S-PSS and S-SSS on the BWP, if configured; 2> not receive PSCCH on the BWP; 2> not receive SL-PRS on the BWP; 2> not receive SL-SCH on the BWP; 2> not transmit PSFCH on the BWP, if configured; 2> suspend any configured sidelink grant of configured grant Type 1; 2> clear any configured sidelink grant of configured grant Type 2; 2> cancel, if any, triggered Scheduling Request procedure for sidelink; 2> cancel, if any, triggered Sidelink Buffer Status Reporting procedure; 2> cancel, if any, triggered Sidelink CSI Reporting procedure; 2> cancel, if any, triggered Sidelink DRX Command MAC CE; 2> cancel, if any, triggered Sidelink IUC-Request transmission procedure; 2> cancel, if any, triggered Sidelink IUC-Information Reporting procedure; 2> if sl-LBT-FailureDetectionTimer is configured: 3> stop the sl-LBT-FailureDetectionTimer for all RB sets in the SL BWP, if running. 2> if sl-LBT-RecoveryTimer is configured: 3> stop the sl-LBT-RecoveryTimer for all RB sets in the SL BWP, if running.

Proposal 7: When the SL BWP is deactivated, UE should stop the SL consistent LBT failure recovery timer for all RB sets in the SL BWP, if running.

Proposal 8: RAN2 to agree with the above TP on SL LBT recovery timer.

3	Conclusion

In this contribution, we discussed some corrections related to TS 38.321 for SL and have corresponding proposals/TPs:

Proposal 1: RAN2 confirms HARQ attribute should be considered during carrier selection.

Proposal 2: RAN2 confirms UE selects carrier first and then selects RP on the selected carrier.

Proposal 3: RAN2 confirms the “pool selection for CBR measurement” and “pool selection for SL grant” are decoupled.

Proposal 4: RAN2 to agree with the above TP on resource pool selection.

Proposal 5: RAN2 confirms enhanced LCP also applies to data transmission on BWP configured with common or dedicated discovery pools.

Proposal 6: RAN2 to agree with the above TP on resource selection.

Proposal 7: When the SL BWP is deactivated, UE should stop the SL consistent LBT failure recovery timer for all RB sets in the SL BWP, if running.

Proposal 8: RAN2 to agree with the above TP on SL LBT recovery timer.

4	Reference

[1] TS 38.321