3GPP TSG-RAN WG2 Meeting #125	R2-2400318

Athens, Greece,  Feb. 26th – Mar. 1st, 2024

Agenda Item:    7.13.3

Source:         Xiaomi

Title:         	Consideration on the leftover issues for the RACH optimization

Document for:	Decision

# Introduction

For the RACH optimisation in R18, there is an open issue about the RACH partition information inclusion in the RA report used for the association the RA Report(s) retrieved at a gNB with the corresponding RACH configuration.

In this contribution, we provide our consideration and the draft TP for this issue.

# Discussion

In the previous discussion, RAN2 agreed and replied to RAN3 LS for this issue which indicates that the inclusion of the information consists of the start preamble index and the number of preambles in the partition for which the RA Report was generated is preferred, quoted as below. [1]

RAN2 has discussed the pros and cons of the three alternatives. RAN2 agreed that all the three alternatives are feasible, and RAN2 prefers following Alt2:

Alt2: Enable the addition in the RA Report of RACH partition configuration information. This information consists of the start preamble index and the number of preambles in the partition for which the RA Report was generated. This enables the NG-RAN to determine the RACH partition in use.

In the last meeting, RAN3 had the final consensus on this issue and pointed out that the RAN3 follows RAN2’s decision and the corresponding standard work needs to be finalized in RAN2. So, from RAN2 perspective, we should confirm on this open issue and finalize the corresponding standard work in our specifications.

Observation: RAN3 has the consensus to follows RAN2’s decision and the corresponding standard work needs to be finalized in RAN2.

Proposal: RAN2 confirms to include the start preamble index and the number of preambles in the partition for which the RA Report was generated and finalizes the standard work as in the annexed draft TP 38.331.

# Conclusions

According to the analysis given above, we have the following proposals:

Observation: RAN3 has the consensus to follows RAN2’s decision and the corresponding standard work needs to be finalized in RAN2.

Proposal: RAN2 confirms to include the start preamble index and the number of preambles in the partition for which the RA Report was generated and finalizes the standard work as in the annexed draft TP.

# References

[1] R2-2311612 Reply LS on RACH enhancement, Source: RAN2, To: RAN3.

# Annex: Draft TP 38.331

#### 5.7.10.5	RA information determination

The UE shall, for the last successfully completed or last failed random-access procedure, set the content in ra-InformationCommon as follows:

<omit>

1>	if one or more of the features including RedCap and/or Slicing and/or SDT and/or MSG3 repetition are applicable for this random-access procedure as specified in clause 5.1.1b of TS 38.321[3]:

2>	set the startPreambleForThisPartition and numberOfPreamblesPerSSB-ForThisPartition associated with the random-access resources used in the random-access procedure.

<omit>

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

UEInformationResponse message

-- ASN1START

-- TAG-UEINFORMATIONRESPONSE-START

UEInformationResponse-r16 ::=        SEQUENCE {

rrc-TransactionIdentifier            RRC-TransactionIdentifier,

criticalExtensions                   CHOICE {

ueInformationResponse-r16            UEInformationResponse-r16-IEs,

criticalExtensionsFuture             SEQUENCE {}

}

}

<omit>

RA-InformationCommon-r16 ::=         SEQUENCE {

absoluteFrequencyPointA-r16          ARFCN-ValueNR,

locationAndBandwidth-r16             INTEGER (0..37949),

subcarrierSpacing-r16                SubcarrierSpacing,

msg1-FrequencyStart-r16              INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-FrequencyStartCFRA-r16          INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-SubcarrierSpacing-r16           SubcarrierSpacing                                OPTIONAL,

msg1-SubcarrierSpacingCFRA-r16       SubcarrierSpacing                                OPTIONAL,

msg1-FDM-r16                         ENUMERATED {one, two, four, eight}               OPTIONAL,

msg1-FDMCFRA-r16                     ENUMERATED {one, two, four, eight}               OPTIONAL,

perRAInfoList-r16                    PerRAInfoList-r16,

...,

[[

perRAInfoList-v1660                  PerRAInfoList-v1660                              OPTIONAL

]],

[[

msg1-SCS-From-prach-ConfigurationIndex-r16  ENUMERATED {kHz1dot25, kHz5, spare2, spare1}  OPTIONAL

]],

[[

msg1-SCS-From-prach-ConfigurationIndexCFRA-r16  ENUMERATED {kHz1dot25, kHz5, spare2, spare1} OPTIONAL

]],

[[

msgA-RO-FrequencyStart-r17           INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msgA-RO-FrequencyStartCFRA-r17       INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msgA-SubcarrierSpacing-r17           SubcarrierSpacing                                OPTIONAL,

msgA-RO-FDM-r17                      ENUMERATED {one, two, four, eight}               OPTIONAL,

msgA-RO-FDMCFRA-r17                  ENUMERATED {one, two, four, eight}               OPTIONAL,

msgA-SCS-From-prach-ConfigurationIndex-r17  ENUMERATED {kHz1dot25, kHz5, spare2, spare1}  OPTIONAL,

msgA-TransMax-r17                    ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}  OPTIONAL,

msgA-MCS-r17                         INTEGER (0..15)                                   OPTIONAL,

nrofPRBs-PerMsgA-PO-r17              INTEGER (1..32)                                  OPTIONAL,

msgA-PUSCH-TimeDomainAllocation-r17  INTEGER (1..maxNrofUL-Allocations)               OPTIONAL,

frequencyStartMsgA-PUSCH-r17         INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

nrofMsgA-PO-FDM-r17                  ENUMERATED {one, two, four, eight}               OPTIONAL,

dlPathlossRSRP-r17                   RSRP-Range                                       OPTIONAL,

intendedSIBs-r17                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-Type-r17      OPTIONAL,

ssbsForSI-Acquisition-r17            SEQUENCE (SIZE (1..maxNrofSSBs-r16)) OF SSB-Index    OPTIONAL,

msgA-PUSCH-PayloadSize-r17           BIT STRING (SIZE (5))                            OPTIONAL,

onDemandSISuccess-r17                ENUMERATED {true}                                OPTIONAL

]],

[[

usedFeatureCombination-r18           ReportedFeatureCombination-r18                   OPTIONAL,

triggeredFeatureCombination-r18      ReportedFeatureCombination-r18                   OPTIONAL,

startPreambleForThisPartition-r18     INTEGER (0..63),								  OPTIONAL,

numberOfPreamblesPerSSB-ForThisPartition-r18 INTEGER (1..64),						  OPTIONAL,

attemptedBWP-InfoList-r18            SEQUENCE (SIZE (1..maxNrofBWPs)) OF AttemptedBWP-Info-r18  OPTIONAL,

numberOfLBTFailures-r18              INTEGER (1..128)                                 OPTIONAL,

perRAInfoList-v1800                  PerRAInfoList-v1800                              OPTIONAL,

sdt-Failed-r18                       ENUMERATED {true}                                OPTIONAL

]]

}

RA-InformationCommon field descriptions

startPreambleForThisPartition This field indicates the first preamble of the used random-access resources associated with one or more features.

numberOfPreamblesPerSSB-ForThisPartition This field indicates the number of consecutive preambles of the used random-access resources associated with one or more features starting from the starting preamble(s) per SSB.