3GPP TSG-RAN WG2 Meeting #125	R2-2400330

Athens, Greece, 26th Feb. – 1st Mar., 2024

Title:		Clarification on the use of "if configured" in MAC spec

Source:	Huawei, HiSilicon

Agenda item:	7.0.4

Document for:	Discussion and Decision

1	Background

This paper discusses how to interpretate the expression of “if configured” that happens in a lot of places in the MAC spec.

2	Discussions

For the usage of “if configured” in the MAC spec, there are currently two cases:

- Case1: The “if configured” expression is used in the UE procedure. In this case, the UE should not apply the UE procedure, if the RRC field is not configured

- Case2: The “if configured” expression is sued in the condition for performing UE proceudre. In this case, the UE should consider the condition to be satisfied when the RRC field is not configured.

We think the usage of the “if configured” expression should be clarified for the above two cases.

Proposal: Clarify in the MAC spec the usage of “if configured” in the two cases: (a) when the “if configured” expression is used in the UE procedure; (b) when the “if configured” expression is used in the condition for a UE procedure. Agree to the CR in Annex A

3	Conclusions

The following proposal is made with the discussions above.

Proposal: Clarify in the MAC spec the usage of “if configured” in the two cases: (a) when the “if configured” expression is used in the UE procedure; (b) when the “if configured” expression is used in the condition for a UE procedure. Agree to the CR in Annex A

Annex A: CR

3GPP TSG-RAN2 Meeting #125	R2-240

# Athens, Greece, 26th Feb- 1st Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the expression of “if configured” in the MAC spec |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, NR_MT_SDT-Core |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the usage of “if configured” in the MAC spec, there are currently two cases: - Case1: The “if configured” expression is used in the UE procedure. In this case, the UE should not apply the UE procedure, if the RRC field is not configured - Case2: The “if configured” expression is sued in the condition for performing UE proceudre. In this case, the UE should consider the condition to be satisfied when the RRC field is not configured. We think the usage of the “if configured” expression should be clarified for the above two cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify on how the expression “if configured” is used |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of “if configured” is not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forward |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

============================CHANGE BEGINS==========================================

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, the expression "if configured" should be interpreted as follows:

If the expression "if configured" is included within a UE procedure, the UE shall apply the UE procedure only when the corresponding RRC field is configured;

If the expression "if configured" is included within a condition for the UE procedure, the condition is satisfied if the corresponding RRC field is not configured.

============================CHANGE ENDS==========================================