3GPP TSG-RAN WG2 Meeting #125	R2-2400339

Athens, Greece, 26th Feb. – 1st Mar., 2024

Title:		Remaining issues for IDLE/INACTIVE mode procedure

Source:	Huawei, HiSilicon

Agenda item:	7.2.8

Document for:	Discussion and Decision

- Background

With the completion of the R18 positioning and the publication of the R18 IDLE mode procedure spec, in this contribution, we intend to address the remaining issues. Text proposals are provided at the end for each proposal. Text proposals for IDLE mode procedure

During the offline discussion, the following two issues have been listed as open issue for IDLE mode procedure for positioning.

| Numbering | Section | Issue |
| --- | --- | --- |
| SL#01 | 8.2.1 | FFS the additional reselection process for selecting a cell on non-serving frequency for SL-PRS transmission. |
| SL#02 | 4.5 | Editor's NOTE: FFS the support for SL positioning in limited service state. |

2	Discussion

2.1	Parameters for cell selection for SL positioning

In the legacy spec, the following has been specified for the parameters for evaluating S criterion, R criterion or inter-frequency cell reselection criterion for SL communication/discovery:

### 8.2.1 Parameters used for cell selection and reselection triggered for sidelink When evaluating S criterion, R criterion (ranking) or inter-frequency cell reselection criterion, as defined in clause 5.2.3.2, clause 5.2.4.6 and clause 5.2.4.5 respectively, for cell selection/reselection triggered for NR sidelink communication/discovery or V2X sidelink communication on a non-serving frequency, UE shall perform the evaluation as follows: - The UE shall use cell selection/reselection parameters broadcast by the concerned cell (i.e. selected cell for the sidelink operation) for the evaluation.

For SL positioning, we think we should follow the same principle as in the legacy SL communications/discovery, that the cell selection/reselection parameters in the concerned cell selected for sidelink operations should be used for evaluation

Proposal1: Follow legacy SL communication/discovery for SL positioning: the cell selection/reselection parameters in the concerned cell selected for sidelink operations should be used for evaluation. Adopt the TP in Annex A.

2.2	SL positioning in limited-service state

In TS 38.304, the following has been defined for limited-service state as one of the service states for a UE in RRC_IDLE state.

## 4.3 Service types in RRC_IDLE state This clause defines the level of service that may be provided by the network to a UE in RRC_IDLE state. The following three levels of services are provided while a UE is in RRC_IDLE state: - Limited service (emergency calls, ETWS and CMAS on an acceptable cell); - Normal service (for public use or non-public use on a suitable cell); - Operator service (for operators only on a reserved cell). ## 4.4 Service types in RRC_INACTIVE state This clause defines the level of service that may be provided by the network to a UE in RRC_INACTIVE state. The following two levels of services are provided while a UE is in RRC_INACTIVE state: - Normal service (for public use or non-public use on a suitable cell); - Operator service (for operators only on a reserved cell).

Then, for SL communications and discovery, the following has been specified on its support in limited-service state

Following exception to these definitions are applicable for UEs: - if a UE has an ongoing emergency call, all acceptable cells of that PLMN/SNPN are treated as suitable for the duration of the emergency call. - camped on a cell that belongs to a tracking area that is forbidden for regional provision of service; a cell that belongs to a tracking area that is forbidden for regional provision service (TS 23.122 [9], TS 24.501 [14]) is suitable but provides only limited service. - if the UE in RRC_IDLE fulfils the conditions to support NR sidelink communication/discovery or V2X sidelink communication in limited service state as specified in TS23.287 [16] clause 5.7, the UE may perform NR sidelink communication/discovery or V2X sidelink communication. NOTE: UE is not required to support manual search and selection of PLMN or CAG or SNPN while in RRC CONNECTED state. The UE may use local release of RRC connection to perform manual search if it is not possible to perform the search while RRC connected.

In legacy SL communication/discovery, the following has been specified in the SA2 spec for the SL communications and discovery in the limited-service state, take ProSe for an example.

However, for SL positioning, to the best of our knowledge, there has been no related discussion in SA2 for its application in the limited-service state. Then, it would not be possible to support SL positioning in limited-service state.

Proposal2: Do not support SL positioning for UE in limited service state.

3	Conclusions

Proposal1: Follow legacy SL communication/discovery for SL positioning: the cell selection/reselection parameters in the concerned cell selected for sidelink operations should be used for evaluation. Adopt the TP in Annex A.

Proposal2: Do not support SL positioning for UE in limited service state.

Annex A

=================================TP BEGIN=======================================

## 8.2	Cell selection and reselection for Sidelink

The requirements defined in this clause for sidelink operation (including sidelink relay operations) apply for UEs in RRC_IDLE, RRC_INACTIVE and in RRC_CONNECTED.

When UE is interested to perform NR sidelink communication/discovery and ranging/sidelink positioning on non-serving frequency, it may perform measurements on that frequency or the frequencies which can provide inter carrier NR sidelink configuration for that frequency for cell selection and reselection purpose in accordance with TS 38.133[8]. When UE is interested to perform V2X sidelink communication on non-serving frequency, it may perform measurements on that frequency or the frequencies which can provide inter carrier V2X sidelink configuration for that frequency for cell selection and intra-frequency reselection purpose in accordance with TS 38.133[8].

If the UE detects at least one cell on the frequency which UE is configured to perform NR sidelink communication/discovery and ranging/sidelink positioning on fulfilling the S criterion in accordance with clause 8.2.1, it shall consider itself to be in-coverage for NR sidelink communication/discovery and ranging/sidelink positioning on that frequency. If the UE cannot detect any cell on that frequency meeting the S criterion, it shall consider itself to be out-of-coverage for NR sidelink communication/discovery and ranging/sidelink positioning on that frequency.

If the UE detects at least one cell on the frequency which UE is configured to perform V2X sidelink communication on fulfilling the S criterion in accordance with clause 8.2.1, it shall consider itself to be in-coverage for V2X sidelink communication on that frequency. If the UE cannot detect any cell on that frequency meeting the S criterion, it shall consider itself to be out-of-coverage for V2X sidelink communication on that frequency.

If the UE has selected a cell on a non-serving frequency for V2X sidelink communication, it shall perform additional intra-frequency reselection process to select a better cell for sidelink operation on that frequency in accordance with clause 8.2.1.

If the UE has selected a cell on a non-serving frequency for NR sidelink communication/discovery, it shall perform additional reselection process to select a better cell for sidelink operation in accordance with clause 8.2.1.

If the UE has selected a cell on a non-serving frequency for NR sidelink Positioning, it shall perform additional reselection process to select a better cell for sidelink operation in accordance with clause 8.2.1.

### 8.2.1	Parameters used for cell selection and reselection triggered for sidelink

When evaluating S criterion, R criterion (ranking) or inter-frequency cell reselection criterion, as defined in clause 5.2.3.2, clause 5.2.4.6 and clause 5.2.4.5 respectively, for cell selection/reselection triggered for NR sidelink communication/discovery or V2X sidelink communication or NR sidelink positioning on a non-serving frequency, UE shall perform the evaluation as follows:

-	The UE shall use cell selection/reselection parameters broadcast by the concerned cell (i.e. selected cell for the sidelink operation) for the evaluation.

====================================TP END======================================