3GPP TSG RAN WG2 #125												   R2-2400367

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda Item:	7.8.3

Source: 	NEC

Title:	Remaining issues on Flight Path Reporting

Document for:	Discussion

- Introduction

In RAN2#124 the below agreements were made [1]:

- Include FlightPathInfoReport in AS-Context within HandoverPreparationInformation (as already captured in the running CR). - UE initiate transmission of UAI again to the target cell if a UAI transmission was initiated during the last 1 second before receiving the reconfigurationWithSync. Rapporteur to check if there are spec changes. - RAN2 can confirm the value zero can be configured for flightPathUpdateDistanceThr and flightPathUpdateTimeThr, and if there is any location/time change in waypoints, the UE should to indicate the flight path update when the value zero is configured. - Flightpath update indication can be triggered due to adding a single waypoint. - Flightpath update indication can be triggered due to removal of a single future waypoint, except if it is removing an outdated waypoints.

This contribution provides our considerations on flightpath availability notification providing.

- Discussion

Based on current agreements, UE may provide flightpath availability notification in RRCReconfigurationComplete, current RRC spec captured it as following procedures:

…

2>	if the UE has flight path information available:

3>	if the UE had not previously provided a flight path information since last entering RRC_CONNECTED state; or

3>	if at least one waypoint was not previously provided; or

3>	if at least one upcoming waypoint that was previously provided is being removed; or

3>	if flightPathUpdateDistanceThr is configured and for at least one waypoint, the 3D distance between the previously provided location and the new location is more than or equal to the distance threshold configured by flightPathUpdateDistanceThr; or

3> if flightPathUpdateTimeThr is configured and for at least one waypoint, the timestamp was not previously provided but is now available, or the time between the previously provided timestamp and the new timestamp, if available, is more than or equal to the time threshold configured by flightPathUpdateTimeThr:

4>	include flightPathInfoAvailable;

…

Considering a UAV UE performing handover from the source gNB to the target gNB, if the UE had previously provided a flightpath in the source gNB (a last RRC_CONNECTED state) and it not changed based on above flightpath update threshold(s), the UE may not need to provide a flightpath availability notification in RRCReconfigurationComplete message.

UE may not provide a flightpath availability notification in RRCReconfigurationComplete message if UE had previously provided a flightpath in the source gNB.

For cases the source gNB include FlightPathInfoReport in AS-Context within HandoverPreparationInformation to the target gNB, the target gNB may has already got the latest flightpath.  However, the FlightPathInfoReport is an optional field in HandoverPreparationInformation, UE cannot assume the target gNB always get the flightpath during handover.  Therefore, our understanding is that UE do not need to provide flightpath availability notification in RRCReconfigurationComplete message during handover only if the target gNB has already got flightpath from the source gNB.

- Proposal 1: RAN2 to confirm that the UE should provide flightpath availability notification in RRCReconfigurationComplete message during handover unless the UE receives an indication that the target gNB already has the latest flightpath from the source gNB.

- Proposal 1a: If P1 is agreed, RAN2 to discuss whether such an indication is introduced or agree on “the UE provides flightpath availability notification in RRCReconfigurationComplete message during handover” as the expected behavior without introducing the indication.

- Conclusion

Observation 1:	UE may not provide a flightpath availability notification in RRCReconfigurationComplete message if UE had previously provided a flightpath in the source gNB.

Proposal 1: RAN2 to confirm that the UE should provide flightpath availability notification in RRCReconfigurationComplete message during handover unless the UE receives an indication that the target gNB already has the latest flightpath from the source gNB.

Proposal 1a: If P1 is agreed, RAN2 to discuss whether such an indication is introduced or agree on “the UE provides flightpath availability notification in RRCReconfigurationComplete message during handover” as the expected behavior without introducing the indication.

- References

[1] R2-23xxxxx, Report of 3GPP TSG RAN WG2 meeting #124, ETSI MCC.