3GPP TSG-RAN WG2 Meeting #125	R2-2400424

# Athens, Greece, 26 Feb - 1 Mar, 2024

Title: 	Discussion on UE capabilities for mobile IAB-MT

Source: 	ZTE, Sanechips

Agenda item:	7.12.2.2

Document for:	Discussion and Approval

Introduction

During RAN2#124meeting, UE capability for mobile IAB-MT was discussed and some updates are made to TS 38.306 to capture mandatory mobile IAB-MT features and optional features without signaling. However, there are still some open issues regarding mobile IAB-MT UE capabilities, e.g.,  whether to introduce a mobileIAB-r18 capability, whether to differentiate the mIAB RACH-less HO capability (/capabilities) from NTN RACH-less. In this contribution, we discuss the remaining open issues regarding mobile IAB-MT UE capabilities.

Discussion

According to the open issue list provided by the CR Rapporteur, the following two issues on mobile IAB-related UE capabilities need to be further discussed:

- Issue 1: Whether we introduce a mobileIAB-r18 capability (i.e. to identify an IAB-MT as mobile IAB-capable)

- Issue 2:  Whether to differentiate the mIAB RACH-less HO capability (/capabilities) from NTN RACH-less

Issue 1: whether to introduce a mobileIAB-r18 capability

During previous meetings, it was discussed whether to introduce a separate mobile-IAB capability in R18 to let the donor know the capability of the mobile IAB-MT and no conclusion is achieved. The first motivation to introduce the mobile-IAB capability is that the capability indication is needed since the target donor needs to be aware of mobile IAB capability during MT HO, and RAN2 agreed that UE capability signalling is the baseline to let CU know that the MT is a “mobile-IAB” type. However, RAN3 agreed to introduce mobile IAB authorization status in the Xn Handover request message during RAN3#121bis meeting, which has two codepoints and could be used to indicate the capability of mobile IAB node implicitly. So there is no need for the separate mobile-IAB capability for this motivation any more.

Observation 1: RAN3 agreed to introduce mobile IAB authorization status in the Xn Handover request message during RAN3#121bis meeting, which has two codepoints and could be used to indicate the capability of mobile IAB node implicitly.

The second motivation to introduce the mobile-IAB capability is that the mobile IAB indication in msg 5 is optional for mobile IAB node, e.g., it is not included if the mobile IAB node doesn’t operate as a MBSR since it’s not authorized. In this situation, if the mobile IAB indication is not included, the donor is not aware of the mobile IAB capability and may select a AMF which doesn’t support mobile IAB for the mobile IAB node. Then if the authorization status changes from “not authorized” to “authorized”, the update of the authorization status cannot be notified from the AMF to the donor. However, this issue has been resolved by SA2 and it is captured in TS 23.501 that if the MBSR node is not authorized, e.g. due to the MBSR authorization indication from AMF, it also provides the mobile IAB-indication when establishing new RRC connection, as copied in the below. In this circumstans, the mobile IAB node indication can serve as a capability indication sent from mobile IAB node to the donor somehow.

23.501 For a MBSR node to operate as a MBSR, it provides a mobile IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports mobile IAB-node and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform MBSR authorization as described in clause 5.35A.4. If the MBSR node is not authorized, e.g. due to the MBSR authorization indication from AMF, it also provides the mobile IAB-indication when establishing new RRC connection so that the AMF supporting mobile IAB-node will be selected by the IAB-donor-CU, to ensure that the operation related to MBSR authorization status change for a registered MBSR node can be performed as described in clause 5.35A.4. If the MBSR receives MBSR authorized indication from AMF, it provides the information about the authorization result to its IAB-DU component based on non-standardized interface as described in clause 5.35A.4.

Obsevation 2: According to TS 23.501, if the MBSR node is not authorized, e.g. due to the MBSR authorization indication from AMF, it also provides the mobile IAB-indication when establishing new RRC connection.

On the other hand, a mobile IAB node may also support IAB and may intend to operate as R16/17 IAB node in some cases. As agreed in RAN2#124 meeting, RAN2 assumes that the IAB-node only indicates either mobile IAB or Rel-16/17 IAB for MSG5, not both. If a mobile IAB node intends to operate as a R16/17 IAB node, it will only include IAB node indication in msg 5, i.e. mobile IAB indication is not included. In this situation, the donor node cannot derive the mobile IAB capability according to the mobile IAB indication in msg 5. However, we think there is no issue here since the donor node doesn’t need to know the MT has mobile IAB capability if it intends to operate as an Rel-16/17 IAB node. If the mobile IAB node intends to operate as a mobile IAB node later, it can deregister and re-register to the network again and includes mobile IAB node indication in msg 5.

R2 assumes that the device can know whether it is intended to operate as R18 mIAB or R16/17-IAB node, (how the device knows is outside R2 scope, e.g. subscription, device internal param etc), the MSG5 indication is an indication of this intended mode of operation. This agreement is not intended to mandate that a mIAB node must support R16/17 operation (FFS pending cap discussion) R2 assumes that the IAB-node only indicates either mobile IAB or Rel-16/17 IAB for MSG5, not both.

Observation 3: The donor node doesn’t need to know the MT has mobile IAB capability if it intends to operate as an Rel-16/17 IAB node.

As analyzed above, there is no motivation to introduce a separate mobile IAB capability in R18.

Proposal 1: Not to introduce a separate mobile IAB capability in R18.

Issue 2:  Whether to differentiate the mIAB RACH-less HO capability (/capabilities) from NTN RACH-less

Another issue is that whether to introduce separate capability for RACH-less HO for mIAB and NTN. Currently a new UE Capability rachLessHandoverNTN-r18 is defined to indicate UE support of RACH-less HO in NTN.  As we know, the overall procedure for RACH-less HO in NTN is extended to mobile IAB. And several aspects are identified to be different between RACH-less HO support in NTN and mIAB scenarios as summarized in R2-2313962 copied in the below.

R2-2313962 Several aspects of RACH-less handover are differentiated between NR NTN and mIAB features (which may be captured in other specifications). These include: - mIAB does not support setting NTA value to 0 within RACH-less handover command. - The beam information for monitoring PDCCH for dynamic grant for initial UL transmission is provided: - As an SSB index for NTN - As a TCI state ID for mIAB - For mIAB, support for perfoming initial UL transmission in RACH-less handover is optional, and separates capabilities are introduced for dynamic grant and configured grant. - mIAB continuously monitors PDCCH (i.e., ignores DRX and measurement gap configurations) during an ongoing RACH-less HO procedure. NTN follows existing (e.g., DRX) configurations during an ongoing RACH-less HO procedure.

For the UL grant used by the UE to transmit the RRCReconfigurationComplete, both configured grant and dynamic grant are supported. And there was an observation in RAN2#123bis that for mIAB, the network can always provide a beam indication. During RAN2#124 meeting, it was agreed that CG RACH less is not assumed to be important for mIAB, and CG RACH less and DG RACH less are separate UE caps.

RAN2#124 meeting agreements: 1: CG RACH less and DG RACH less are separate UE caps 2: CG RACH less is not assumed to be important for IAB and need not to be optimized for the IAB scenario (but also no strict need to prohibit).

As we can see, there are several aspects which are different in supporting RACH-less HO between mIAB and NTN. So we think it’s more clear to use different capabilities for RACH-less HO in these two scenarios. And two separate capabilities can be introduced to indicate the capability of CG and DG RACH less, e.g., rachLessHandovermIAB-cg-r18 and rachLessHandovermIAB-cg-r18 are introduced as a part of IE RF-Parameters.

Observation 4: Several aspects in supporting RACH-less HO are identified to be different between NTN and mIAB scenarios.

Proposal 2: Two separate capabilities (e.g., rachLessHandovermIAB-cg-r18 and rachLessHandovermIAB-cg-r18) are introduced to indicate the capability of supporting CG and DG RACH-less HO in mobile IAB.

Conclusion

In this contribution, we discussed the remaining open issues regarding mobile IAB-MT UE capabilities. The following observations and proposals are given:

Observation 1: RAN3 agreed to introduce mobile IAB authorization status in the Xn Handover request message during RAN3#121bis meeting, which has two codepoints and could be used to indicate the capability of mobile IAB node implicitly.

Obsevation 2: According to TS 23.501, if the MBSR node is not authorized, e.g. due to the MBSR authorization indication from AMF, it also provides the mobile IAB-indication when establishing new RRC connection.

Observation 3: The donor node doesn’t need to know the MT has mobile IAB capability if it intends to operate as an Rel-16/17 IAB node.

Observation 4: Several aspects in supporting RACH-less HO are identified to be different between NTN and mIAB scenarios.

Proposal 1: Not to introduce a separate mobile IAB capability in R18.

Proposal 2: Two separate capabilities (e.g., rachLessHandovermIAB-cg-r18 and rachLessHandovermIAB-cg-r18) are introduced to indicate the capability of supporting CG and DG RACH-less HO in mobile IAB.

Reference