3GPP TSG-RAN WG2 Meeting #125                                                               R2-2400435

Athens, Greece, Februrary 26 – March 1, 2024

Agenda item: 		7.12.2.2

Source: 		Qualcomm Incorporated

Title: 	Remaining issues for UE capabilities for mobile IAB

Document for:	Discussion

# 1	Introduction

This contribution discussed remaining issue related to UE capabilities for mobile IAB.

# 2	Discussion

## 2.1	mIAB-specific capabilities of mIAB-MT

TS 38.306, section 4.2.15.1a presently captures all aspects related to mobile IAB, which are:

Mobile IAB-MT shall apply the same capabilities as IAB-MT unless indicated otherwise. In addition, it is mandatory for mobile IAB-MT to support the following features: - Acquisition of gNB-ID-Length from SIB1, as specified in TS 38.331 [9]. - Cell barring based on mobileIAB-Support, as specified in TS 38.331 [9]. - Inclusion of mobileIAB-NodeIndication, as specified in TS 38.331 [9].

This list of mIAB-specific mandatory parameters of the mIAB-MT is complete. No further mIAB-specific parameters need to be defined for the mIAB-MT.

Observation 1: The list of mIAB-specific mandatory parameters of the mIAB-MT presently captured in TS 38.306 is complete.

TS 38.306 further defines a long list of IAB-specific optional parameters for the IAB-MT. Several of these parameters do not apply to the mIAB-MT. However, presently, there is no indication that these paramters do not apply to the mIAB-MT.

Observation 2: TS 38.306 includes multiple IAB-specific optional parameters, which do not apply to the mIAB-MT, but which are not indicated as such.

This applies to the following parameters:

| Table | Parameters | Reasons why parameters do not apply to mIAB |
| --- | --- | --- |
| 4.2.15.5 BAP Parameters | bapHeaderRewriting-Rerouting-r17 Indicates whether the IAB-MT supports BAP header rewriting for inter-donor-DU re-routing, as specified in TS 38.340 [23] and TS 38.300 [28]. IAB-donor-DUs can belong to the same or different IAB-donor CUs. | Only applies to the boundary node. The mIAB-node is not considered a boundary node. |
| 4.2.15.5 BAP Parameters | bapHeaderRewriting-Routing-r17 Indicates whether the IAB-MT supports BAP header rewriting for inter-donor CU partial migration, inter-donor-CU RLF recovery and inter-donor-CU topology redundancy, as specified in TS 38.340 [23] and TS 38.300 [28]. | Only applies to the boundary node. The mIAB-node is not considered a boundary node. |
| 4.2.15.7.2 Phy-Parameters | directionalCollisionDC-IAB-r17 Indicates the support for directional collision handling between MCG and SCG cell(s) of the dual parent nodes for simultaneous operation in inter-donor and/or intra-donor DC operation. | DC is not supported for mIAB-MT. |
| 4.2.15.9 MR-DC Parameters | All parameters in table | DC is not supported for mIAB-MT. |
| 4.2.15.10 NRDC Parameters | All parameters in table | DC is not supported for mIAB-MT. |

These parameters should be indicated with “not applicable to mobile IAB”.

Proposal 1: The following IAB-specific optional parameters for the IAB-MT to be indicated with “non applicable to mobile IAB”:

- 4.2.15.5  	BAP Parameters: bapHeaderRewriting-Rerouting-r17

- 4.2.15.5  	BAP Parameters: bapHeaderRewriting-Routing-r17

- 4.2.15.7.2	Phy-Parameters: directionalCollisionDC-IAB-r17

- 4.2.15.9	MR-DC Parameters: entire table

- 4.2.15.10	NRDC Parameters: entire table.

RAN2 further discussed, whether there should be a mobile-IAB capability parameter. This parameter is not necessary since:

- In case the mIAB-MT wants to use mobile IAB, it has to include the mobile IAB indicator into MSG 5. The IAB-donor-CU therefore knows about the mIAB-MT’s mobile-IAB support.

- When the mIAB-MT hands over to another IAB-donor-CU, the mobile-IAB authorization status is included in the Handover Request message. The target IAB-donor-CU therefore also knows about mobile-IAB support.

Proposal 2: There is no need to introduce a “Mobile-IAB” capability.

## 2.2 	mIAB-specific capabilities for UEs

The Mobile IAB WI introduced RACH-less handover for DU migration as well as mIAB-specific inter-frequency cell-reselection prioritization. Both of these features should be optional and reported in UE capabilities. These parameters can be added to table 4.2.9 MeasAndMobParameters.

Proposal 3: RACH-less handover and mIAB-specific inter-frequency cell-reselection prioritization to be reported as optional capabilities for the UE. These parameters can be added to table 4.2.9 MeasAndMobParameters.

RAN2 needs to discuss whether RACH-less handover for mobile IAB, LTM and NTN are captured as one common or as separate capabilities.

Proposal 4: RAN2 to discuss whether RACH-less handover for mobile IAB, LTM and NTN are captured as one common or as separate capabilities.

# Conclusion

This paper discussed remaining issues related to UE capabilities for mobile IAB. The following observations and proposals were made:

Observation 1: The list of mIAB-specific mandatory parameters of the mIAB-MT presently captured in TS 38.306 is complete.

Observation 2: TS 38.306 includes multiple IAB-specific optional parameters, which do not apply to the mIAB-MT, but which are not indicated as such.

Proposal 1: The following IAB-specific optional parameters for the IAB-MT to be indicated with “non applicable to mobile IAB”:

- 4.2.15.5  	BAP Parameters: bapHeaderRewriting-Rerouting-r17

- 4.2.15.5  	BAP Parameters: bapHeaderRewriting-Routing-r17

- 4.2.15.7.2	Phy-Parameters: directionalCollisionDC-IAB-r17

- 4.2.15.9	MR-DC Parameters: entire table

- 4.2.15.10	NRDC Parameters: entire table.

Proposal 2: There is no need to introduce a “Mobile-IAB” capability.

Proposal 3: RACH-less handover and mIAB-specific inter-frequency cell-reselection prioritization to be reported as optional capabilities for the UE. These parameters can be added to table 4.2.9 MeasAndMobParameters.

Proposal 4: RAN2 to discuss whether RACH-less handover for mobile IAB, LTM and NTN are captured as one common or as separate capabilities.