3GPP TSG-RAN WG2 Meeting #125		R2-2400451

Athens, Greece, 26th Feb. – 1st Mar. 2024

Source:	vivo

Title:	Discussion on remaining issues and corrections for DSR

Agenda Item:	7.5.3.2

Document for:	Discussion and Decision

- Introduction

In RAN2#124 [1], there were the following agreements regarding DSR (non-relevant bullets have been removed):

Agreements on BSR/DSR - The DSR MAC CE uses one-octet eLCID - The DSR MAC CE has a logical channel priority lower than the Timing Advanced Report and higher than the SL-BSR (prioritized). - Dynamic indication of BSR table in the DSR MAC CE is supported. Network can RRC configure whether LCG can use new BSR table for BSR. If the network configures new table for BSR the UE uses new table for DSR MAC CE. If configured, DSR MAC CE includes indicators on which BSR table is used to report data volume of an LCG. The same principles for BSR are used to determine whether legacy or new table is included. - Remaining time field range is 1 to 64 and 6 bits in DSR MAC CE. Update value in RRC to align.

Agreements: - No dedicated SR configuration for DSR will be introduced, we use the same SR configuration for BSR - UE triggers SR after a DSR is triggered, if there is no PUSCH available to send the DSR MAC CE and there is no pending SR already triggered for this LCH. - If a MAC PDU is large enough to include all PDUs/SDUs within the triggering threshold from the LCGs that have pending DSRs, the UE shall not include DSR MAC CE in the MAC PDU - A pending DSR is cancelled if all the data within the triggering threshold is discarded or transmitted - DSR with a remaining time value zero ms for all LCG is not transmitted. The shortest non-zero remaining time is reported for a LCG. - UE cancels all pending DSRs upon MAC reset

In this paper, we will discuss:

- How non-zero remaining time can be better captured in MAC.

- Whether padding DSR can be supported

- When a DSR triggers SR, whether conditions similar to those for BSR triggering SR be applied too?

- Discussion

  - Non-zero remaining time report

In RAN2#124, it was already agreed that the UE should report the smallest non-zero remaining time of the packets for one LCG whose delay status is to be reported. To facilitate the discussion, the relevant MAC and RLC specification have been copied below for reference:

Relevant part in 3GPP TS 38.321 v18.0.0:

### 5.4.9 Delay status reporting The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the PDCP discardTimers among SDUs buffered for the LCG as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.6 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively. RRC controls the DSR procedure by configuring the following parameter: - remainingTimeThreshold: the threshold on remaining time for triggering a DSR for an LCG. If an LCG is configured for delay status reporting, the MAC entity shall: 1> if the smallest remaining value of the PDCP discardTimers among all the data buffered for the LCG that has not been transmitted in any MAC PDU or reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the LCG since the last transmission of a DSR MAC CE: 2> trigger a DSR for the LCG. If there is at least one DSR pending, the MAC entity shall: 1> if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization: 2> instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE; 1> else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR: 2> trigger a Scheduling Request. NOTE: The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5. An SDU is considered to be associated with a DSR if it is associated with the LCG which triggered the DSR and the remaining value of its PDCP discardTimer is below remainingTimeThreshold. A MAC PDU shall contain at most one DSR MAC CE. The MAC entity shall not include a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate the SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, either when all the SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes either all the SDUs associated with the DSR or a DSR MAC CE that contains the delay information of all the SDUs associated with the DSR (as described in the clause 6.1.3.72). --------------------------------------------------non-related part omitted---------------------------------------------------- #### 6.1.3.72 Delay Status Report MAC CE The Delay Status Report (DSR) MAC CE is identified by MAC subheader with an eLCID as specified in Table 6.2.1-2b. The fields in the DSR MAC CE are defined as follows: - LCGi: This field indicates the presence of delay information (i.e. the Remaining Time and Buffer Size fields) for the LCG i. The LCGi field set to 1 indicates that the delay information for the LCG i is reported. The LCGi field set to 0 indicates that the delay information for the LCG i is not reported; - Remaining Time: This field indicates the shortest remaining value of PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs buffered for an LCG, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. The value r in this field indicates a remaining time within the range of (r, r + 1] msec; ----------------------------------------------------non-related part omitted---------------------------------------------------

Relevant part in 3GPP TS 38.323 v18.0.0:

#### 3.1 Definitions For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1]. AM DRB: a data radio bearer which utilizes RLC AM. AM MRB: an MRB associated with at least one AM RLC bearer for PTP transmission. Broadcast MRB: a radio bearer configured for MBS broadcast delivery. DAPS bearer: a bearer whose radio protocols are located in both the source gNB and the target gNB during DAPS handover to use both source gNB and target gNB resources. Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold. ----------------------------------------------------non-related part omitted---------------------------------------------------

According the above agreements, the UE should report non-zero remaining time. However, according the current MAC spec and PDCP spec, the UE may derive the smallest remaining time for one LCG without considering whether the remaining time is zero or not. In the DSR MAC CE part, it was defined that the remaining time should be within (r, r+1] ms. It results in that if the UE derive the smallest remaining time zero it cannot fill it into the DSR MAC CE.

- It was agreed that the UE should report non-zero remaining time in DSR.

- According to MAC spec (v18.0.0) and PDCP spec (v18.0.0), it can happen that the UE may determine the smallest remaining time value 0 for one LCG but the remaining time value 0 cannot be filled into the delay field ((r, r + 1] ms) of DSR MAC CE.

To conquer the twisted behavior between DSR triggering and DSR generation, the UE behavior to derive the smallest remaining time should be clarified. One proper way for the clarification is that the UE should derive the smallest remaining time value based on the packets with running discardTimers, which could imply that the remaining time is non-zero.

- For DSR report for an LCG, the UE should determine the smallest remaining time among the packets with running discardTimers for the LCG.

The corresponding TP for the Proposal 1 is provided the Annex.

- If Proposal 1 is agreeable, RAN2 to adopt the TP in the Annex.

  - Padding DSR

Currently, if there is still room available in the MAC PDU after including data into the MAC PDU, the UE may include padding BSR into the MAC PDU to update the buffer status at the gNB side. The similar logic can be reused for DSR. If there is still room available and there are delay-critical data at the UE side, the UE should be allowed to send padding DSR to the gNB. The padding DSR can be used to update the delay status information to the gNB so that the gNB can arrange proper resource allocation for the remaining delay-critical data at the UE.

- If there is still capacity left in the MAC PDU after data filling, the UE should be allowed to use the left capacity for padding DSR transmission.

If padding DSR is supported, truncated DSR MAC CE format should be defined so that the most important delay status can be reported if the available room is not enough to carry the delay information for all LCGs with delay-critical data. In this case, the delay status for high priority LCGs with delay critical data should be prioritized to be reported.

- If Proposal 3 is agreeable, Truncated DSR MAC CE format should be defined for padding DSR transmission.

It could happen that the UE has both padding BSR and padding DSR, but the available room in the MAC PDU is not enough to accommodate both. Then there is an issue on whether the UE should prioritize padding DSR or padding BSR. This issue can be left for UE implementation.

- When the available compacity is only enough for one of padding BSR and padding DSR, whether to transmit padding BSR or padding DSR can be left for UE implementation.

  - Transmission restriction for DSR triggered SR

In the current MAC specification, there are a few timers which are defined to avoid frequently SR transmission triggered by BSR:

-	logicalChannelSR-DelayTimerApplied;

-	logicalChannelSR-DelayTimer;

-	logicalChannelSR-Mask;

-	sr-ProhibitTimer (per SR configuration);

These parameters are defined to avoid frequent SR transmission for BSR. For DSR, the DSR transmit latency is the critical issue. Hence, the SR mask or SR DelayTimer for SR transmission triggered by BSR should not be applied for DSR.

- Delay of SR transmission triggered by DSR may result in delay critical data loss.

### 5.4.9 Delay status reporting -------------------------------------------non-related are omitted---------------------------------------------------- If there is at least one DSR pending, the MAC entity shall: 1> if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization: 2> instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE; 1> else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR: 2> trigger a Scheduling Request. NOTE: The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5. -------------------------------------------non-related are omitted----------------------------------------------------

- In the current MAC spec (v18.0.0), DSR can already trigger SR transmission directly without checking logicalChannelSR-DelayTimerApplied, logicalChannelSR-DelayTimer and logicalChannelSR-Mask.

- SR transmission triggered by DSR should not be restricted by logicalChannelSR-DelayTimerApplied, logicalChannelSR-DelayTimer and logicalChannelSR-Mask, and no specification change is needed.

- Conclusion

In this contribution, we have discussed the remaining issues for DSR transmission. Based on the above discussions, we have the following observations:

Observation 1	It was agreed that the UE should report non-zero remaining time in DSR.

Observation 2	According to MAC spec (v18.0.0) and PDCP spec (v18.0.0), it can happen that the UE may determine the smallest remaining time value 0 for one LCG but the remaining time value 0 cannot be filled into the delay field ((r, r + 1] ms) of DSR MAC CE.

Observation 3	Delay of SR transmission triggered by DSR may result in delay critical data loss.

Observation 4	In the current MAC spec (v18.0.0), DSR can already trigger SR transmission directly without checking logicalChannelSR-DelayTimerApplied, logicalChannelSR-DelayTimer and logicalChannelSR-Mask.

Based on the discussions and the observations, we have the following proposals:

Proposal 1	For DSR report for an LCG, the UE should determine the smallest remaining time among the packets with running discardTimers for the LCG.

Proposal 2	If Proposal 1 is agreeable, RAN2 to adopt the TP in the Annex.

Proposal 3	If there is still capacity left in the MAC PDU after data filling, the UE should be allowed to use the left capacity for padding DSR transmission.

Proposal 4	If Proposal 3 is agreeable, Truncated DSR MAC CE format should be defined for padding DSR transmission.

Proposal 5	When the available compacity is only enough for one of padding BSR and padding DSR, whether to transmit padding BSR or padding DSR can be left for UE implementation.

Proposal 6	SR transmission triggered by DSR should not be restricted by logicalChannelSR-DelayTimerApplied, logicalChannelSR-DelayTimer and logicalChannelSR-Mask, and no specification change is needed.

- References

- RAN2#124, Chairman notes.

- Annex – TP for DSR based on the non-zero remaining time

Start of change on TS 38.321

### 5.4.9	Delay status reporting

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among SDUs buffered for the LCG as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.6 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold: the threshold on remaining time for triggering a DSR for an LCG.

If an LCG is configured for delay status reporting, the MAC entity shall:

1>	if the smallest remaining value of the running PDCP discardTimers among all the data buffered for the LCG that has not been transmitted in any MAC PDU or reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the LCG since the last transmission of a DSR MAC CE:

2>	trigger a DSR for the LCG.

If there is at least one DSR pending, the MAC entity shall:

1>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE;

1>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

2>	trigger a Scheduling Request.

NOTE:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same critieria specified in clause 5.4.5.

An SDU is considered to be associated with a DSR if it is associated with the LCG which triggered the DSR and the remaining value of its PDCP discardTimer is below remainingTimeThreshold.

A MAC PDU shall contain at most one DSR MAC CE. The MAC entity shall not include a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate the SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, either when all the SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes either all the SDUs associated with the DSR or a DSR MAC CE that contains the delay information of all the SDUs associated with the DSR (as described in the clause 6.1.3.72).

End of change on TS 38.321