3GPP TSG-RAN WG2 Meeting #125	R2-2400484

Athens, Greece,  Feb. 26th – Mar. 1st, 2024

Agenda Item:	7.3.2

Source:	OPPO

Title:	Open issue on the cellDTRX-RNTI monitoring

Document for:	Discussion, Decision

Introduction

This paper will discuss the cellDTRX-RNTI (i.e. NES-RNTI) monitoring.

Discussion

In Rel-18 NES, it was agreed to use DCI format 2_9 to activate/de-activate the cell DTX/DRX as well as provide the NES-mode indication. Such a DCI format is addressed to cellDTRX-RNTI and would be monitored at least during active periods of C-DRX.

- RAN1 agreements

Agreement UE is expected to monitor DCI format 2_9 during active periods of C-DRX Conclusion There is no consensus in RAN1 on whether or not the UE is expected to monitor DCI format 2_9 during non-active periods on C-DRX

- RAN2 agreements

Agreements: 1. RAN2 will capture the NES-RNTI monitoring behavior in February meeting (once discussion is finalized)

The remaining issue herein is whether the UE is expected to monitor DCI format 2_9 during non-active periods of C-DRX. If the answer is negative, the delayed network signalling and the corresponding unexpected UE behaviour will be introduced. However, it is not an NES-specific result. In legacy, the UE is also not expected to monitor the group-common DCI format transmitted through Type-3 CSS during the non-active period of C-DRX, e.g. DCI format 2_0. The intention is to save the UE’s power consumption. In our view, it seems good to reuse the legacy monitoring rule to DCI format 2_9 since it is also transmitted through Type-3 CSS. Thus, we suggest to conclude that the UE is not expected to monitor DCI format 2_9 during the non-active period of C-DRX.

The UE is not expected to monitor DCI format 2_9 during non-active periods on C-DRX.

Conclusion

We have the following proposals:

Proposal 1	The UE is not expected to monitor DCI format 2_9 during non-active periods on C-DRX.

Text proposal

- 5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI and cellDTRX-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

Reference

[1] RP-223540 New WID: Network energy savings for NR

[2] TR 38.864 Study on network energy savings for NR