3GPP TSG-RAN WG2 Meeting #125	R2-2400486

Athens, Greece,  Feb. 26th – Mar. 1st, 2024

Agenda Item:	7.3.2

Source:	OPPO

Title:	Discussion on remaining issues for the emergency call

Document for:	Discussion, Decision

Introduction

This paper will discuss the remaining issues for the emergency call.

Discussion

In the previous meetings, it was agreed that the UE triggers an RA procedure once an emergency call is initiated during the cell DRX non-active period.

#123bis Agreements Working assumption: UE triggers RACH upon determining that an emergency call is initiated during the cell DTX/DRX non active period. We rely on the UE implementation to determine whether an emergency call is initiated. We will take time to check until next meeting to confirm the WA. #124 Agreements 1. Confirm WA emergency call: UE triggers RACH upon determining that an emergency call is initiated during the cell DTX/DRX non active period 2. In running MAC CR, capture a NOTE similar to section 5.3.13.2 of TS 38.331 (i.e., “NOTE: How the MAC layer in the UE is aware of an ongoing emergency service is up to UE implementation.”) 3. No need to explicitly specify that the UE keeps monitoring PDCCH for followed transmission after successful completion of RA, i.e., it is left to NW implementation to complete followed transmission (e.g., emergency call) after RA (e.g., initiate followed transmission when the retransmission timer is running)

The below text in the current MAC spec intends to capture the above agreements.

- 5.34.3 Cell Discontinuous Reception [omitted] 1> if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period: 2> not instruct the physical layer to signal a SR on a PUCCH resource for SR; 2> not increment the SR_COUNTER for a SR; 2> not start the sr-ProhibitTimer for a SR; 2> not deliver any configured uplink grant and the associated HARQ information to the HARQ entity; 2> not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission; 2> not report periodic CSI on PUCCH and semi-persistent CSI configured on PUSCH; 2> if an emergency service is initiated by upper layers and this Serving Cell is the SpCell: 3> initiate a Random Access procedure (as specified in clause 5.1.1).

In our view, the current text is not fully correct in the CA case. As an example, the UE can also be configured with other serving cell(s), on which cell DRX is de-configured/deactivated or is in the active period. If that is the case, the UE can directly communicate with the network via the available uplink resource on that serving cell without the RA triggering. Thus, we think it is necessary to have further restrictions.

UE triggers RACH on the SpCell for an emergency call if no available uplink grant transmission is allowed on any serving cell of the UE based on the cell DRX mechanism.

Conclusion

We have the following proposals:

Proposal 1	UE triggers RACH on the SpCell for an emergency call if no available uplink grant transmission is allowed on any serving cell of the UE based on the cell DRX mechanism.

Text Proposal

- 5.34.3	Cell Discontinuous Reception

Cell DRX is configured if cellDTXDRXconfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring CellDTXDRX-Config by upper layers: if cell DRX is configured and cellDTXDRXactivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and cellDTXDRXactivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if CellDTXDRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	celldtxdrx-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DRX, the MAC entity shall:

1>	if cell DRX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (celldtxdrx-Cycle) = (celldtxdrx-StartOffset):

3>	start celldtxdrx-onDurationTimer for this serving cell after celldtxdrx-SlotOffset from the beginning of the subframe.

1>	if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period:

2>	not instruct the physical layer to signal a SR on a PUCCH resource for SR;

2>	not increment the SR_COUNTER for a SR;

2>	not start the sr-ProhibitTimer for a SR;

2>	not deliver any configured uplink grant and the associated HARQ information to the HARQ entity;

2>	not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission;

2>	not report periodic CSI on PUCCH and semi-persistent CSI configured on PUSCH;

2>	if an emergency service is initiated by upper layers and this Serving Cell is the SpCell; and

2>	if there is no available UL-SCH resource on any other serving cell, if any, on which cell DRX is not configured, deactivated, or in the Active Period:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

Reference

[1] RP-223540 New WID: Network energy savings for NR

[2] TR 38.864 Study on network energy savings for NR