3GPP TSG-RAN WG2 Meeting #125	R2-2400582

Athens, Greece, 26 February – 01 March 2024

Agenda item:	    7.24.2.1

Source:	Vodafone, Verizon, Apple

Title:	Support of initial access for 2 RX XR devices in Legacy Networks

Document for:	Discussion and Decision

# 1	Introduction

During the RAN Plenary RAN#102 the following agreements were done:

Task RAN2 and RAN3 to develop signalling support for ‘2Rx non-REDCAP XR devices’, send corresponding Release-18 draft CR(s) to RAN#103:

- A new dedicated UE capability indication per band and setting of corresponding existing UE capability(ies). [RAN2]
Note: no need is foreseen to add an indication to Msg1 or Msg3.

- Indication in SIB, to be used to re-direct to another frequency layer, or to bar the device altogether. [RAN2]

The aim of this paper is to discuss how the barring of such devices should be designed.

# 2	Scenarios for barring support

Scenario 1: 2Rx non-REDCAP XR devices roam into a legacy network (e.g. a Network not supporting any 2Rx non-REDCAP XR functionality) .

The deployment of 2Rx non-REDCAP XR devices is expected to have an impact to network capacity and the UE’s coverage and the operator should have means to provide guidance and restrictions to such devices in that cell. At the same time, there are no technical radio related reasons why a legacy RAN could not support access by 2Rx non-REDCAP XR devices (which is different e.g. to RedCAPs devices where the legacy network would not be able to serve such devices due to the limited bandwidth of RedCAP devices) and therefore we believe that the access of 2Rx non-REDCAP XR devices should be allowed from AS point of view as default behaviour

Proposal 1: In legacy NWs, 2Rx non-REDCAP XR devices are supported from AS point of view and the absence of any 2Rx non-REDCAP XR specific Barring IEs means that 2Rx non-REDCAP XR are allowed from AS point of view.

In general, the restrictions for 2Rx non-REDCAP XR to use services in a particular area/cell/frequency should be provided based on subscription, using e.g. corresponding NAS reject cause values. The use of NAS signalling gives greater flexibility in moving the UE to another RAT or PLMN and for a longer period than with AS signalling. but to realise this scenario, RAN networks might need to be updated.

Scenario 2: 2Rx non-REDCAP XR in release 18 Network supporting  2Rx non-REDCAP XR restrictions.

As per RAN Plenary guidance, the operator deploying release 18 network should be able to bar 2Rx devices if it wishes to do so and therefore, there is a need to introduce a similar signalling as for RedCAP devices (e.g. Cellbarred2RxNon-REDCAPXR)

Proposal 2: It is proposed to introduce Cellbarred2RxNon-REDCAPXR to restrict the access of 2Rx non-REDCAP XR to that cell in release 18.

If such an indication is introduced it is important to understand what are the actions the UE will perform once receiving the indication. There are several alternatives:

- The UE will bar the cell completely and try to find other cell using (possibly using Cellbarred2RxNon-REDCAPXR specific bits similar to what we have in “intraFreqReselection” or “eRedCapAccessAllowed” in SIB4)

- The UE will bar the cell if indicated by Cellbarred2RxNon-REDCAPXR, but still can use it for e,g, emergency calls.

Proposal 3: Emergency calls should be allowed on cells barred with 2Rx non-REDCAP XR

Proposal 4: For stage 3, it is proposed to use CRs in R2-2400935, R2-2400934, R2-2401454 as starting point for discussion and agreements

# Conclusion

It is proposed to agree on the following proposals:

Proposal 1: In legacy NWs, 2Rx non-REDCAP XR devices are supported from AS point of view and the absence of any 2Rx non-REDCAP XR Barring IEs means that 2Rx non-REDCAP XR are allowed in the NW from AS point of view.

Proposal 2: It is proposed to introduce Cellbarred2RxNon-REDCAPXR to restrict the access of 2Rx non-REDCAP XR in Rel 18.

Proposal 3: Emergency calls should be allowed on cells barred with 2Rx non-REDCAP XR

Proposal 4: For stage 3, it is proposed to use CRs in R2-2400935, R2-2400934, R2-2401454 as starting point for discussion and agreements.