3GPP TSG-RAN WG2 #125	R2-2400605

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Source:		NEC

Agenda item:	7.17.2	RRC

Title:	Remaining issue of MUSIM temporary capability restriction

Document for:		Discussion and decision

- Introduction

In this paper, we provide our view on the following remaining RRC issue of MUSIM temporary capability restriction listed by the rapporteur.

Issue 2: UAI/Early indication processing during handover procedure

- Discussion

For other UE assistance information in UAI, when performing handover procedure, the source gNB would transmit the latest UE assistance information in UAI to the target gNB by inter-node RRC message HandoverPreparationInformation. For UAI for MUSIM temporary capability restriction, the same can be applied. And if the target gNB also supports MUSIM temporary capability restriction, it will prepare the HO command by considering the UAI for temporary capability restriction received.

Additionally, when applying the HO command, if the UE initiates transmission of a UAI message during the last 1 second, and the UE is still configured to provide the concerned UE assistance information, the UE would initiate a transmission of a UEAssistanceInformation message to provide the concerned UE assistance information. This is because for the UAI sent to the source gNB after starting of HO preparation by the source gNB, there is a possibility that it is not forwarded to the target gNB. This retransmission during handover can also be applied to UAI for temporary capability restriction, because a retransmission can help the target gNB to be aware of the temporary restricted capability of the UE as soon as possible. And additionally, in Rel-17 MUSIM, the UE also starts or restarts the prohibit timer or the leave without response timer for the MUSIM associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration. This can also be reused for Rel-18 MUSIM, i.e., UE also starts or restarts the prohibit timer or wait timer for Rel-18 MUSIM.

Proposal 1: The legacy handing for Rel-17 MUSIM UAI information during handover procedure is reused for temporary capability restriction of MUSIM.

In Rel-18, the UE can also send early indication to the network indicating that its capability is temporary restricted in RRCSetupComplete and RRCResumeComplete message. Then one question is whether the source gNB transmits the early indication to the target gNB during handover preparation procedure. There may be some benefit to support this, for example when HO is triggered by the source gNB, it does not have the exact temporary capability restriction from the UE yet, or the UE hasn’t sent the temporary capability restriction to the source gNB before reception of HO command. However usually, upon reception of the early indication, the gNB would configure the UE to report temporary capability restriction as soon as possible. It is a rare case that the UE is handed over to the target gNB before reporting the temporary capability restriction to the source gNB. Hence, we think that there is no strong need for further enhancement to support it.

Proposal 2: No further enhancement is needed for early indication of temporary capability restriction during handover.

# 3. Conclusion

In this paper, we discuss the remaining issue on MUSIM UAI/Early indication processing during handover procedure, and have the following proposals:

Proposal 1: The legacy handing for Rel-17 MUSIM UAI information during handover procedure is reused for temporary capability restriction of MUSIM.

Proposal 2: No further enhancement is needed for early indication of temporary capability restriction during handover.