3GPP TSG-RAN WG2 #125	R2-2400632

Athens, Greece, February-March 2024

Agenda Item:	7.9.1

Source:	OPPO

Title:	SRAP open issues for R18 sidelink Relay

Document for:	Discussion, Decision

Introduction

This document summarizes the SRAP open issues for R18 sidelink Relay.

Discussion

| No. | Open Issue | CR-Rapporteur Comment |
| --- | --- | --- |
| 1 | The error handling for U2U Remote UE and U2U Relay UE. (There was an EN for this issue, which was removed in last meeting but not fully addressed yet) | The error handling can be implemented following the U2N manner, e.g., UE discards the SRAP PDU when a SRAP Data PDU with SRAP header that contains UE ID fields or BEARER ID field which does not match sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in SL-SRAP-ConfigPC5 or slrb-PC5-ConfigIndex included in SLRB-Config for the concerned link is received. |