3GPP TSG-RAN2 Meeting #125	R2-2400633

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0030 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR sidelink relay enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In section 5.3a.1, there is no egress link determination for the transmission operation of U2U Remote UE which should be added since there may be multiple U2U Relay links for the remote UE, and the egress link determination is needed. - In section 5.3a.2, change the lowercase letter after a dash to uppercase for the practice of the spec. - In section 5.3a.1.3/5.3a.3.3, for the egress RLC channel determination for DRB, it is captured as “Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to RLC channel configured for the concerned bearer as specified in TS 38.331 [3].”. But “configured” is not accurate since for the IDLE/INACTIVE/OOC case, the egress RLC channel is derived by the RRC layer based on SIB/Pre-configuration but not configured by the network. Therefore, saying “indicated by upper layer” to cover all the cases is more accurate. - In section 5.4, how to handle the error case when PDU with an unknown UE ID/Bearer ID is received for U2U Remote and Relay UE should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In section 5.3a.1, add a new sub-section 5.3a.1.x for the egress link determination at U2U Remote UE. - In section 5.3a.2, change the lowercase letter after a dash to uppercase. - In section 5.3a.1.3/5.3a.3.3, change “configured” to “indicated by upper layer” - In section 5.4, add the error case handling for U2U Relay case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The egress link determination behavior at U2U Remote UE is missing; - The use of uppercase/lowercase letters after a dash is not unified in the spec. - It is incorrect to say “configured egress RLC channel” for IDLE/INACTIVE/OOC case since there is no configuration on that. - The handling of error case when PDU with unknown UE ID/Bearer ID is received for U2U Remote and Relay UE is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3a.1, 5.3a.2, 5.3a.3.3, 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

5.3a.1	Transmitting operation of U2U Remote UE

5.3a.1.1	General

The transmitting part of the SRAP entity on the PC5 interface of U2U Remote UE can receive SRAP SDU from upper layer and constructs U2U SRAP Data PDU.

Upon receiving an SRAP SDU from upper layer, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the UE ID fields and BEARER ID field in accordance with clause 5.3a.1.2;

-	Construct an U2U SRAP Data PDU with SRAP header, where the UE ID fields and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	Determine the egress link in accordance with clause 5.3a.1.x;

-	Determine the egress RLC channel in accordance with clause 5.3a.1.3;

-	Submit this U2U SRAP Data PDU to the determined egress PC5 Relay RLC channel.

5.3a.1.2	UE ID fields and BEARER ID field determination

For an U2U SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID (for SRC) field corresponding to sl-RemoteUE-LocalIdentity and UE ID (for DST) field corresponding to sl-PeerRemoteUE-LocalIdentity, configured as specified in TS 38.331 [3];

-	Determine the BEARER ID field for SL-SRBs as the fixed value (i.e., set 0/1/2/3 for SL-SRB0/1/2/3 respectively) or for SL-DRBs as the 5 LSBs of slrb-PC5-ConfigIndex used in end-to-end SL DRB configuration procedure as specified in TS 38.331 [3].

5.3a.1.3	Egress RLC channel determination

For a U2U SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If the U2U SRAP Data PDU is for SRB (i.e., the BEARER ID field is 0/1/2/3):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-U2U-RLC as specified in TS 38.331 [3].

-	Else if the SRAP Data is for DRB:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to RLC channel ID indicated by upper layer for the concerned bearer as specified in TS 38.331 [3].

5.3a.1.x	Egress link determination

For a U2U SRAP Data PDU to be transmitted, SRAP entity shall:

-	Determine the egress link on PC5 interface towards the U2U relay UE based on the determined UE ID fields in accordance with clause 5.3a.1.2.

Next Change

5.3a.2	Receiving operation of U2U Relay UE

Upon receiving an U2U SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the PC5 interface between the U2U Relay UE and the U2U Remote UE shall:

-	Deliver the SRAP data packet to the transmitting part of the SRAP entity on the PC5 interface between the U2U Relay UE and the peer U2U Remote UE.

Next Change

5.3a.3.3	Egress RLC channel determination

For a U2U SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If the U2U SRAP Data PDU is for SRB (i.e., the BEARER ID field is 0/1/2/3):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-U2U-RLC as specified in TS 38.331 [3].

-	Else if the U2U SRAP Data PDU is for DRB:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to RLC channel indicated by upper layer for the concerned bearer as specified in TS 38.331 [3].

Next Change

5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE, if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, the SRAP entity shall:

-	discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	discard the received SRAP Data PDU.

For U2U Remote UE, when a SRAP Data PDU with SRAP header that contains UE ID fields which does not match sl-PeerRemoteUE-LocalIdentity and sl-RemoteUE-LocalIdentity included in sl-SRAP-ConfigPC5 or a SRAP Data PDU with SRAP header that contains BEARER ID field which does not match slrb-PC5-ConfigIndex included in SLRB-Config is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2U Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID which does not match sl-PeerRemoteUE-LocalIdentity included in sl-SRAP-ConfigPC5 configured by the U2U Relay UE or a SRAP Data PDU with SRAP header contains BEARER ID field which does not match slrb-PC5-ConfigIndex included in SLRB-Config is received, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-RemoteUE-LocalIdentity corresponding to sl-RemoteUE-L2Identity of the ingress link is received by U2U Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

When any of the U2N Remote UE, the U2N Relay UE, the U2U Remote UE or the U2U Relay UE receives a SRAP PDU with invalid or reserved values, the SRAP entity shall:

-	discard the received SRAP PDU.

End of Change