3GPP TSG-RAN2 Meeting #125	R2-2400649

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.351 | CR | 0031 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections for NR sidelink relay enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In section 5.2.2 and 5.3.3, there is hanging paragraph which should be avoided according to the guidance in 21.801 clause 5.2.4. - In section 5.1, 5.2, 5.3, 5.4, 6.1.1, change the lowercase letter after dash to uppercase for the practise of the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In section 5.2.2 and 5.3.3, add new sub-section 5.2.2.0/5.3.3.0 General for the hanging paragraph. - In section 5.1, 5.2, 5.3, 5.4, 6.1.1, change the lowercase letter after dash to uppercase. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Hanging paragraph in the specification which against the guidance in 21.801; - The using of uppercase/lowercase letter after dash is not unified in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.2, 5.3, 5.4, 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

5.1	SRAP entity handling

5.1.1	SRAP entity establishment

When upper layers request establishment of an SRAP entity, UE shall:

-	Establish an SRAP entity;

-	Follow the procedures in clause 5.

5.1.2	SRAP entity release

When upper layers request release of an SRAP entity, UE shall:

-	Release the SRAP entity and the related SRAP configurations.

Next Change

5.2	DL Data transfer

5.2.1	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the Uu interface of U2N Relay UE shall:

-	Deliver the SRAP data packet to the transmitting part of the collocated SRAP entity on the PC5 interface.

5.2.2	Transmitting operation of U2N Relay UE

5.2.2.0	General

The transmitting part of the SRAP entity on the PC5 interface of U2N Relay UE receives SRAP data packets from the receiving part of the SRAP entity on the Uu interface of the same U2N Relay UE, and construct SRAP Data PDUs as needed (see clause 4.2.2).

When the transmitting part of the SRAP entity on the PC5 interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the egress link in accordance with clause 5.2.2.1;

-	Determine the egress RLC channel in accordance with clause 5.2.2.2;

-	If the SRAP Data PDU is for SRB0 (the BEARER ID field is 0, and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received):

-	Remove the SRAP header from the SRAP Data PDU;

-	Submit this SRAP Data PDU to the determined egress RLC channel of the determined egress link.

5.2.2.1	Egress link determination

For a SRAP Data PDU to be transmitted, SRAP entity shall:

-	If there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU:

-	Determine the egress link on PC5 interface corresponding to sl-L2IdentityRemote configured for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

5.2.2.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If the SRAP Data PDU is for SRB0 (the BEARER ID field is 0 and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3].

-	Else if there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU, which includes an sl-RemoteUE-RB-Identity that matches the SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1):

-	If the SRAP Data PDU is for SRB1 but the corresponding sl-EgressRLC-ChannelPC5 is absent in sl-SRAP-ConfigRelay:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3].

-	Else:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

5.2.3	Receiving operation of U2N Remote UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity shall:

-	If the SRAP Data PDU is not for SRB0 (not received from SL-RLC0 as specified in TS 38.331 [3]):

-	If the SRAP Data PDU is received from SL-RLC1 as specified in TS 38.331 [3]:

-	Remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to PDCP entity of SRB1 by ignoring the UE ID field and BEARER ID field of this SRAP Data PDU;

-	Else:

-	Remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer entity corresponding to the BEARER ID field of this SRAP Data PDU (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1);

-	Else:

-	Deliver the SRAP SDU (i.e., same as SRAP PDU for SRB0) to upper layer, i.e., RRC layer entity (TS 38.331 [3]).

Next Change

5.3	UL Data transfer

5.3.1	Transmitting operation of U2N Remote UE

The transmitting part of the SRAP entity on the PC5 interface of U2N Remote UE can receive SRAP SDU from upper layer and constructs SRAP Data PDU.

Upon receiving an SRAP SDU from upper layer, the transmitting part of the SRAP entity on the PC5 interface shall:

-	If the SRAP SDU is not for SRB0:

-	Determine the UE ID field and BEARER ID field in accordance with clause 5.3.1.1;

-	Construct an SRAP Data PDU with SRAP header, where the UE ID field and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	Else:

-	Construct an SRAP Data PDU without SRAP header in accordance with clause 6.2.2.

-	Determine the egress RLC channel in accordance with clause 5.3.1.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

5.3.1.1	UE ID field and BEARER ID field determination

For an SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID field corresponding to sl-LocalIdentity, configured as specified in TS 38.331 [3];

-	Determine the BEARER ID field corresponding to SRB identity for SRB (i.e., set the BEARER ID field to srb-Identity), or corresponding to DRB identity minus 1 for DRB (i.e., set the BEARER ID field to drb-Identity minus 1), from which the SRAP SDU is received, configured as specified in TS 38.331 [3].

5.3.1.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If the SRAP Data PDU is for SRB0:

-	Determine the egress PC5 Relay RLC channel in the link with U2N Relay UE corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3].

-	Else if the SRAP Data PDU is for SRB1 and if there is not an entry in sl-MappingToAddModList, whose sl-RemoteUE-RB-Identity matches the SRB identity of the SRAP Data PDU, or if there is an entry in sl-MappingToAddModList without the corresponding sl-EgressRLC-ChannelPC5:

-	Determine the egress PC5 Relay RLC channel in the link with U2N Relay UE corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3].

-	Else if there is an entry in sl-MappingToAddModList, whose sl-RemoteUE-RB-Identity matches the SRB identity or DRB identity of the SRAP Data PDU:

-	Determine the egress PC5 Relay RLC channel of the link with U2N Relay UE corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

5.3.2	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the PC5 interface shall:

-	Deliver the SRAP data packet to the transmitting part of the collocated SRAP entity on the Uu interface.

5.3.3	Transmitting operation of U2N Relay UE

The transmitting part of the SRAP entity on the Uu interface of U2N Relay UE can receive SRAP data packets from the receiving part of the SRAP entity on the PC5 interface of the same U2N Relay UE, and construct SRAP Data PDUs as needed (see clause 4.2.2).

When the transmitting part of the SRAP entity on the Uu interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the Uu interface shall:

-	If the SRAP Data PDU is received from SL-RLC0 as specified in TS 38.331 [3]:

-	Determine the UE ID field and BEARER ID field in accordance with clause 5.3.3.1;

-	Construct an SRAP Data PDU with SRAP header, where the UE ID field and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	Determine the egress RLC channel in accordance with clause 5.3.3.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

5.3.3.1	UE ID field and BEARER ID field determination

For an SRAP Data PDU received from SL-RLC0 as specified in TS 38.331 [3], the SRAP entity shall:

-	If there is an entry in sl-RemoteUE-ToAddModList, whose sl-L2IdentityRemote matches the Layer-2 ID of the remote UE from which the SRAP Data PDU is received:

-	Determine the UE ID field corresponding to sl-LocalIdentity configured for the concerned sl-L2IdentityRemote as specified in TS 38.331 [3];

-	Determine the BEARER ID field as 0 (i.e., set BEARER ID field as 0).

5.3.3.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU:

-	If the SRAP Data PDU is for SRB0:

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for SRB0 for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

-	Else if the SRAP Data PDU is received from SL-RLC1 as specified in TS 38.331 [3]:

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for SRB1 for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

-	Else if there is an entry in sl-RemoteUE-ToAddModList which includes an sl-RemoteUE-RB-Identity matches SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1):

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

Next Change

5.4	Handling of unknown, unforeseen, and erroneous protocol data

For U2N Remote UE, if sl-LocalIdentity and sl-RemoteUE-RB-Identity are both configured, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRemote is received, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

For U2N Relay UE, when a SRAP Data PDU with SRAP header that contains a UE ID field or BEARER ID field which does not match sl-LocalIdentity or sl-RemoteUE-RB-Identity included in sl-SRAP-ConfigRelay is received except in the case where the SRAP Data PDU from SL-RLC1 as specified in TS 38.331 [3] is the first SRAP Data PDU received from a U2N Remote UE, or when a SRAP Data PDU that contains a UE ID which does not match the concerned sl-LocalIdentity corresponding to sl-L2IdentityRemote of the ingress link is received by U2N Relay UE, the SRAP entity shall:

-	Discard the received SRAP Data PDU.

When the U2N Remote UE or the U2N Relay UE receives a SRAP PDU with invalid or reserved values, the SRAP entity shall:

-	Discard the received SRAP PDU.

Next Change

6.1.1	Data PDU

The SRAP Data PDU is used to convey the following with or without the PDU header:

-	Upper layer data.

End of Change