3GPP TSG-RAN WG2 Meeting #125	R2-2400665

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:		7.5.3.1

Source:		Futurewei

Title:		MAC BS table correction for Rel-18 XR

Document for:		Discussion and Decision

# Introduction

In this contribution, we discuss an issue with the values in Table 6.1.3.1-3 in [1] and propose TP to fix the issue.

# Discussions

## Issues with current BS table

In meeting #123-bis, RAN2 agreed on the following w.r.t. the use of the new and legacy BS tables [1]:

In meeting #124, RAN2 agreed on the maximum and minimum buffer sizes in the new BS table as follows [2]:

As a result, index 0 in Table 6.1.3.1-3 (the new BS table) corresponds to a buffer size range between 4903 and 5000 bytes [3], as shown below.

This buffer size range partially overlaps with the buffer size range corresponding to index 99 in Table 6.1.3.1-2 (the legacy BS table), as shown below:

Based on such design, the table below shows a range of actual data volumes in bytes, corresponding BS tables to be selected, corresponding indices to be reported, and corresponding buffer size levels interpreted by the gNB in bytes.

| Data Volume | Table Selected | Index Reported | Buffer Size Level Interpreted by gNB |
| --- | --- | --- | --- |
| … | … | … | … |
| 4751 | legacy | 98 | ≤ 4751 |
| 4752 | legacy | 99 | ≤ 5059 |
| …. | … | … | … |
| 4901 | legacy | 99 | ≤ 5059 |
| 4902 | legacy | 99 | ≤ 5059 |
| 4903 | legacy | 99 | ≤ 5059 |
| 4904 | new | 0 | ≤ 5000 |
| 4905 | new | 0 | ≤ 5000 |
| 4906 | new | 0 | ≤ 5000 |
| …. | … | … | … |
| 5000 | new | 0 | ≤ 5000 |
| 5001 | new | 1 | ≤ 5099 |
| … | … | … | … |

This table shows a strange phenomenon between the red numbers and the green numbers, where although the corresponding actual data volumes are monotonically increasing, the gNB doesn’t interpret that way. This is due to the partial overlap between the buffer size ranges of index 0 in the new table and of index 99 in the legacy table. Such strange interpretation doesn’t exist on the other (i.e., large BS) side of the table, even though partial overlapping also exists between index 255 of the new table and index 179 of the legacy table.

Observation:  Due to a partial overlap between the buffer size ranges of index 0 in Table 6.1.3.1-3 and of index 99 in Table 6.1.3.1-2, a data volume from 4752 to 4903 bytes, which is reported using Table 6.1.3.1-2, is interpreted by the gNB as being larger than a data volume from 4904 to 5000 bytes, which is reported using Table 6.1.3.1-3.

## Solution

To fix the inconsistent interpretations by the gNB, we need to avoid a partial overlap between the buffer size range of index 0 in the new table with that of any indices in the legacy table. In the Annex, we provide TP for an alternative Table 6.1.3.1.-3, where the quantization step in percentage is calculated as (750000/5059)^(1/256), assuming there is an index -1 corresponding to <= 5059. Then, the individual values are calculated using MS Excel spreadsheet with ROUNDDOWN function. In this way, buffer size range of index 0 in Table 6.1.3.1-3 would no longer partially overlap with that of index 99 or any other indices in Table 6.1.3.1-2, as shown in the Annex.

In the TP, the header row, which is currently missing in Table 6.1.3.1.-3, is also inserted.

Proposal 1. Updating the buffer size values in Table 6.1.3.1-3 to avoid the partial overlap between the buffer size range of index 0 in Table 6.1.3.1-3 and that of any indices in Table 6.1.3.1-2.

Proposal 2. Adopt the TP in the Annex for updating TS 38.321 [3].

# Conclusions

Observation:  Due to a partial overlap between the buffer size ranges of index 0 in Table 6.1.3.1-3 and of index 99 in Table 6.1.3.1-2, a data volume from 4752 to 4903 bytes, which is reported using Table 6.1.3.1-2, is interpreted by the gNB as being larger than a data volume from 4904 to 5000 bytes, which is reported using Table 6.1.3.1-3.

Based on this observation, we propose the following:

Proposal 1. Updating the buffer size values in Table 6.1.3.1-3 to avoid the partial overlap between the buffer size range of index 0 in Table 6.1.3.1-3 and that of any indices in Table 6.1.3.1-2.

Proposal 2. Adopt the TP in the Annex for updating TS 38.321 [3].

# References

R2_123bis_ChairNotes_23-10-13_13-00_final

R2_124_ChairNotes_23-11-17_final

TS 38.321 v18.0.0.

# Annex: TP for TS 38.321

Start of changes

Table 6.1.3.1-3. Refined buffer size levels (in bytes) for 8-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | > 5059 and ≤ 5158 | 64 | ≤ 18000 | 128 | ≤ 62812 | 192 | ≤ 219175 |
| 1 | ≤ 5260 | 65 | ≤ 18355 | 129 | ≤ 64050 | 193 | ≤ 223497 |
| 2 | ≤ 5364 | 66 | ≤ 18717 | 130 | ≤ 65313 | 194 | ≤ 227904 |
| 3 | ≤ 5469 | 67 | ≤ 19086 | 131 | ≤ 66601 | 195 | ≤ 232398 |
| 4 | ≤ 5577 | 68 | ≤ 19463 | 132 | ≤ 67914 | 196 | ≤ 236981 |
| 5 | ≤ 5687 | 69 | ≤ 19847 | 133 | ≤ 69254 | 197 | ≤ 241654 |
| 6 | ≤ 5799 | 70 | ≤ 20238 | 134 | ≤ 70619 | 198 | ≤ 246419 |
| 7 | ≤ 5914 | 71 | ≤ 20637 | 135 | ≤ 72012 | 199 | ≤ 251278 |
| 8 | ≤ 6030 | 72 | ≤ 21044 | 136 | ≤ 73432 | 200 | ≤ 256233 |
| 9 | ≤ 6149 | 73 | ≤ 21459 | 137 | ≤ 74880 | 201 | ≤ 261286 |
| 10 | ≤ 6271 | 74 | ≤ 21882 | 138 | ≤ 76356 | 202 | ≤ 266438 |
| 11 | ≤ 6394 | 75 | ≤ 22314 | 139 | ≤ 77862 | 203 | ≤ 271692 |
| 12 | ≤ 6520 | 76 | ≤ 22754 | 140 | ≤ 79397 | 204 | ≤ 277049 |
| 13 | ≤ 6649 | 77 | ≤ 23202 | 141 | ≤ 80963 | 205 | ≤ 282513 |
| 14 | ≤ 6780 | 78 | ≤ 23660 | 142 | ≤ 82560 | 206 | ≤ 288083 |
| 15 | ≤ 6914 | 79 | ≤ 24126 | 143 | ≤ 84188 | 207 | ≤ 293764 |
| 16 | ≤ 7050 | 80 | ≤ 24602 | 144 | ≤ 85848 | 208 | ≤ 299557 |
| 17 | ≤ 7189 | 81 | ≤ 25087 | 145 | ≤ 87540 | 209 | ≤ 305464 |
| 18 | ≤ 7331 | 82 | ≤ 25582 | 146 | ≤ 89267 | 210 | ≤ 311487 |
| 19 | ≤ 7476 | 83 | ≤ 26086 | 147 | ≤ 91027 | 211 | ≤ 317629 |
| 20 | ≤ 7623 | 84 | ≤ 26601 | 148 | ≤ 92822 | 212 | ≤ 323893 |
| 21 | ≤ 7773 | 85 | ≤ 27125 | 149 | ≤ 94652 | 213 | ≤ 330279 |
| 22 | ≤ 7927 | 86 | ≤ 27660 | 150 | ≤ 96519 | 214 | ≤ 336792 |
| 23 | ≤ 8083 | 87 | ≤ 28206 | 151 | ≤ 98422 | 215 | ≤ 343433 |
| 24 | ≤ 8242 | 88 | ≤ 28762 | 152 | ≤ 100363 | 216 | ≤ 350205 |
| 25 | ≤ 8405 | 89 | ≤ 29329 | 153 | ≤ 102342 | 217 | ≤ 357111 |
| 26 | ≤ 8571 | 90 | ≤ 29907 | 154 | ≤ 104360 | 218 | ≤ 364153 |
| 27 | ≤ 8740 | 91 | ≤ 30497 | 155 | ≤ 106418 | 219 | ≤ 371334 |
| 28 | ≤ 8912 | 92 | ≤ 31099 | 156 | ≤ 108516 | 220 | ≤ 378656 |
| 29 | ≤ 9088 | 93 | ≤ 31712 | 157 | ≤ 110656 | 221 | ≤ 386123 |
| 30 | ≤ 9267 | 94 | ≤ 32337 | 158 | ≤ 112838 | 222 | ≤ 393737 |
| 31 | ≤ 9450 | 95 | ≤ 32975 | 159 | ≤ 115063 | 223 | ≤ 401501 |
| 32 | ≤ 9636 | 96 | ≤ 33625 | 160 | ≤ 117332 | 224 | ≤ 409418 |
| 33 | ≤ 9826 | 97 | ≤ 34288 | 161 | ≤ 119646 | 225 | ≤ 417491 |
| 34 | ≤ 10020 | 98 | ≤ 34964 | 162 | ≤ 122005 | 226 | ≤ 425723 |
| 35 | ≤ 10217 | 99 | ≤ 35654 | 163 | ≤ 124411 | 227 | ≤ 434118 |
| 36 | ≤ 10419 | 100 | ≤ 36357 | 164 | ≤ 126864 | 228 | ≤ 442679 |
| 37 | ≤ 10624 | 101 | ≤ 37074 | 165 | ≤ 129366 | 229 | ≤ 451408 |
| 38 | ≤ 10834 | 102 | ≤ 37805 | 166 | ≤ 131916 | 230 | ≤ 460309 |
| 39 | ≤ 11047 | 103 | ≤ 38550 | 167 | ≤ 134518 | 231 | ≤ 469386 |
| 40 | ≤ 11265 | 104 | ≤ 39310 | 168 | ≤ 137170 | 232 | ≤ 478641 |
| 41 | ≤ 11487 | 105 | ≤ 40086 | 169 | ≤ 139875 | 233 | ≤ 488080 |
| 42 | ≤ 11714 | 106 | ≤ 40876 | 170 | ≤ 142633 | 234 | ≤ 497704 |
| 43 | ≤ 11945 | 107 | ≤ 41682 | 171 | ≤ 145446 | 235 | ≤ 507518 |
| 44 | ≤ 12181 | 108 | ≤ 42504 | 172 | ≤ 148314 | 236 | ≤ 517526 |
| 45 | ≤ 12421 | 109 | ≤ 43342 | 173 | ≤ 151239 | 237 | ≤ 527731 |
| 46 | ≤ 12666 | 110 | ≤ 44197 | 174 | ≤ 154221 | 238 | ≤ 538137 |
| 47 | ≤ 12915 | 111 | ≤ 45068 | 175 | ≤ 157262 | 239 | ≤ 548749 |
| 48 | ≤ 13170 | 112 | ≤ 45957 | 176 | ≤ 160363 | 240 | ≤ 559570 |
| 49 | ≤ 13430 | 113 | ≤ 46863 | 177 | ≤ 163525 | 241 | ≤ 570604 |
| 50 | ≤ 13695 | 114 | ≤ 47787 | 178 | ≤ 166750 | 242 | ≤ 581855 |
| 51 | ≤ 13965 | 115 | ≤ 48730 | 179 | ≤ 170038 | 243 | ≤ 593329 |
| 52 | ≤ 14240 | 116 | ≤ 49691 | 180 | ≤ 173391 | 244 | ≤ 605029 |
| 53 | ≤ 14521 | 117 | ≤ 50670 | 181 | ≤ 176810 | 245 | ≤ 616959 |
| 54 | ≤ 14807 | 118 | ≤ 51670 | 182 | ≤ 180296 | 246 | ≤ 629125 |
| 55 | ≤ 15099 | 119 | ≤ 52688 | 183 | ≤ 183852 | 247 | ≤ 641530 |
| 56 | ≤ 15397 | 120 | ≤ 53727 | 184 | ≤ 187477 | 248 | ≤ 654181 |
| 57 | ≤ 15701 | 121 | ≤ 54787 | 185 | ≤ 191174 | 249 | ≤ 667080 |
| 58 | ≤ 16010 | 122 | ≤ 55867 | 186 | ≤ 194943 | 250 | ≤ 680235 |
| 59 | ≤ 16326 | 123 | ≤ 56969 | 187 | ≤ 198788 | 251 | ≤ 693648 |
| 60 | ≤ 16648 | 124 | ≤ 58092 | 188 | ≤ 202707 | 252 | ≤ 707326 |
| 61 | ≤ 16976 | 125 | ≤ 59238 | 189 | ≤ 206705 | 253 | ≤ 721274 |
| 62 | ≤ 17311 | 126 | ≤ 60406 | 190 | ≤ 210781 | 254 | ≤ 735496 |
| 63 | ≤ 17652 | 127 | ≤ 61597 | 191 | 214937 | 255 | ≤ 750000 |

End of changes