3GPP TSG-RAN WG2 #125	Tdoc R2-2400756

Athens, Greece, February 26 – March 01, 2024

Agenda Item:	7.3.3

Source:	Ericsson

Title:	Description of UE support for NW DTX-DRX

Document for:	Discussion, Decision

# 1	Introduction

The following open issue was raised for cell DTX/DRX:

Issue 1: Relation of NES cell barring with cell DTX/DRX UE capability.

RAN1 designed the cell DTX/DRX UE capability as 3 options: {cell DTX only, cell DRX only, both}; this implementation does not work well with 1-bit NES cell DTX/DRX barring we have in RAN2. The relation of NES cell barring with cell DTX/DRX UE capability needs to be resolved.

This contribution discusses how to address this open issue.

# 2	Discussion

The TS 38.331 is not consistent when referring to UE capabilities and in some cases it is referred to an actual capability field while in other cases it is referred to “general” support of a feature (i.e. without mentioning a specific capability field). In the latter case, it is usually clear which capability is applicable since there is a single field indicating support of this capability.

However, for cell DTX/DRX, there are 3 possible values for the UE capability support:

nes-CellDTX-DRX-r18                                             ENUMERATED {cellDTXonly, cellDRXonly, both}                OPTIONAL,

Therefore, it would be clearer to refer to the specific capability field in 38.331. But it should be clarified which values within the capability field are applicable to the NES cell barring in 38.331 (i.e. legacy UEs can be barred from a NES cell while UEs supporting cell DTX/DRX are allowed to camp on that cell). In our understanding, either if the UE supports only cell DTX or only cell DRX, the UE should be allowed to camp on a cell that bars legacy UEs (by setting cellBarred in MIB to barred) and allows NES capable UEs (by setting cellBarredNES to notBarred). It can be that the cell is applying both cell DTX and DRX, but in that case, the NW can anyway handover this UE to another cell (in case of PCell) or release that serving cell configuratrio from the UE (in case of SCell).

NES cell barring behaviour is applicable to a UE supporting any value of nes-CellDTX-DRX.

To clarify this in 38.331, in principle it would be sufficient to replace occurrences of NES cell DTX/DRX support with the field nes-CellDTX-DRX (e.g. “if the UE supports nes-CellDTX-DRX”), since inclusion of this field (from its ASN.1 structure) already implies that one of the supported values is set. Nevertheless, to be clearer, it could also be stated in 38.331 that any of the values of nes-CellDTX-DRX is applicable (e.g. “if the UE indicates any of the values in nes-CellDTX-DRX”). The ASN.1 capability could also be simplified by having two different fields (i.e. one for DTX and one for DRX).

Cell DTX/DRX support in 38.331 is split into two capabilities (nes-CellDTX and nes-CellDRX) and procedural text clarified as “if the UE supports nes-CellDTX or nes-CellDRX”.

A TP is depicted in section 4 to account for these changes.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	NES cell barring behaviour is applicable to a UE supporting any value of nes-CellDTX-DRX.

Proposal 2	Cell DTX/DRX support in 38.331 is split into two capabilities (nes-CellDTX and nes-CellDRX) and procedural text clarified as “if the UE supports nes-CellDTX or nes-CellDRX”.

# 4	TP for 38.331 and 38.306

## 38.331

##### 5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

2>	if the access is not for NTN or the UE is not capable of NTN or the UE does not support neither nes-CellDTX nor nes-CellDRX; and

2>	if the access is not for ATG or the UE is not capable of ATG; and

2>	if the cellBarred in the acquired MIB is set to barred:

3>	if the UE is an (e)RedCap UE and ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]):

4>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

2>	else:

3>	apply the received systemFrameNumber, pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position.

NOTE 1:	A UE capable of NTN access should acquire SIB1 to determine whether the cell is an NTN cell.

NOTE 2:	A UE capable of ATG access should acquire SIB1 to determine whether the cell is an ATG cell.

NOTE 3:	A UE supporting either nes-CellDTX or nes-CellDRX should acquire SIB1 to determine the cell barring status when the cellBarred in MIB is set to barred.

##### 5.2.2.4.2	Actions upon reception of the SIB1

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the access is for NTN:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running; and

2>	if the cellBarredNTN in the acquired SIB1 is set to barred or the cellBarredNTN is not included in the acquired SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the access is for ATG:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running; and

2>	if the cellBarredNTN in the acquired SIB1 is set to barred or the cellBarredNTN is not included in the acquired SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

1>	if the UE is a RedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the RedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselectionRedCap is not present in SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform barring as if intraFreqReselectionRedCap is set to allowed, upon which the procedure ends;

2> else:

3>	if the cellBarredRedCap1Rx is present in the acquired SIB1 and is set to barred and the UE is equipped with 1 Rx branch; or

3>	if the cellBarredRedCap2Rx is present in the acquired SIB1 and is set to barred and the UE is equipped with 2 Rx branches; or

3>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform barring based on intraFreqReselectionRedCap as specified in TS 38.304 [20], upon which the procedure ends;

1>	if the UE supports either nes-CellDTX or nes-CellDRX and it is in RRC_IDLE or in RRC_INACTIVE, or if the UE supporting either nes-CellDTX or nes-CellDRX is in RRC_CONNECTED while T311 is running:

2>	if cellBarred in the acquired MIB is set to barred:

3>	if cellBarredNES is absent in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20];

1>	if the UE is an eRedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the eRedCap UE is in RRC_CONNECTED while T311 is running:

2>	if intraFreqReselection-eRedCap is not present in SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform barring as if intraFreqReselection-eRedCap is set to allowed upon which the procedure ends;

2>	else:

3>	if the cellBarred-eRedCap1Rx is present in the acquired SIB1 and is set to barred and the UE is equipped with 1 Rx branch; or

3>	if the cellBarred-eRedCap2Rx is present in the acquired SIB1 and is set to barred and the UE is equipped with 2 Rx branches; or

3>	if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform barring based on intraFreqReselection-eRedCap as specified in TS 38.304 [20] upon which the procedure ends;

***********omitted unchanged parts*************

nes-CellDTX-r18                                                 ENUMERATED {supported}                OPTIONAL,

nes-CellDRX-r18                                                 ENUMERATED {supported}                OPTIONAL,

-- R1 42-5: Cell DTX/DRX operation triggered by DCI format 2_9

nes-CellDTX-DRX-DCI2-9-r18                                      ENUMERATED {supported}                                     OPTIONAL,

## 38.306

| nesBasedCondHandoverWithDCI-r18 Indicates whether the UE supports DCI-based enabling/disabling NES-specific CHO execution condition, i.e. NES-specific CHO execution condition based on source cell NES mode indicated via DCI format 2_9 as specified in TS 38.331 [9]. A UE supporting this feature shall also indicate the support of condHandover-r16. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| nes-CellDTX-r18 Indicates whether the UE supports cell DTX operation by RRC configuration. The supported number of cell DTX patterns per cell group is 2, regardless if each pattern is for cell DTX only, cell DRX only, or both. | Band | No | N/A | N/A |
| nes-CellDRX-r18 Indicates whether the UE supports cell DRX operation by RRC configuration. The supported number of cell DRX patterns per cell group is 2, regardless if each pattern is for cell DTX only, cell DRX only, or both. | Band | No | N/A | N/A |
| nes-CellDTX-DRX-DCI2-9-r18 Indicates whether the UE supports cell DTX/DRX configuration activation and deactivation via DCI 2_9. A UE supporting this feature shall also indicate support of nes-CellDTX-DRX-r18. | Band | No | N/A | N/A |