3GPP TSG-RAN WG2 Meeting #125	R2-2400761

Athens, USA, 26 March – 1 February 2024

Agenda item:	7.13.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RIL N022 - Clarification on location information included in SPR

WID/SID:	NR_ENDC_SON_MDT_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The following text is included in clause 5.7.10.7 of TS 38.331 referring to the location information to be added to SPR:

3>	if sn-InitiatedPSCellChange is configured in the RRCReconfiguration including the last applied RRCReconfiguration with reconfigurationWithSync for the SCG:

4>	if available, set the locationInfo as in 5.3.3.7 7 according to the otherConfig associated with the source PSCell;

3>	else:

4>	if available, set the locationInfo as in 5.3.3.7 7 according to the otherConfig associated with the PCell;

In this paper we propose a clarification regarding the location information.

# 2	Discussion

Location information is often included in various reports (e.g. RLF report, SHR, CEF, SCGFailureIndication) as an optional element, e.g. if the UE has been configured to record this information previously thus making it available at the time of the report generation.

In Rel-18, it has also been decided that location information will be added to the SPR. In RAN2#124 it was decided that:

For the location information in the SPR the UE logs what is configured by the network (MN or SN) and as per the network node initiating the change.

The agreement was captured in SPR procedural text as copied above: if the PSCell change is SN-initiated, the SN configured location information will be logged, otherwise location information configured by the MN will be logged.

However, unlike other information the UE may be instructed to log, e,g., measurements  which may be configured differently by the MN and the SN (list of cells to be measured, events, etc), the location of the UE is the same no matter which node configures it. It can also happen that the initiating node of the PScell change (MN or SN) did not configure location information to be tracked by UE, but the other node did (SN or MN). According to the current text in the CR, even though the UE has the location information available, it will not add it to the SPR if it does not correspond to the initiating node. Thus, the network would be missing a possibly crucial piece of information that is available in the UE.

With this in mind, we believe that any available location information should be added to the SPR, independent of the configuring node. If two such information elements are available in the UE, we can keep the current agreement of including the location information associated with the initiating node can be kept.

Proposal 1: In case the UE have location information available, this should be included in the SPR, regardless of node that configured it.

Proposal 2: Adopt the TP in the Annex to implement Proposal 1 in TS 38.331.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: In case the UE have location information available, this should be included in the SPR, regardless of node that configured it.

Proposal 2: Adopt the TP in the Annex to implement Proposal 1 in TS 38.331.

# Annex:	TP for 38.331

***** Start of Changes *****

#### 5.7.10.7	Actions for the successful PSCell change or addition report determination

<Text omitted>

3>	include sn-InitiatedPSCellChange if sn-InitiatedPSCellChange is included in the RRCReconfiguration including the applied RRCReconfiguration message with reconfigurationWithSync for the SCG;

3>	if sn-InitiatedPSCellChange is configured in the RRCReconfiguration including the last applied RRCReconfiguration with reconfigurationWithSync for the SCG:

4>	if available, set the locationInfo as in 5.3.3.7 7 according to the otherConfig associated with the source PSCell;

4>	else if available, set the locationInfo as in 5.3.3.7 7 according to the otherConfig associated with the source PCell;

3>	else:

4>	if available, set the locationInfo as in 5.3.3.7 7 according to the otherConfig associated with the PCell;

4>	else if available, set the locationInfo as in 5.3.3.7 7 according to the otherConfig associated with the source PSCell;

1>	release successPSCell-Config configured by the source PSCell if available and thresholdPercentageT304 if configured by the target PSCell.

The UE may discard the successful PSCell change or addition information, i.e., release the UE variable VarSuccessPSCell-Report, 48 hours after the last successful PSCell change or addition information is added to the VarSuccessPSCell-Report or upon detaching from the network.

***** End of Changes *****